TSG-SA Working Group 1 (Services) meeting #7	TSG S1 (00) 119

Sophia Antipolis, France 7 - 11 February 2000	Agenda Item: 5.2

3GPP TSG-SA WG2 #11	Tdoc S2-000182

Puerto Vallarta, Mexico

24-28 January 2000

# LIAISON STATEMENT

Source: 		3GPP TSG SA WG2

Title: 		Proposed Response to the Liaison Statement on Push Services for GPRS

To:		SA WG1

Cc:		CN WG3

___________________________________________________________________________

S2 thanks S1 for the liaison statement in Tdoc S2-000068 (S1-991061) where S1 requests information from S2 regarding push services.

A prerequisite for a GPRS MS to be available for push services is that a suitable PDP context has been activated, and hence that the MS can be reached via an IP address.

PDP context activation can be initiated by the MS, or activation can be requested by the network as long as the MS is GPRS-attached.

For MS-initiated PDP context activation, the IP address to be activated can either be a fixed, subscribed address (static address), or the IP address can be allocated to the MS for the activation period (dynamic address).

Network-requested PDP context activation supports only the static-address option.

Given that the host initiating the push knows how to find the IP address that the target MS can be reached with, then push services is supported in GPRS today for all three flavours of PDP context activations mentioned above.

On S1's request that possible inconsistencies between the way access to external IP-networks is described in 22.060 section 4.6 and the way it is described in 23.060 and 29.061 be clarified, S2 requests S1 to be more specific since the referred section 4.6 does not exist in the 22.060 versions available to S2.

S2 looks forward to continue working with S1 on push services, and to learn more about S1's requirements on such services.