TSG-SA Working Group 1 (Services) meeting #7	TSG S1 (00)135

Sophia Antipolis, 9th – 11th February 2000	Agenda Item: 5.19

| CHANGE REQUEST No : |  |  |  |  |  |  |  | Please see embedded help file at the bottom of this page for instructions on how to fill in this form correctly. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Specification / Report UMTS |  |  |  | 22.030 |  | Version: |  |  | 3.2.0 |  |  |  |
| Submitted to: | TSG SA1#7 |  | for approval |  | X |  | without presentation ("non-strategic") |  |  |  |  |  |
| list TSG plenary meeting no. here  |  | for information |  |  |  |  | with presentation ("strategic") |  |  |  | X |  |
| PT SMG CR cover form is available from: http://docbox.etsi.org/tech-org/smg/Document/smg/tools/CR_form/crf28_1.zip |  |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | USIM |  | TE |  | Network | x |
| --- | --- | --- | --- | --- | --- | --- |

(at least one should be marked with an X)

| Work item: | Follow Me |
| --- | --- |

| Source: | Siemens | Date: | 11/2/2000 |
| --- | --- | --- | --- |

| Subject: | Introduction of Service Code 212 for “Follow Me” |
| --- | --- |

| Category: | F Correction |  | Release: | Phase 2 |  |
| --- | --- | --- | --- | --- | --- |
|  | A Corresponds to a correction in an earlier release |  |  | Release 96 |  |
| (one category | B Addition of feature | X |  | Release 97 |  |
| And one release | C Functional modification of feature |  |  | Release 98 |  |
| Only shall be | D Editorial modification |  |  | UMTS 99 | X |
| Marked with an X) |  |  |  |  |  |

| Reason for change: | “Follow Me” (FM stage 1 – 22.094, FM stage 2 – 23.094) has been specified for UMTS Release ’99. This CR introduces the service code for the FM feature. |
| --- | --- |

| Clauses affected: | Changed: 0.1, ANNEX B (normative) New chapter: 3.5.6.3 |
| --- | --- |

| Other specs | Other releases of same spec |  |  List of CRs: |  |
| --- | --- | --- | --- | --- |
| Affected: | Other core specifications |  |  List of CRs: |  |
|  | MS test specifications / TBRs |  |  List of CRs: |  |
|  | BSS test specifications |  |  List of CRs: |  |
|  | O&M specifications |  |  List of CRs: |  |

| Other comments: |  |
| --- | --- |

<--------- double-click here for help and instructions on how to create a CR.

## 0.1	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.

- For this Release 1999 document, references to GSM documents are for Release 1999 versions (version 8.x.y).

[1]	GSM 01.04: "Digital cellular telecommunication system (Phase 2+); Abbreviations and acronyms".

[2]	TS 22.004: " General on supplementary services".

[3]	TS 22.011: " Service accessibility".

[4]	TS 22.016: " International Mobile station Equipment Identities (IMEI)".

[5]	TS 22.083: " Call Waiting (CW) and Call Hold (HOLD) supplementary services - Stage 1".

[6]	TS 22.084: " MultiParty (MPTY) supplementary services - Stage 1".

[7]	TS 22.090: " Stage 1 description of Unstructured Supplementary Service Data (USSD)".

[8]	TS 23.038: Alphabets and language".

[9]	TS 24.008: " Mobile radio interface layer 3 specification; Core Network Protocols – Stage 3".

[10]	TS 24.080: " Mobile radio interface layer 3 supplementary services specification Formats and coding".

[11]	TS 29.002: " Mobile Application Part (MAP) ".

[12]	GSM 02.81: "Line Identification Supplementary Services – Stage 1"

[13]	ITU-T Recommendation E.164: "Numbering plan for the ISDN era"

[14]	ITU-T Recommendation E.121: "Pictograms and symbols to assist users of the telephone service"

[15]	TS 22.072: "Call Deflection; Stage 1"

[16]	TS 22.091: "Explicit Call Transfer Supplementary Service; Stage 1"

[17]	TS 22.093: "Call Completion to Busy Subscriber (CCBS); Stage 1"

[18]	TR 21.905: "Vocabulary for 3GPP Specifications"

[19]	TS 22.101; "Service principles"

[20]	TS 22.001: "Principles of circuit  telecommunication services supported by a Public Land Mobile Network (PLMN)"

[21]	TS 22.094  “Follow Me  Service description – Stage 1”

#### 3.5.6.3	Follow Me (FM)

The Follow Me Feature is defined in TS 22.094  [21].
FM control procedures make use of  USSD.

Annex B (normative):
Codes for defined Supplementary Services

Table B.1: Input information for handling of defined Supplementary Services

Supplementary	Service
		Service	Code	SIA	SIB	SIC

22.067
		eMLPP	75 and 75n		where n=0-4

22.072
		CD	66

22.081	
		CLIP	30	-	-	-
		CLIR	31	-	-	-
		COLP	76	-	-	-
		COLR	77	-	-	-

	In temporary mode, to suppress CLIR for a single call, enter:	
		" * 31 # <called number> SEND "	

	In temporary mode, to invoke CLIR for a single call enter:	
		" # 31 # <called number> SEND "

22.082
		CFU	21	DN	BS	-
		CF Busy	67	DN	BS	-
		CF No Reply	61	DN	BS	T
		CF Not Reachable	62	DN	BS	-

	all CF		002	DN	BS	T
	all conditional CF	004	DN	BS	T

22.083
		WAIT	43	BS	-	-
			see section 4.5.5	
		HOLD	see section 4.5.5

22.084
		MPTY	see section 4.5.5

22.087
		UUS Service 1	361 	R	-	-
		UUS Service 2	362	R	-	-
		UUS Service 3	363	R	-	-
		all UUS Services	360	R	-	-

		If UUS shall be activated when originating a call, enter:	
		" * 36X * R # <called number> SEND"	
		(X is indicating the requested UUS service)

Table B.1(concluded): Input information for handling of defined Supplementary Services

Supplementary	Service
		Service	Code	SIA	SIB	SIC

22.088
		BAOC	33	PW	BS	-
		BAOIC	331	PW	BS	-
		BAOIC exc home	332	PW	BS	-
		BAIC	35	PW	BS	-
		BAIC roaming	351	PW	BS	-

	all Barring Serv.	330	PW	BS	-
	Outg. Barr. Serv.	333	PW	BS
	Inc. Barr. Serv.	353	PW	BS

22.091
		ECT	96		see section 4.5.5

22.093
		CCBS	37	n	See Section 4.5.5
				where n=1-5

22.094
		FM	212		See Section 3.5.6.3

22.096
		CNAP	300	-	-	-

22.097
		MSP	59n	PW	where n=1-4

DN = Directory Number;
PW = Password (see subclause 4.5.4);
BS = Basic Service Group (if required) - see annex C;
T  = No Reply Condition Timer (5-30 seconds);
R  = UUS required option.
SI required	Y	=	Yes;
				N	=	No;
				-	=	Not applicable.

"UUS required" option

For the "UUS required" option two values are defined:

R = 0		UUS not required;
R = 1		UUS required.

NOTE:	If the "UUS required" option is requested for a call, the call will only be established if the requested UUS capabilities are available.

If the "UUS required" option is not contained in an activation request UUS shall be activated without the UUS required option.