TSG-SA Working Group 1 (Services) meeting	TSGS1#8(00)0222

Meeting #8, Beijing, China, 10-15 April 2000

Source:	Ericsson

Title:	Global Text Telephony, service descrtiption

Document for:	Discussion

Agenda Item:		  6.6.1

Abstract:	         This stage 1 draft specification describes the Global Text Telephonyand text conversation  feature, adding real time text conversation to voice and multimedia services. Interworking with corresponding fixed services is included. The original requirements are included in the document.

3G TS 22.GlobalText 0.0.1 (2000-04)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects

Service aspects;

Stage 1

Global Text Telephony

(3G TS 22.GlobalText version 0.0.1)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.	 
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Reference

DTR/TSGS-xxxxxxxU

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 1999, 3GPP Organizational Partners (ARIB, CWTS, ETSI, T1, TTA,TTC).

All rights reserved.

# Contents

Contents	3

Foreword	3

Introduction	4

1.1	Total Conversation	4

1.2	Interworking	5

1.3	Additional services	5

1.3.1	Emergency services	5

1.3.2	Relay services	5

2	Scope	6

3	References	6

4	Definitions and abbreviations	7

4.1	Definitions	7

4.2	Abbreviations	7

5	High level Requirements	7

6	General Requirements	8

6.1	Text conversation management	8

6.2	Text conversation handling during calls	8

6.3	Alerting and Answering	9

6.4	Addressing	9

6.5	Roaming	9

7	Profile	9

8	Security and reliability	9

9	Charging	9

10	External Interface	10

11	Interworking	10

12	Initial requirements	10

12.1	R&O on Transmission Performance	10

12.2	R&O on Man Machine Interface (MMI)	11

12.3	R&O on Compatibility	12

12.4	R&O on Complexity of Implementation and Roll Out	12

Annex A: Change history	14

History	15

# Foreword

This Technical Specification has been produced by the 3GPP.

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of this TR, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version 3.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	Indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the specification;

# Introduction

Text telephone systems were implemented mainly for distant conversation with deaf, hard-of-hearing, speech-impaired and deaf-blind users. The text telephone systems offer a real time, character by character, conversation in text, optionally combinable with voice. With proper implementation in mobile systems, the service is expected to be of interest for any user.

The general user needs are described in ETSI ETR 333 Human Factors, Text Telephony; User requirements and Recommendations [2).

In PSTN, seven different, openly specified systems for text telephony exist, and are used in different regions. Some proprietary modes are also used.

The open specifications are called Baudot, DTMF, EDT, V.21, Bell103, Minitel and V.18. They all use different modem technologies and character coding for the transmission of text. They are briefly defined in the annexes of ITU-T V.18.

ITU-T V.18 is an automoding mechanism that enables communication with all the legacy modes on the modem level. It also detects when both parties have V.18, and invokes an internationally useful character set, defined in ITU-T T.140, and optionally transmission of text and voice simultaneously. Native modulation for V.18 is V.21, while V.61 is used when simultaneous voice and text is wanted. Other modulations can be negotiated.

V.18 and T.140 are intended for use in new PSTN text telephones. They are also intended for use in gateways, bridging from the fragmented situation in the PSTN, into new services, designed according to the Total Conversation concept.

## Total Conversation

Total Conversation adds text conversation to multimedia protocols in a standardised way, so that simultaneous communication in video, text and voice is accomplished. For the text part, the Unicode based text presentation protocol ITU-T T.140 is used. It is transmitted with a specific standardised transport channel for each environment. Subsets can be used for example for text telephony enabled in the multimedia architectures.

Total Conversation is currently defined in the following environments:

1. Packet networks, where the procedures described in H.323 Annex G can be used for text conversation sessions, using TCP or RTP/T140 for the transport of T.140.

2. Packet networks, where the IETF Session Initiation Protocol SIP can be used for setting up and conducting text conversation sessions using RTP/T140 for the transport of T.140.

3. The H.324 multimedia environment in PSTN, ISDN and Mobile networks, where an AL1 channel connected by H.245 procedures is used for T.140.

4. The H.320 multimedia environment, where a H.224 channel with client ID=2 is specified for   transport of T.140.

5. The T.120 data conferencing environment, that can be used alone or in conjunction with any of the environments above, where T.134 specifies the application entity and T.125 the data channel for T.140.

6. Text Telephony in the PSTN using the ITU-T V.18 modem and T.140 for the presentation without any further transport protocol.

## Interworking

Interworking between these forms of Text Conversation can be achieved through the use of gateways or interworking functions. One gateway mechanism (in the process of standardisation) is ITU-T H.248, where Annex F describes additions for handling text telephony and text conversation.

## Additional services

Apart from the basic user – to – user conversation, the following are examples of important additional services that can be offered text telephone users and Total Conversation users.

### Emergency services

The ability to call to emergency services, announce an emergency situation and discuss the actions can be offered text telephone users and Total Conversation users.

### Relay services

In order to have conversations with users of plain voice telephones, text relay services are established. Currently, manned relay services dominate, offering real time translation between written and spoken language. Automatic and semi-automatic versions are emerging. A through connection of the voice channel is usually offered as an option to satisfy users who can benefit from multiple modes.

With Total Conversation services, both traditional text relay services and other relay services are established. Examples are sign language relay services with text support during service establishment and speech-support relay services with visual enhancement.

# Scope

This Technical Specification defines the stage one description of the Global Text Telephone Feature, GTT.. Stage one is  the set of requirements which shall be supported for the provision of the real-time text conversation feature, seen primarily from the subscriber’s and service providers’ points of view.

This TS includes information applicable to network operators, service providers, terminal and network manufacturers.

This TS contains the core requirements for the Global Text Telephony feature, which are sufficient to provide a complete feature to incorporate in conversational services.

Additional functionalities not documented in this TS may implement requirements which are considered outside the scope of this TS. Such additional functionality may be on a network-wide basis, nation-wide basis or particular to a group of users. Such additional functionality shall not compromise conformance to the core requirements of the feature.

This TS defines the requirements for GTT to be understood as a framework to enable real-time transmission of text, alone or in combination with other media, especially video and voice.

Thus the GTT enables text conversation to be included in any mobile conversational service.

Interworking with existing text telephony in PSTN as well as emerging forms of standardised text conversation in all networks is within the scope of this document, as well as interworking with Multimedia Messaging Services.

# References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.

-	A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

[1]	3G TS 22.101 (V3.6.0 onwards): “Service Principles”

[2]	ETSI ETR333; Text Telephony; User requirements and Recommendations

[3]	ITU-T H.248 Annex F Facsimili, Text conversation and call discrimination packages (not yet approved)

# Definitions and abbreviations

## Definitions

## Abbreviations

For the purposes of this document the following abbreviations apply:

EDT	European Deaf Telephone
GTT	Global Text Telephony; The feature that adds real time text conversation to any 3G conversational environment.
TTY	Here used as the term for the text telephone type dominating in USA.
DTMF	Here used as a term for the text telephone type used in Holland, using DTMF tones.
VCO	Voice Carry Over: Alternating (or parallel) sending of Speech and receiving Text 
HCO	Hear Carry Over: Alternating (or parallel) receiving Speech and transmission of Text
FCC	Federal Communications Commission (of United States of America)
PER	Printable character Error Rate
MMI	Man Machine Interface
SIM	Subscriber Identification Module
ITU	International Telecom Standardisation Union
GSM	Global System for Mobile communication
UMTS	Universal Mobile Telecommunication System
3GPP	Third Generation Partnership Project

# High level Requirements

The following list gives the high level requirements of the GTT. These are requirements which are independent of the user’s perception of the feature:-

- Global Text Telephony environments

A standardised method for Global Text Telephony should at least be specified for each environment in the mobile networks that can carry voice.

That means at least the following mobile environments:

- Voice telephony, with the text conversation carried inband in the voice channel.

- Voice telephony, with the text conversation carried by an out of band data transmission procedure

- Circuit switched multimedia.

- Packet switched multimedia

- Forward compatible text conversation

Global Text Telephone mechanisms shall provide the capability to support current and evolving text telephony and text conversation features by re-using existing standards as far as possible and proposing extensions (as necessary) to existing standards  (i.e. the global text telephony feature shall support the evolution of Total Conversation  technologies in all networks).

- Consistent text conversation

Regardless of the service, the far end terminal, the interworking facilities involved, GTT shall be capable of providing a consistent way of handling the conversation.

- Global Text Telephone access

Within the capabilities of networks and terminals, the user shall be able to experience consistent access to the GTT regardless of the access point.

For example the user should be capable of accessing the text conversation features through a number of different access points, which should include 3G and 2G networks, fixed networks, the Internet etc..

- Interoperability

The GTT shall support a minimum set of functionality to ensure interoperability with existing and emerging text telephone systems and text conversation features.

The GTT shall provide a minimum set of environments where text conversation is supported to ensure full interoperability between different terminals and networks from the very beginning of service provisioning.

The real-time global text telephone feature shall be supported using the standardised set of service capabilities..

# General Requirements

Network operators have many differing requirements, and GTT shall be supported in the network in a manner which allows network operators to consider different configurations depending on their network and commercial requirements.  Thus, an identified set of functionalities and formats shall be standardised to ensure interoperability across networks and terminals to support GTT.

However, some network operators may wish to design and configure networks in different ways, and the subsequent requirements are identified to allow flexibility in how the GTT functionality is supported.  For example in some networks the network operators may wish to implement the GTT functionality within the core network, whereas other may wish to place the GTT functionality on the periphery of the core network (e.g. a centralised network model instead of a distributed architecture).  Further, some network operators may wish to support a limited set of GTT functionality, while others may require extensive and elaborate GTT support according to their business models (e.g. basic GTT instead of advanced GTT).  Interoperability shall always be maintained within this flexible architecture.

The following sub-clauses use the term "The GTT shall be able to support a request for …" and similar phrases to allow network operators to consider these different network models and business requirements, to permit flexible architectures and ensure GTT interoperability.

The following sub-clauses use the term "This requirement shall be supported at the application layer in the terminal (and/or the network), and will not be further elaborated.” and similar phrases to identify those service requirements that shall be supported by GTT but do not require standardisation.

The following general requirements shall be supported via the use of service capability features.

## Text conversation management

It shall be possible to notify in the subscriber data base that a user has got the authority to use GTT.

## Text conversation handling during calls

Text telephony should be enabled only if more than one terminals in a call can support text telephony.

It should be possible to start text conversation during a call.

Text transmission shall be done character by character as entered, or in small groups transmitted so, that no character is delayed before transmission more than 0.5 seconds. (as stated in ITU-T T.140)

The rate of transmission should allow 10 characters per second for US and Western Europe with normal character arrangement.

## Alerting and Answering

Since many GTT users are deaf or hard-of-hearing, the terminals used for GTT shall provide alternative alerting signals.

## Addressing

GTT shall support different addressing formats to identify the different kinds of endpoints of the call (see section 11, interworking.)

## Roaming

It shall be possible to implement GTT so, that calls to and from mobile stations supporting GTT are routed through GTT specific service elements in the home network to provide the feature. The visited network need no specific GTT support for that configuration.

# Profile

# Security and reliability

The user shall be able to use and access GTT in a secure manner. It shall be possible for the contents of GTT sessions to be read only by the intended recipient(s).  A recipient shall be informed of the reliability of the identity of the sender in case the sender has authorised his identity to be transmitted.

The integrity of GTT during transmission shall be assured to extent of the network capabilities.

The GTT shall be intrinsically resistant to attempts of malicious or fraudulent use.

The “Security Threats and Requirements” specified in 22.133 [3] shall not be compromised.

The character corruption rate should be less than 1% in conditions where users experience the voice transmission quality to be low.

# Charging

The GTT shall be able to support various charging mechanisms. The following charging characteristics may be considered:-

- duration

- roaming conditions

- location conditions

# External Interface

External interfaces for controlling and operating GTT between the terminal and an external digital device e.g. portable computer should be supported.

External interfaces for controlling and operating GTT between the terminal and an external analogue text telephone device may be supported.

# Interworking

The standard shall permit interworking with other existing or emerging text conversation technologies, either located within or outside a mobile network. It shall also permit interworking with messaging services, intelligent network services and  use of supplementary services.

The modes and methods for interworking are specified in ITU-T H.248 Annex F; packages for Text Conversation and Text Telephony. [4]

For mobile services, interworking should be arranged between different forms of GTT, e.g. voice band based, data channel based, CS multimedia based and PS miltimedia based services.

The mobile station shall be able to initiate calls, receive calls, add text later in the call, alternate between voice and text and use text and voice simultaneously for those technologies that enable it.

For interworking with mobile messaging services, it is sufficient if the text telephone users in the fixed networks can initiate and receive messages.

# Initial requirements

This section is informative and specifies the initial requirements that formed the base of definition of GTT. They are included here as information only. The 3GPP GTT specifications together define what requirements will be met.

Requirements within brackets [ ] are for discussion.

Most requirements in this chapter are derived from the requirements expressed to be urgent by the FCC and the CTIA TTY forum in USA. The relation with these requirements are documented for each requirement.

## R&O on Transmission Performance

A1: The printable character error rate (PER) shall be less than [1%] for stationary calls under nominal radio conditions, i.e. where also speech calls show an acceptable quality (TTY/FCC 1).

A2: The printable character error rate (PER) shall be less than [1%] for calls with pedestrian speed or typical vehicle speed under nominal radio conditions (TTY/FCC 13, TTY Forum, extended).

A3: An output volume control should be provided in order to allow adaptation to existing Text Telephone equipment for optimal receiving quality (TTY/FCC 4). 

A4: The input range shall be automatically adapted to the proper receiving level (new).

A5: The Text Telephone shall allow to transmit Speech and Text (VCO/HCO) in an automated way, without user interaction, either by alternating between speech and text mode or by transmitting speech and text in parallel (objective) (TTY/FCC 9, extended).

A6: The delay of the speech path shall not exceed that of a normal speech call in that situation by more than [40ms] in one way, if the text telephone functionality is activated. Otherwise [no] additional delay shall occur to the speech signal. The delay of the speech path should not vary during a speech phrase.

A7: The delay of the text path shall not exceed that of the speech path by more than [1000 ms] (one way) at typing speed of [7] characters per second. The delay of the text path may be variable.

A8: The time of switching between Text and Speech mode shall not exceed [1000ms] at typing speed.

A9: The printable character throughput shall not be smaller than [10] characters per second under nominal radio operating conditions (TTY/FCC 10, extended).

A10: Under impaired radio conditions the character throughput may be reduced in order to achieve the desired printable character error rate (TTY/FCC 10, modified).

A11: The signals used for transmission should pass existing fixed and wireless “analogue” speech channels with a printable character error rate below [1%] under nominal radio conditions, if two devices are connected “back to back”.

A12: The data transmission bit rate should be as high as possible under the given radio conditions with the objective of [300] Bit/s with a bit error rate below [10^-3] and a residual bit error rate below 
[10^-5] (after removing detected bit errors).

A13: It shall be possible to transmit both, single characters, as well as long text or data strings (from file) efficiently.

A14: The transmission mechanism should provide data flow control to adapt the source data rate (when in file transfer mode) to the varying radio transmission conditions, in order not to loose data on the path.

A15: The character coding shall allow transmission of characters from any language in a consistent way.

## R&O on Man Machine Interface (MMI)

These Requirements and Objectives are to some extend depending on implementation and not on the transmission standard. This list shall by non means restrict the innovation capabilities of vendors, but give guidelines and define a minimum set, against which the transmission standard needs to be checked.

B1: The Text Telephone user must be able to monitor all aspects of call progress (same or more information as provided to voice users) by tones and visualisation (TTY/FCC 2). The transmission standard shall provide the necessary monitor information to the MMI.

B2: There must be an indication, by tones and visualisation, when the call is connected or disconnected (TTY/FCC 3).

B3: Call information such as caller identification, where provided in mobile voice services, should also be provided for Text Telephony calls (TTY/FCC 11 adapted).

B4: The Text Telephone system must be able to send “Text Tones” to a normal telephone user, to indicate that he is using a text telephone, even if the other user has only a normal phone (TTY/FCC 6 adjusted).

B5: The Text Telephone user must have a means of tactile (vibrating) ring signal indication. (TTY/FCC 5), besides an acoustical and optical ring signal indication (new).

B6: Emergency calls (e.g. to 112 or 911 numbers) shall not require any further user interaction than for any normal voice call (except to connect the possible additional equipment) (new).

B7: Call back from Emergency Call Centres should not require any further user interaction than for any normal voice call (except to connect the possible additional equipment) (new).

B8: Call setup to and from other Text Telephone users of the same type according to the new standard should not require any further user interaction than for any normal voice call (new).

B9: Call setup to and from other Text Telephone users of an other kind ( e.g. those defined in ITU V.18) may require some user activity, like

- sending a short additional precode (e.g. [2..3] digits) or

- typing a short digit sequence to invoke the service (e.g. like [#55*]) or

- (on user´s preference) the permanent subscription to such a service

- a capability signalling by the mobile

- activating a text telephone application

- others

B10: The new standard should allow the usage of ordinary unmodified mobile phones as already available to the user or on the mass market, in order to get the advantage of high volumes price level.

B10: The new standard shall allow to hide the intermediate (with existing phones) necessary user interaction, as described in the items before, in modern equipment (automatic service).

B11: The use of the Text Telephone for emergency calls with unregistered phones or without SIM card shall be possible as for normal voice calls.

B12: The interface to the possible adapters shall be exactly specified with

- mandatory interfaces (as to allow interaction to existing Text Telephones and the basic access to the mobile phone) and

- optional interfaces (as to connect microphone and loudspeaker, the control port of the mobile phones, …)

## R&O on Compatibility

C1: The standard shall be compatible to equipment on the landline side as specified in ITU recommendation V.18 (TTY/FCC 12, extended), including all annexes of V.18 and V.18 with V.61 for voice and text simultaneously

C2: The landline party’s Text Telephone equipment shall not need modifications or additions in order to be compatible and to achieve the desired error rate (TTY/FCC 7).

C3: It shall be possible to deploy the Text Telephone standard world wide in all GSM and UMTS networks (new).

C4: The wireless Text Telephone (i.e. on the Mobile user side) may require modifications or additions or the development of new equipment (TTY/FCC 8). The smaller the modifications or additions the better.

C5: Roaming between networks of different operators of the same kind of wireless technology shall be possible, provided the operators has installed the service.

C6: Is should be possible to connect equipment implementing the standard to any existing mobile phone of any existing wireless standard.

C7: Communication between Text Telephones in different kinds of wireless technologies shall be possible.

C8: Communication with text between mobile text telephones and multimedia devices with text shall be possible.

## R&O on Complexity of Implementation and Roll Out

D1: The standard shall allow a fast first step implementation and roll out

D2: The standard shall allow a smart integration into the mobile phone or the Text Telephone terminal or an integration of everything together into a single device.

D3: The interaction between the new and existing standards may require a network based server, which may be deployed in different steps in different levels of integration as user demand grows over time.
Note: since at start up of the service no or only few users with this new Text Telephone standard will be online, a SW-based solution may be a reasonable, fast rolled out step, while high volume traffic (after some time) will possibly require more cost efficient solutions.

- Annex A:
Change history

| Change history |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TSG SA# | Spec | Version | CR | <Phase> | New Version | Subject/Comment |

# History

| Document history |  |  |
| --- | --- | --- |
| V. 0.0.1 | April 2000 | First Draft (Presented at TSG-S1 10th – 15th April 00) |