| 3GPP SA1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Document |  |  |  |  |  |  |  |  | S1-000252 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sophia Antipolis, France, 5-6 April 2000 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 22.078 |  |  |  | CR |  |  | xxx |  |  |  |  |  | Current Version: |  |  |  |  |  |  |  |  |  | 3.3.0 |  |  |  |  |
| For submission to: |  |  |  |  |  |  |  | for approval |  |  |  |  |  | X |  |  | strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | for information |  |  |  |  |  |  |  |  |  |  | non-strategic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  | (U)SIM |  |  | ME |  |  |  |  | UTRAN / Radio |  |  |  |  |  |  |  |  |  |  | Core Network |  |  |  |  |  |  | x |  |
| Source: |  |  | Ericsson |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  | 05/04/2000 |  |  |  |  |  |
| Subject: |  |  | Clarification for conditional triggering for subscribed dialled services |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item: |  |  | CAMEL Phase 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Category: | F Correction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  |  |  |  | Phase 2 |  |  |  |  |  |  |
|  | A Corresponds to a correction in an earlier release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 96 |  |  |  |  |  |  |
|  | B Addition of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 97 |  |  |  |  |  |  |
|  | C Functional modification of feature |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 98 |  |  |  |  |  |  |
|  | D Editorial modification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  | Release 99 |  |  |  | x |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Release 00 |  |  |  |  |  |  |
| Reason for change: |  |  | The present CR proposes the inclusion of a clarifying text on conditional triggering for subscribed dialled services. When conditional triggering for a subscribed dialled service is performed, the number that shall be used for the conditional triggering check may have been modified by a previous CAMEL service, which was invoked when digits were collected. This behaviour needs to be properly documented in the stage 1 document. This CR proposes the clarifying text in sect. 5.2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 5.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | Other 3G core specifications |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  | Other GSM core specifications |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MS test specifications |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BSS test specifications |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | O&M specifications |  |  |  |  |  |  |  |  |  |  |  List of CRs: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Other comments: | The same behaviour applies for Serving network dialled services. This behaviour is already described in the stage 1 document (chapter 7). |
| --- | --- |

****First Modified Section ****

## 5.2	Criteria for contact with the CSE $(CAMEL2$)

It shall be possible for the HPLMN to specify criteria which must be satisfied before the CSE is contacted.

The following criteria may be defined.

### 5.2.1	CSI criteria applicable at call setup

#### 5.2.1.1	CSI criteria applicable at call setup when dialled digits have been collected $(CAMEL2$)

CSI criteria may be defined for a subscriber for the case where collection of dialled digits has been performed $(CAMEL3$).

-	Criteria on the dialled number; these consist of:

-	The contents of the dialled number (a list of up to 10 dialled number strings may be defined in the criteria. Each dialled number string may be of any type of number (TON) format. supported by the access protocol); There is no restriction on the Number Plan Indicator (NPI).

-	The length of the dialled number (a list of up to three lengths may be defined).

-	The criteria on the dialled number may be collectively defined to be either "enabling" triggering criteria or "inhibiting" triggering criteria (see below). The HPLMN may also choose not to define any criteria on the dialled number.

-	A criterion on the basic service: this consists of a list of up to 5 basic service codes for individual basic services or basic service groups. The HPLMN may also choose not to define any criterion on the basic service.

-	A criterion on the type of call: this consists of defining whether or not the call must be a forwarded call.

A call is treated as forwarded in this respect when either a forwarding supplementary service applies or when the call is forwarded as a result of a terminating CAMEL based service. The HPLMN may also choose not to define any criterion on the type of call.

If the criteria on dialled number are "enabling" then the dialled number criteria are satisfied if:

-	the dialled number matches a dialled number string defined in the criteria; or

-	the length of the dialled number matches a dialled number length defined in the criteria.

If the criteria on the dialled number are "inhibiting" then the dialled number criteria are satisfied if:

-	the dialled number does not match any of the dialled number strings defined in the critera; and

-	the length of the dialled number is not the same as any dialled number length defined in the criteria.

In these tests the dialled number matches one of the dialled number strings if:

-	the two numbers are of the same Type Of Number (TON); and

-	the two numbers have the same Number Plan Indicator (NPI); and

-	the dialled number is at least as long as the dialled number string in the criteria; and

-	all the digits in the dialled number string in the criteria match the leading digits of the dialled number.

If no criterion on the dialled number is specified then the dialled number criteria are satisfied.

The criterion on the basic service is satisfied if the basic service used for the call corresponds to any basic service code or basic service group defined in the criterion or if no basic service criterion is specified.

The criterion on the type of call is satisfied if the type of the call is the same as the type defined in the criterion or if no call type criterion is specified.

The criteria on the call setup event procedure are satisfied if:

-	the criteria on the dialled number are satisfied; and

-	the criterion on the basic service is satisfied; and

-	the criterion on the type of call is satisfied.

#### 5.2.1.2	CSI criterion applicable at call setup for subscribed dialled services $(CAMEL3S)

A CSI criterion on the contents of the called number shall be defined for subscribed dialled services. A list of up to 10 called number strings may be defined in the criterion. Each entry in the called number list has associated with it a CSE identity and a service key which defines the service to be triggered if the criterion is satisfied.

If any other CAMEL dialogue has changed the called number, then the modified called number shall be used for the conditional triggering check.

The called number criterion is satisfied if the called number matches a called number string defined in the criterion.

In this test the called number matches one of the called number strings if:

-	the two numbers are of the same Type Of Number (TON); and

-	the two numbers have the same Number Plan Indicator (NPI); and

-	the called number is at least as long as the called number string in the criteria; and

-	all the digits in the called number string in the criteria match the leading digits of the called number.

#### 5.2.1.3	CSI criterion applicable on detection of unsuccessful call establishment $(CAMEL3$)

A criterion on the release cause may be defined. This consists of a list of up to 5 cause values. The criterion on the release cause is satisfied if the received call release cause corresponds to any cause value defined in the list or if no criterion is defined.

If any other CAMEL dialogue has changed called party number then the modified called party number is used for conditional triggering check.