TITLE:	Handover Requirements for Rel'00

DATE:	March, 30th  2000

RECOMMENDATION:	For discussion at the S1 ad-hoc cyber-meeting

SOURCE:	Erwin Postmann

SIEMENS AG

Email address:		erwin.postmann@siemens.at

Office Phone #:		+43.51707.21398

Mobile Phone #:		+43.664.1010034

FAX #:			+43.51707.51924

1. Introduction

This contribution provides the SIEMENS point of view regarding the Handover Requirements for Rel'00 and is proposed as input for the TR 22.976.

The following possible handover scenarios were identified:

- CS to CS (voice service)

- PS to PS

- IP based multimedia service

- real time IP connectivity services (conversational/streaming  service classes)

- non-real time IP connectivity services (interactive/background service classes)

- CS to PS / PS to CS

2. Definitions

The following terminology agreed at the last S2 meeting (S2 TDocs S2-000537 and S2-000544) is used within this document.

CS Services: 	Telecommunication services provided to "GSM/ISDN" clients via 24.008 CC. CS services will be provided in both R99 and R00 with "normal evolution" of services between releases.

PS Connectivity Services: 	IP connectivity service provided to IP clients via 24.008 SM. PS Connectivity Services will be provided in both R99 and R00 with "normal evolution" of services between releases.

IM Services: 	IP Multimedia Services that require support on the Call Control level carried on top of the PS connectivity services (this may include an equivalent set of services to the relevant subset of CS Services). IM Services are provided by UMTS in R00 but not in R99

PS services: 	Services provided by a UMTS packet network. In R99 it consists of PS connectivity services, in release 2000 it is the superset of IM services and PS connectivity Services. Also services provided by the GERAN in R00 are referred to as PS services.

2G-GSM-CS Services	Services provided by a 2nd Generation GSM network, regardless of the access technology, in particular telecommunication services provided to "GSM/ISDN" clients via 04.08 / 24.008 CC.

2G-GSM PS Services	Services provided by a 2nd Generation GPRS network, regardless of the access technology, in particular IP connectivity Service provided to IP clients via 04.08 / 24.008 SM.

NOTE: 	The teleservice speech may be in R00 either a CS (provided by the CS CN domain) or a PS service (provided by the PS CN domain / IM CN subsystem).

3. Handover Scenarios

3.1 CS to CS

For this handover scenario only voice services are analysed. In case of CS Multimedia Services (CS MM support of H.324) only the voice part shall be considered.

| from/to | UMTS CS R99 and R00 | UMTS CS MM R 99 and R00 | GSM R99 and R00 |
| --- | --- | --- | --- |
| UMTS CS R99 and R00 | ##### R99/00 | NO (complex, due changing handling single media streams instead of voice) | ##### R99/00 |
| UMTS CS MM R99 and R00 | NO (complex, due to splitting of media streams and reducing to voice only) | ##### R99/00 | ##### NO |
| GSM R99 and R00 | ##### R99/00 | ##### NO | ##### R99/00 |

3.2 PS to PS

For the PS to PS handover cases we have to distinguish between handover for IP multimedia service, handover for real time IP connectivity Services and handover for non-real time IP connectivity services.

3.2.1  IP Multimedia Services

The handover of IP Multimedia Services is in general supported if the IP connectivity services provided  by the involved networks support handover for conversational/streaming (real time) service classes. This is available within UMTS PS R99/R00 (UTRAN R99/00, GERAN R00) networks and for combined UTRAN/GERAN networks and therefore such a handover shall also be supported.

Due to that these service classes are not supported by the GSM PS (GPRS) within R99/R00 no handover to and from GSM PS (GPRS R99/R00) is possible.

NOTE:	The "voice-only" service is seen as a special case of the IP Multimedia service

3.2.2  real time IP connectivity services

The handover of  real time IP connectivity services (conversational/streaming service classes) is in general possible between all combinations of   UMTS PS R99/00 (with UTRAN R99/00 and /or GERAN R00) networks and shall be supported.

The handover is not possible to and from GSM PS (GPRS) R99/00 (using a GSM BSS), due to the lack of support for the real time IP connectivity services within GSM PS (GPRS) R99/00 (using a GSM BSS).

3.2.3  non-real time IP connectivity

The handover of  non real time IP connectivity services (interactive/background service classes) is in general possible between all combinations of   UMTS PS R99/00 (UTRAN R99/00, GERAN R00) networks and also to and from GSM PS (GPRS) R99/00 (using a GSM BSS) and shall be supported.

3.3 CS to PS and PS to CS

The most controversial handover scenarios is a handover between a UMTS PS service "voice-only" (special case of IP Multimedia Service) towards the CS Service "Telephony" (3rd and/or 2nd Generation).

Due to the complexity of CS <> PS handover it is proposed to study the market relevance for this scenario in R01+. In parallel the technical solution can  be analysed (e.g. by S2, RAN).

This kind of handover shall not be required for Rel00 for a realistic scope within the R00 timeframe, but it shall be studied forRel01+.

In the following some of the open items for the support of this kind of handover scenarios are given:

- How are the PS sessions mapped onto CS calls ?

- Which sessions are handovered and which are dropped ?

- How shall the UE be notified that it is handled by the other domain ?

- How do the CS and PS stacks interwork within the UE ?

- How can the discontinuity due to handover be reduced to an acceptable value (e.g. better than GSM speech ) although there are various entities involved in the procedure (e.g. UE, RNS, BSS, SGSN, GGSN, CSCF, MGCF, MGW, MSC, SGW) ?

- Within the PS domain there is a clean separation of call control and bearer control, however for a handover to the CS domain both call control protocols would need to be involved. How shall the PS bearer control and call control interwork ? (i.e. How does the SGSN determine which sessions involve the CSCF ?) How shall the PS and CS call control interwork (i.e. 04.08 CC and H.323/SIP) ?

- If the call was to a multimedia IP network, there is no MGW in the transport path in the first place, but a MGW is necessary for media conversion towards a 2G network.

- The scenario is interesting for greenfield operators, but in that case the scenario is an inter network handover as well. Thus, other issues such as charging and identifier and address availability of target entities need to be solved first.