3GPP TSG-SA1 Meeting #102	S1-231124

# Berlin, Germany, 22nd May 2023 - 26th May 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.268 | CR | 0079 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE based geo-fencing requirements for KPAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, KT Corp, SK Telecom, LG Uplus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The requirements to support UE based geo-fencing are already defined in TS 22.268 and this feature is implemented on many mobile phones. The Korean Ministry of Interior and Safety(MOIS) has decided to provide PWS using UE based geo-fencing to optimize the accuracy of receiving warning messages by ensuring that warning messages are sent only to affeted areas. However, the requirements regarding UE based geo-fencing are not yet specified in the KPAS specific clause. Therefore, this CR proposes to add these requirements to clause 8. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In clause 8.2 - add requirements to provide PWS using UE based geo-fencing for KPAS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some important KPAS requirements are not fully covered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------- Start of Change 1 -----------------------------------

8.2	Additional PWS Requirements Specific to Korean Public Alert System (KPAS)

In addition to the General Requirements specified in Clause 4, the following requirements are specified for the Korean Public Alert System (KPAS) [7]:

-	Two classes of Warning Notification shall be supported; class 0 & class 1.

-	The classes are differentiated per opt-out function. Class 0 shall have no opt-out and class 1 shall allow opt-out by the user.

Different types of Warning Notification shall be supported: Extreme Emergency, Emergency, Public Safety, and Amber. The Extreme Emergency type Warning Notifications are defined as class 0 and the other types are defined as class 1.

The characteristics of the Warning Notification shall follow the requirements specified in clause 4.

The current implementation requirement is for the message of up to 90 characters text. In addition, Warning Notifications using the UCS2 coding scheme [9] shall support up to 157 characters and Warning Notifications using the GSM 7 bit Default Alphabet coding scheme [9] shall support up to 360 characters from LTE onwards.

KPAS shall support Warning Notifications in various languages. If a Warning Notification is received in a language other than Korean, then it shall only be displayed by the UE if the user has selected that language.

A PWS-UE shall display Warning Notifications whose first texts are dedicated words when a PWS-UE receives Warning Notifications in other languages than Korean.

NOTE 1:	Dedicated words are specified in the regulations of Korean Ministry of Interior and Safety.

A PWS-UE in KPAS shall support two dedicated alerting indication (audio attention signals).

NOTE 2:	Two dedicated audio attention signals are specified in the regulations of Korean Ministry of Interior and Safety.

Warning Notifications may contain URLs and/or phone numbers. A UE shall allow the user to navigate to the URL or initiate a voice call to the phone number which may be included in the Warning Notification.

KPAS shall support that the PLMN broadcasting KPAS Warning Notifications also broadcasts corresponding Warning Notification Area(s).

A KPAS-capable UE shall utilize UE-Based Location Calculation (see 3GPP TS 22.071 [11]) to determine whether it is located within the corresponding Warning Notification Area(s) in order to minimize overloading network LCS resources.

---------------------- End of Change 1 ----------------------------------------