3GPP TSG SA WG 1 Meeting #102 	S1-231299

Berlin, Germany,  22 - 26 May 2023	(revision of S1-23xxxx)

Source:	Huawei

pCR Title:	Pseudo-CR on FS_Ambient IoT, update Max. allowed end-to-end latency KPI of clause 5.11 medical instruments status modification

Draft Spec:	3GPP TR 22.840 v1.1.0

Agenda item:	7.2

Document for:	Approval

Contact:	Shuang ZHANG (zhang.shuang2@huawei.com)

Abstract: This document proposes to update max allowed latency KPI to clause 5.11 medical instruments status modification in TR 22.840 v.1.1.0.

1. Introduction

Clause 5.11 has been agreed and captured in TR 22.840, which introduces a communication service that enables user to use medical instrument management platform to remotely modify status information of Ambient-IoT device attached medical instrument remotely.

Service Flow captures operation a user to acquire and/or change status information of Ambient IoT attached medical instrument. As this service is not time-critical, the user does not need hundreds of ms level response time. The experience of service level latency is more suitable to be specified at the level of several seconds.

2. Reason for Change

Currently the KPI value of “Max. allowed end-to-end latency” is captured as “hundreds ms level” in the KPI Table 5.11.6-1. This value need to be updated to specific the actual needs of the service more accurately.

3. Conclusions

Update KPI Table 5.11.6-1 by updating the value of Max. allowed end-to-end latency from “hundreds ms level” to “Typically several seconds”.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.840 v1.1.0.

* * * First Change * * * *

### 5.2.6	Potential New Requirements needed to support the use case

[PR 5.2.6-001] The 5G system shall be able to communicate with an Ambient-IoT device.

[PR 5.2.6-002] The 5G system shall be able to provide group communication for a group of Ambient-IoT devices.

[PR 5.2.6-003] The 5G system shall be able to provide a mechanism to expose the information collected from an Ambient-IoT device to a trusted 3rd party.

[PR 5.2.6-004] The 5G system shall be able to support positioning for an Ambient-IoT device.

[PR 5.2.6-005] The 5G system shall be able to provide communication service with KPIs listed in Table 5.2.6-1 for the Ambient IoT device/s.

Table 5.2.6-1: KPIs for use case of Medical Instrument Inventory management

| Scenario | Max. allowed end-to-end latency | Communication Service Availability | Reliability | User-experienced data rate | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Medical instrument inventory management and positioning | Typically several seconds (note 2) | 99% | NA | <2Kbps (note 1) | 176bits | ≥1000/km2 (note 3) | 50m Indoor 200m Outdoor | NA | Static or walking speed <6km/h | NA | NA | NA | NA |
| Note 1: User experienced data rate is calculated based on inventory information (172 bits) within time period of e.g. 100 ms; Note 2: It is one trip delay between an Ambient-IoT device and the interface of application platform. Note 3: It refers typical medical instrument density condition in Chinese hospital. |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * End of the First Change * * * *