3GPP TSG SA WG 1 Meeting #104 	S1-233057

Chicago, US, 13 - 17 November 2023	(revision of S1-23XXXX)

Source:	Xiaomi

pCR Title:	Adding a new requirement to section 5.2.2 Configuration and authorization

Draft Spec:	3GPP TS 22.137

Agenda item:	7.1.2

Document for:	Approval

Contact:                     Dong Chen chendong7@xiaomi.com

Abstract: This document introduces a new requirement to Configuration and authorization subsection TS 22.137

1. Introduction

This pCR introduced a new requirement to section 5.2.2 configuration and authorization into the new TS on Sensing.

2. Reason for Change

Based on the discussion and output in the study report TR 22.837, a new requirement “Based on location, the 5G network shall be able to ensure that sensing transmitters and sensing receivers use licensed spectrum only in network coverage and under the full control of the operator who provides the coverage.” has been introduced in the Configuration and authorization subsection of TS 22.137. It is proposed to introduce the requirement to TS 22.137 as well.

3. Proposal

It is proposed to agree the following change to 3GPP TS 22.137.

* * * First Change * * * *

5.2.2 Configuration and authorization

Subject to regulation and operator’s policies, the 5G network shall be able to configure and/or authorize or revoke authorization of sensing transmitter(s) and sensing receiver(s) for 5G wireless sensing service.

NOTE 1: Such configuration and authorization can be based on sensing transmitter or sensing receiver location, specific time, sensing duration, sensing accuracy, target sensing geographical area, establishing of communication to transfer sensing data, etc.

NOTE 2: Such configuration and authorization can also include the selection of multiple sensing transmitters/receivers for 5G wireless sensing services.

The 5G network shall be able to provide a mechanism for an MNO to configure UEs supporting V2X applications to support 5G Wireless sensing service when not served by RAN.

Based on location, the 5G network shall be able to ensure that sensing transmitters and sensing receivers use licensed spectrum only in network coverage and under the full control of the operator who provides the coverage.

NOTE 3: The above requirement does not apply for public safety and V2X networks with dedicated spectrum, where 5G wireless sensing can be allowed out of coverage or in partial coverage as well.

* * * End of Changes * * * *