3GPP TSG SA WG 1 Meeting #106		 S1-241153

Jeju , Korea, 27-31 May 2024

Title:	[DRAFT] Reply LS on traffic steering and/or switching of user data across two 3GPP access networks

Response to:	LS S2-2403670 on traffic steering and/or switching of user data across two 3GPP access networks from SA2

Release:	Release 19

Work Item:	FS_MASSS

Source:	SA1

To:	SA2

Cc:	CT1

Contact person:	Qing Wan

wanqing1@cictmobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1	Overall description

SA1 thanks SA2 for the LS S2-2403670 asking for clarification on the impact to PLMN selection by the requirement of traffic steering and/or switching of user data across two 3GPP access networks defined in 3GPP TS 22.261.

The mentioned requirement is applicable for all the scenarios below(as depicted in clause 6.50.1 3GPP TS22.261).

Target scenarios cover two 3GPP access networks belonging to the same PLMN, or between two different PLMNs, or between one PLMN and one PLMN-integrated NPN, over same or different RAT, which can use terrestrial and/or satellite access (including the case of two different satellite orbits).

Depending on the scenario, the network selection can be enhanced to support a DualSteer device connecting to two 3GPP access networks belonging to two different PLMNs, or one PLMN and one NPN subject to the policies and criteria of the home network operator.

SA1 thinks the requirement implies the impact to network selection.

2	Actions

To SA2

ACTION: 	SA1 kindly asks SA2 to take the above information into account.

3	Dates of next TSG SA WG 1 meetings

SA1#107	19 - 23 Aug 2024			Netherlands, Maastricht

SA1#108	18-22 Nov 2024			Orlando, Florida, USA