3GPP TSG SA WG 1 Meeting #107 	S1-242041

Maastricht, The Netherlands, 19-23 August 2024	(revision of S1-24xxxx)

Source:	IPLOOK

pCR Title:	Pseudo-CR: Use Case on Resilient Notification and Pre-notification to OTA update

Draft Spec:	3GPP TR 22.887 v0.1.0

Agenda item:	7.3

Document for:	Approval

Contact:	Junle Liao liao.junle@iplook.com

Abstract: This contribution proposes a use case of satellite access applied in vehicle OTA update Resilient Notification Service, Pre-notification Service, and potential service requirements for TR 22.887.

1. Introduction

To ensure the continuity and reachability of the vehicular communication such as OTA update in the satellite access system, this use case proposes to consider providing Resilient Notification Service and Pre-notification Service as requirements.

2. Reason for Change

Introduce a use case about Resilient Notification Service and Pre-notification Service with satellite access only applied in vehicular communication to FS_5GSAT_Ph4.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed that the following changes to 3GPP TR 22.887 v0.1.0 be agreed upon.

* * * First Change * * * *

5 Use cases

## 5.X	Use case on Resilient Notification and Pre-notification to OTA update

### 5.X.1	Description

OTA (Over-the-Air) technology is being used to provide new firmware updates to Electric Vehicles (EVs). OTA updates improve the driving experience and safety. Based on V2N (Vehicle-to-Network) technology, EVs can seamlessly connect to mobile networks such as 5G. In urban areas, terrestrial networks have a good coverage of vehicles. Thus, those vehicles in urban can receive OTA update notifications and finish downloading through terrestrial network access. However, for those vehicles in rural or remote areas, the only option is satellite access. Due to incomplete coverage and satellite mobility (LEO, MEO), EVs may miss OTA update notifications when satellite access is unavailable in remote areas. In the update process, they may suspend or terminate downloading due to the satellite coverage gap. Therefore, to ensure the continuity of the OTA update download and reachability of OTA update notification in the satellite access system, it requires “Pre-notification Service” and “Resilient Notification Service”. “Pre-notification Service” is to pre-notify EVs before the vehicle's terminal moves out of the satellite coverage so EVs can store the data before disconnection. With “Resilient Notification Service”, EVs can receive missed OTA notifications when satellite access is reconnected.

### 5.X.2	Pre-conditions

Satellites (SAT#1, SAT#2, SAT#N) were deployed by the same satellite operator SATNET.

An Electric Vehicle (EV#1) is capable of LEO and MEO satellite access. EV#1 has deployed OTA service. The operator SATNET has deployed the Resilient Notification Service and the Pre-Notification Service for satellite access vehicles.

The network is capable of calculating satellite coverage information of  EV#1 to recognize when is the leaving time of EV#1 approximately.

### 5.X.3	Service Flows

- EV#1 is driving to a rural area with satellite coverage. The rural area now is in winter and the control center plans to propose an OTA update for energy optimization in cold regions. Based on the location information and the low-temperature warning sent from EV#1’s sensors, update lists include EV#1.

- The control center proposes an OTA update notification. However, EV#1 entered a satellite coverage gap that cannot receive any OTA notification. The network activates the “Resilient Notification Service” to ensure the OTA update notification can reach EV#1 eventually after the next satellite coverage. Then EV#1 is reconnected to the next satellite and starts to process the OTA update as Fig 5.X.3-1 shows.

- When EV#1 starts to download the OTA update data, the satellite may send a pre-notification to EV#1 if EV#1 is about to leave the satellite coverage area. This may allow EV#1 to process the highest priority tasks and back up the download session to ensure service continuity as Fig 5.X.3-2 shows. Then EV#1 suspends the OTA update session and continues downloading when the next satellite reconnected.

- Fig 5.X.3-1 Resilient Notification Service Ensure Reachability of OTA Update Notification

- Fig 5.X.3-2 Pre-notification Service Ensure Continuity of OTA Update Process

### 5.X.4	Post-conditions

EV#1 did not miss any OTA update notifications and finished the OTA update steadily in rural areas. The driver was happy that the car updated immediately to respond to different road environments.

### 5.X.5	Existing features partly or fully covering the use case functionality

·The 3GPP system shall allow UEs supporting V2X applications to use NR for direct communication when the UEs are not served by a RAN using NR. [Reference: TS 22.186]

·The 5G system shall support a mechanism for a 3rd party application to request resilient timing with specific KPIs (e.g., accuracy, interval, coverage area). [Reference: TS 22.261]

### 5.X.6	Potential New Requirements needed to support the use case

[PR 5.X.6-1] Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support Resilient Notification Service to notify the vehicle terminal of a missed notification when the vehicle terminal is unreachable via satellite access.

[PR 5.X.6-2] Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall support Pre-notification Service to pre-notify the vehicle terminal of satellite coverage leaving time before the vehicle is driving to the edge of coverage.

[PR 5.X.6-3] Subject to regulatory requirements and operator’s policy, a 5G system with satellite access shall provide a secure mechanism for OTA updates.