3GPP TSG SA WG 1 Meeting #107 	S1-242054

Maastricht, The Netherlands, 19-23 August 2024	(revision of S1-24xxxx)

Source:	CATT

pCR Title:	Pseudo-CR on Update to 5.5 Use case

Draft Spec:	3GPP TR 22.887 v0.1.0

Agenda item:	7.3

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution proposes update to the potential requirements of use case on support for MBSRs through multi-orbit satellite networks in TR22.887 clause 5.5.

1. Introduction

The use case of supporting mobile base station relays through multi-orbit satellite networks was approved and included in TR22.887. However, the requirements still need further study. The update is proposed to clarify the dependency on existing features and solve the editor’s note.

2. Reason for Change

Update the gap analysis and potential requirements to solve the editor’s note.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree on the following changes to 3GPP TR22.887 v0.1.0

* * * First Change * * * *

5.5	Use case on support for Mobile base station relays (MBSRs) through multi-orbit satellite networks

5.5.1	Description

Mobile base station relay (MBSR) is a good option for providing seamless connectivity to moving UEs in a vehicle. MBSRs can be connected to the network through non-terrestrial networks (NTN). NTN deployments are essential for providing connectivity to relays in isolated and remote areas. The 5G system supports a multi-dimensional heterogeneous architecture, incorporating terrestrial access, low altitude aerial devices (e.g., UAVs), High Altitude Platform Station (HAPS) and satellites (Low Earth orbit (LEO), Medium Earth Orbit (MEO) and GEO)) to meet varying connectivity requirements. Different types of satellites in various orbits can serve different services based on their latency requirements. For example, GEO satellites can support services like video streaming (buffered), email, chat, file sharing while LEO satellites are better suited for services requiring low latency like V2X messaging and real-time gaming. With the movement of relays and depending on the requirements of connected UEs, switching between multi-orbit satellites can ensure all use case scenarios covered using NTN deployments only (as shown in Figure 5.5.1-1).

Figure 5.5.1-1: Multi-orbit satellite networks support for Mobile base station relays (The moving vehicle in the figure represents various scenarios for a mobile relay, whether it is on a ship, train, or aeroplane)

5.5.2	Pre-conditions

MBSR is connected through GEO satellite.  The service provider’s 5G system supports switching from GEO to available LEO connectivity based on service requirements.

5.5.3	Service Flows

Initially, the MBSR is connected to the network through GEO satellite links. When the vehicle moves where LEO coverage is available, the MBSR remains connected to GEO. However, since GEO cannot provide low-latency services, the MBSR’s connectivity is switched to LEO, ensuring that NTN can deliver all types of services to the connected UEs.

5.5.4	Post-conditions

MBSR is able to provide seamless connectivity to the connected UEs in the remote areas with the help of multi-orbit satellite network (when terrestrial network is not available for MBSR).

5.5.5	Existing features partly or fully covering the use case functionality

TS 22.261 specifies the following requirements associated with the use case:

- A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency.

NOTE: 5 ms network latency is assumed and added to satellite one-way delay.

- A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 35 ms end-to-end latency.

NOTE: 5 ms network latency is assumed and added to satellite one-way delay. The 5G system shall support operation of downlink only broadcast/multicast over a specific geographic area (e.g., a cell sector, a cell or a group of cells).

- A 5G satellite access network connected to 5G core networks in multiple countries shall be able to meet the corresponding regulatory requirements from these countries (e.g., Lawful Interception).

- The 5G system shall be able to support mobile base station relays using 3GPP satellite NG-RAN (NR satellite access).

5.5.6	Potential New Requirements needed to support the use case

[PR 5.5.6-001] Subject to operator’s policy, the 5G network with multi-orbits satellite access shall be able to support mobile base station relays to access different orbit satellites considering e.g. availability of the coverage, required QoS.

* * * End of Changes * * * *