3GPP TSG-SA WG1 Meeting #107 	S1-242062

Maastricht, Netherlands, 19-23 August 2024

Title:	Network entry via GEO in multi orbit satellite network

Agenda Item:	7.3

Source:	SES, Novamint, Thales, TNO

Contact:	joel.grotz@SES.com, stavros.domouchtsidis@ses.com, abbas.karaki@SES.com

Abstract: This document proposes a use case on Multi Orbit with Satellite Access System and potential requirements for TR22.887 v0.1.0 (FS_5GSAT_ph4)

## x.1	Network entry via GEO in multi orbit satellite network

### x.1.1	Description

Satellite-based 5G networks may play a vital role in bridging connectivity gaps, , especially in remote area thanks to the following characteristics:

- Global Coverage: Satellites can provide coverage across vast geographical regions, including oceans, deserts, and polar regions. Unlike terrestrial networks, which have limitations due to infrastructure placement, satellites can reach virtually any point on Earth.

- Ubiquitous Connectivity: Satellite networks ensure continuous connectivity, even areas with sparse population. This makes them ideal for emergency communication, disaster response, and remote monitoring.

- Quick Deployment: Setting up ground-based infrastructure can be time-consuming and expensive. Satellites can be launched relatively quickly, allowing rapid deployment of communication services.

However, a Satellite Communication System comes with additional challenges compared to terrestrial infrastructures.

- LEO satellites operate at altitudes ranging from approximately 400 km to 1,500 km above Earth’s surface. Due to their proximity, they experience higher atmospheric drag, necessitating frequent orbital adjustments. These adjustments consume energy. MEO satellites orbit at altitudes between 7,000 km and 35,786 km. While they experience less drag than LEO satellites, they still require energy for maintaining their orbits and for communication with ground stations.

- To cover a large area, the NGSO networks need to provide an efficient solution for areas with very few users with an efficient usage of capacity in sparsely populated areas, such as Oceans and remote locations.

- Also, the payload of LEO and MEO satellites consume power for data processing, transmission, and reception. Balancing power allocation between payload operations and essential subsystems (such as attitude control and thermal management) is crucial.

- The Solar panels on satellites aims to convert sunlight into electrical energy. Satellites need onboard batteries to store excess energy generated by solar panels during sunlight hours. However, the size of solar panel (and batteries) is limited by the satellite’s surface area. LEO satellites have smaller solar panels due to their compact design. These panels generate less power compared to larger panels on geostationary satellites. MEO satellites can accommodate slightly larger panels, but they still face limitations. Furthermore, the exposure of the LEO/MEO satellites to sunlight varies significantly due to orbital dynamics.

In summary, managing power resources efficiently is a critical challenge for both LEO and MEO satellites. Engineers continually strive to optimize power usage, extend battery life, and ensure reliable operation in the demanding space environment.

Approximately 71% of Earth’s surface is covered by oceans. NGSO satellites can play a vital role in providing connectivity to maritime vessels, airplanes, offshore platforms, and remote islands. When there are no active users in these particular regions (such as over the ocean), NGSO satellites can be intelligently turned off or the power of specific beams be reduced. This energy-saving strategy minimizes unnecessary transmission and reception, extending satellite lifetime and optimizing power budgets. However, for non-active users in the middle of the ocean that need to access to the NGSO assets (while beams are turned off), the procedure can be problematic on relying on the same NGSO system to request resource allocation.

For such reason, an out-of-band control channel from an existing GEO link could be considered to request the NGSO asset to allocate resource and to power-on a dedicated beam over the region where the user is located. This makes it easier for the user since the user needs only to hold the TLE of 3 GEO satellites (covering the global earth) to start the procedure.

Figure 5.x.1-1 Requesting access to NGSO assets via GEO Narrowband Control Plane  (Multi Orbit Satellite Network)

### x.1.2	Pre-conditions

Captain Alan and his team work aboard a cargo ship. They work for a logistics company that bridges Europe and South Africa. During their journey, unexpected weather events cause an engine failure on the boat, temporarily cutting off communication with their Logistic Support Center in Marseille. They need to contact the nearest maintenance station to obtain replacement parts and inform the Marseille’s Logistic Support Center of a probable one-week delay in delivering the goods compared to the originally scheduled date.

Normally, their ship uses a connection with a GSO/NGSO satellite operator.

Located in the middle of the Atlantic Ocean, the team must establish a connection with the NGSO satellite constellation. Since the operator no longer has precise location information for the ship due to the loss of connection, the satellite operator decides to switch the NGSO satellite beams covering the Atlantic Ocean on standby mode (beams turned off).

However, the operator provides to the ship mounted user equipment a narrowband control channel via GSO.

The onboard VSAT terminal has a list of TLE coordinates for three GEO satellites and the access frequencies for the GEO constellation based on the geographical region.

### x.1.3	Service Flows

The VSAT terminal on board the ship, based on its current position, determines which GEO satellite it should establish a link with and on which frequency. 

The VSAT terminal connects to the satellite and sends its request (along with its current position) to access the MEO constellation with the required QoS characteristics for setting up a broadband service with low latency. 

The satellite operator’s resource allocation system, aware of this request and the VSAT’s position, determines which NGSO satellite and beam should be illuminated to serve the specific VSAT terminal.

### x.1.4	Post-conditions

The network operator has successfully determined which beam/satellite from the NGSO constellation needs to be illuminated.

The VSAT Terminal has successfully established a link access to the network via the NGSO Constellation.

The Team is now able to communicate again with the Marseille’s Logistic Support Center and the proximity Maintenance Station.

### x.1.5	Existing features partly or fully covering the use case functionality

< Highlight existing features in the existing set of normative specifications that partly or fully cover this use case.>

### x.1.6	Potential New Requirements needed to support the use case

[PR x.1.6-001] A 5G System supporting satellite access with multi orbit operation shall be able to provide a mechanism to allow any UE terminal to access services on NGSO when NGSO Beams are turned off for power efficiency purposes.

[PR x.1.6-002] A 5G System supporting multi orbit satellite Access network shall be able to determine which NGSO beams can be turned on and respectively turned off based on the geographical distribution of the traffic demand.

[PR x.1.6-003] A 5G System supporting multi orbit satellite Access shall be able to have access to a set of pre-configured parameters (TLE, frequency access depending on geolocation …) allowing UE to select the relevant GSO through which network entry procedure can be successfully achieved