3GPP TSG-SA WG1 Meeting #107 	S1-242063

Maastricht, Netherlands, 19-23 August 2024

Title:	Enhanced PWS in satellite access network

Agenda Item:	7.3

Source:	SES, Novamint, Thales, TNO

Contact:	joel.grotz@SES.com, abbas.karaki@SES.com

Abstract: This document proposes a use case on Enhanced Support for Emergency Communications with Satellite Access System and potential requirements for TR22.887 v0.1.0 (FS_5GSAT_ph4)

## x.1	Enhanced PWS in satellite access network

### x.1.1	Description

Satellite-based 5G serves as a crucial backup communication option during natural disasters and emergencies when terrestrial networks may be damaged or overwhelmed. This ensures that essential communication and services remain available when they are needed most.

To alert people about any emergency, the message alert shall be sent in real-time within seconds and with a high degree of reliability and the message shall reach a high percentage of people in the targeted area, not just residents but roaming visitors using their native language.

As of today, some countries are still using Location-Based SMS to alert the population of an emergency. This kind of system deliver data in a unicast channel which brings some disadvantages as:

- Longer delivery times of warning messages (several hours) to groups of people in the geo-target area.

- Limitation in terms of scalability since the resource allocation for SMS is not well optimized to address a huge number of people.

Satellite-based 5G excels at broadcasting data and content over wide geographic areas, making it valuable for delivering consistent information to large audiences. This capability is beneficial for applications like emergency alerts where it enables high scalability and real-time for alert delivery, allowing to reach people in isolated area outside the coverage of the Terrestrial Network.

Figure 5.x.1-1 Extending the Broadcast area outside of the originating country for Emergency Communications using Satellite Access

### x.1.2	Pre-conditions

Alice is a cross-border worker, living on the French border and taking the A3 freeway to her workplace in Luxembourg every morning. Half of Luxembourg workers are in the similar situation as Alice (cross-border workers).

The terrestrial network on the border is very weak and almost unavailable, so satellite access is the alternative. When Alice is home, she sets in her phone as default network access the satellite access.

Alice has a business phone with an operator in Luxembourg. After her subscription, (following the EU rules, article 294 [X1]) Alice received automatically an SMS to inform her about the existence of a Public Warning System and easily understandable information on how to receive public warnings.

Following the received instructions, Alice enters in the settings of her professional phone and subscribes to the “EU-Alert” for “Extreme Threats” communication channels and “Severe Threats” communication channels.

It is December, the weather conditions are severe with a very high risk of black ice on the roads, including the A3 freeway.

As a precautionary measure, and in order to protect the lives of the population, the emergency services of the Luxembourg government are sending out a warning “Severe Threats” very late in the evening that people are strongly advised to stay at home and off the roads due to a very high risk of icy conditions.

### x.1.3	Service Flows

Since this can concern cross-border workers, Luxembourg authorities in accordance with neighbour countries authorities has defined the broadcast area of this “Severe Threats” to some extent outside of the Luxembourg geographical frontier.

Alice receives the “Severe Threats” alert via the satellite access network, while she is in the France frontier.

### x.1.4	Post-conditions

The network operator has successfully mapped the broadcast area for the alert in some region outside of the country to reach cross-border worker.

The network operator has made an agreement with a satellite network operator to expand the broadcast area for the alert outside of the country.

The alert message was delivered reliably to all users subscribed to the “Severe Threats” communication channel through the terrestrial access and the satellite access.

### x.1.5	Existing features partly or fully covering the use case functionality

TS 22.268 [X2] which is providing the service requirements for PWS does not include dedicated requirements in case of satellite access

### x.1.6	Potential New Requirements needed to support the use case

[PR x.1.6-001] Subject to regulatory requirements and operator’s policy, the 5G System supporting both satellite access and terrestrial access shall be able to setup a Public Warning System and to inform automatically the users how to receive alert from the PWS.

[PR x.1.6-002] Subject to regulatory requirements and operator’s policy, the 5G System supporting both satellite access and terrestrial access, shall be able to map the broadcast area of a specific Alert from PWS to a geographical area that can go to some extent outside of the originating country.

[PR x.1.6-003] Subject to regulatory requirements and operator’s policy, the 5G System supporting satellite access and PWS shall be able to deliver with a reliability of 99,99% alert messages to handsets UE (without acknowledgements from the UE).

Note: reliability shall address cases where user is not able to receive the alert message due to multiple reasons (alerting end-users entering the relevant geographic area after the initial warning, or no good coverage from terrestrial access and satellite access), this could be achieved by several different mechanisms e.g. by constantly broadcasting warning messages (ensuring that these aren’t displayed again after they have been received) or by using repetition mechanisms (frequency and time domain interleaving) for coverage enhancement purposes

[PR x.1.6-004] Subject to regulatory requirements and operator’s policy, the 5G System supporting satellite access and PWS shall be able to target only a dedicated geographic area.

* * * Next changes * * * *

### 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: “Service requirements for the 5G system”

[3]	3GPP TS 22.228: “Service requirements for the Internet Protocol (IP) Multimedia core network Subsystem“.

[4]	ITU-T E.800: "Definitions of terms related to quality of service ".

[5]	ITU-T G.114: "One-way transmission time".

[6]	ITU-T G.107: "The E-model: a computational model for use in transmission planning "

[7]	X. Huang, W. Qi, X. Xia, Y. Sun, Z. Sun and M. Peng, "IoT NTN for Voice Services: Architectures, Protocols, and Challenges," in IEEE Network, 2024.

[8]	“Satellite firms forge unlikely alliances to create seamless multi-orbit networks”, https://spacenews.com/satellite-firms-forge-unlikely-alliances-to-create-seamless-multi-orbit-networks/, March 2024

[9]	“Autonomous shipping”, https://www.imo.org/en/MediaCentre/HotTopics/Pages/Autonomous-shipping.aspx

[10]	Dhinesh Kumar R, Rammohan A, Revolutionizing Intelligent Transportation Systems with Cellular Vehicle-to-Everything (C-V2X) technology: Current trends, use cases, emerging technologies, standardization bodies, industry analytics and future directions. Vehicular Communications, Vol.43, 2023, 100638

[11]	5G Automotive Association (5GAA), C-v2x use cases and service level requirements -volume i，5GAA website, https://5gaa.org/c-v2x-use-cases-and-service-level-requirements-volume-i/, 2019

[X1] 	"Directive (EU) 2018/1972 establishing the European Electronic Communications Code". European Union. Retrieved 11 December 2018: https://eur-lex.europa.eu/legal-content/EN/TXT/HTML/?uri=CELEX:32018L1972&from=EN

[X2] 	3GPP TS 22.268: Public Warning System (PWS) requirements