3GPP TSG- SA1 Meeting # 107	S1-242083

Maastricht, The Netherlands, 19-23 August 2024

# Source:	III, AT&T, GE Network Technologies, ISSDU

# Title:	Proposal for 6G SID: Quantum-Safe Network (QSN)

# Document for:	Approval

# Agenda Item:	8.1

# Contact:		Yi-Hsueh Tsai	lucas@iii.org.tw
	Stephen F Bush	stephen.bush@ge.com
	Feng-Ming Yang	kelvinfomi@issdu.com.tw

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Proposal for 6G SID: Quantum-Safe Network (QSN)

Acronym: FS_6G

Unique identifier:

[Leave blank]

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | N/A |

Dependency on non-3GPP (draft) specification:

3	Justification

As we move towards developing 6G technology, it's crucial that its design inherently protects against quantum computing threats. Quantum computing poses new risks that could compromise our current security measures. Therefore, making the 6G infrastructure quantum-safe from the start is essential to protect against these advanced threats. To achieve this, we need to define a set of specific requirements that address the main challenges in securing 6G networks. These challenges include developing new security methods that can resist quantum attacks, ensuring that the network can adapt to new cryptographic technologies quickly to counteract evolving threats, and establishing secure ways to manage identities within the network. Each area requires detailed study and careful planning to ensure that future 6G networks are both safe and reliable.

As specified in clause 6.12.2 of 3GPP TS 33.501, the SUCI is generated using a protection scheme such as Elliptic Curve Integrated Encryption Scheme (ECIES) with the Home Network public key. Regarding the symmetric components of the ECIES protocol [1], we note that migration should be rather easy as MAC are already generated using 256-bit keys (which are deemed sufficient to withstand quantum computing) and as AES inherently supports 256-bit keys. Moving from AES-128 to AES-256 would then be the main change in this part of the specifications, along with the necessary adaptations of the key derivation function. The main vulnerability of the ECIES protocol with respect to the quantum threat is actually the Diffie-Hellman key exchange step, regardless of the used profile. Although there is no drop-in Post Quantum replacement for this protocol, it is well-known that a Key Encapsulation Mechanism can achieve the same goal, namely share a common secret. The main remaining question is thus the one of the performances as moving to Post Quantum cryptography (PQC) will increase the ciphertext size and dramatically change the nature of the computations. As the current version of the specifications allows the operator to decide whether the SUCI computation should be performed within the USIM or within the Mobile Equipment, there is no unique answer to this question. Arguably, the case where the USIM performs this computation is the most challenging one given the constrained nature of the device.

When considering the move from AES-128 to AES-256, the level of entropy that is used to generate the keys for symmetric key encryption maybe overlooked as part of the transition to Post-Quantum Cryptography (PQC). Current SA3 documents did not mention that if entropy is poor, then even a 256 bits’ key can be easily compromised by Quantum computer. ATIS has conducted a study into this to understand the “Implications of Entropy on Symmetric Key Encryption Resilience to Quantum” [3]. If the level of security is to also be quantifiably secure, then entropy used within the communications needs to be quantifiable and/or have minimum requirements.

For industrial control system use, the ultra-low latency service has extreme requirements on user-plane data transmitting delay [4]. Without security, data transmitting is very quick using nowadays technology. Introducing of security in user-plane data transmitting will delay the data transmitting, but it is a trade-off between performance and security. The in user-plane data path between UE and UPF includes radio interface and N3 interface, and may include F1 interface and Xn interface. In current specification, IPsec protocol may be used to protect above interfaces. It has been identified that IPsec and TLS/DTLS will slow down the data forwarding performance significantly, which means user-plane data transmitting over above interfaces will be delayed significantly if IPsec is deployed. It has been identified that security performance on radio interface also is able to be improved, e.g. by using AES GCM mode. Besides integrating PQC into 5G Time-Sensitive Networking (TSN) [5][6] is essential for future-proofing the network against quantum computing threats. However, this integration can introduce performance overheads in several areas, primarily due to the inherent characteristics of PQC algorithms compared to their classical counterparts. Here's a detailed explanation of where performance overhead might increase:

- Increased Computational Complexity:

Encryption and Decryption Processes: PQC algorithms, especially those designed for key exchange and digital signatures, tend to have higher computational complexity. This means that devices and network infrastructure may require more processing power to perform cryptographic operations, potentially increasing the time it takes to encrypt and decrypt data.

Key Generation: The process of generating cryptographic keys in PQC can also be more computationally intensive, which might impact the setup time for secure communications, especially in scenarios where keys are frequently refreshed.

- Larger Key Sizes:

Bandwidth Consumption: PQC algorithms generally use larger key sizes to achieve the same level of security as classical algorithms against quantum attacks. Transmitting these larger keys over the network can consume more bandwidth, which is particularly concerning for 5G TSN applications that require efficient use of network resources.

Storage Requirements: Increased key sizes mean that more storage space will be needed on devices and within network infrastructure to store these keys. This could be challenging for devices with limited storage capacity.

- Increased Latency:

Handshake and Authentication Delays: The initial handshake and authentication processes, critical for establishing secure connections, may take longer due to the increased computational complexity and larger key sizes associated with PQC. This could introduce additional latency, affecting applications that rely on ultra-reliable low-latency communications (URLLC), a key feature of 5G TSN.

Data Packet Processing: The overhead of encrypting and decrypting larger data packets, due to the increased size of cryptographic primitives, could introduce additional latency in data processing and transmission.

- Impact on Battery Life:

Energy Consumption: The higher computational requirements for performing PQC operations can lead to increased energy consumption, impacting the battery life of mobile devices. This is particularly relevant for Internet of Things (IoT) devices and sensors that are part of 5G TSN ecosystems and often operate on battery power.

- Challenges in Real-Time Applications:

Determinism and Jitter: The variability in processing times for PQC operations could introduce jitter and affect the determinism of network communications, which is critical for many TSN applications. Ensuring consistent and predictable network performance becomes more challenging with the introduction of PQC.

- Time synchronization is crucial and vulnerability for TSN.

Maintaining “secure” nanosecond precision time sync (mobile network sync + TSN sync) without ruining its performance due to security overhead will also be a fun challenge. A favorite attack on 5G TSN is slowly moving time sync out of phase and throwing off the deterministic cycle timing.

Therefore, we are concerned that the increasing complexity of implementing Post Quantum Cryptography (PQC), as this may be too slow and unacceptable for industrial control system application. Introducing Quantum Key Distribution (QKD) [7] technology could reduce overhead, shortens latency, and provides warnings of eavesdropper attacks.

This study is aiming at study potential requirements for enabling Quantum-Safe Network (SQN).

- 3GPP TS 33.501-i60, “Security architecture and procedures for 5G System”

- GSMA, “PQ.03-Post-Quantum-Cryptography-Guidelines-for-Telecom-Use-v1.0”

- ATIS, “Implications of Entropy on Symmetric Key Encryption Resilience to Quantum” (https://atis.org/resources/implications-of-entropy-on-symmetric-key-encryption-resilience-to-quantum/)

- 3GPP TR 33.825-g01, “Study on the security of Ultra-Reliable Low-Latency Communication (URLLC) for the 5G System (5GS)”

- 3GPP TS 22.261-j50, “Service requirements for the 5G system”

- 3GPP TS 23.501-j00, “System architecture for the 5G System (5GS);”

- Bush, Stephen F., “Time-Sensitive Quantum Key Distribution (TSQKD)” (https://www.osti.gov/biblio/1870109)

4	Objective

This study is aiming at study potential requirements for enabling Quantum-Safe Network (SQN) including

- Post-Quantum Cryptography (PQC) for both asymmetric and symmetric key encryptions to protect user’s identity and 6G system communication, respectively.

- Quantum Key Distribution (QKD) to provide secure key management for ultra-low-latency small-data-rate communication for critical applications like smart factory.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
|  | TR 22.8xx | Study on 6G Use cases and requirements | TSG#110 | TSG#111 | TBD |

6	Work item Rapporteur(s)

TBD

7	Work item leadership

SA WG1

8	Aspects that involve other WGs

9	Supporting Individual Members

To be handled separately directly at SA1#107

Supporting IM name

AT&T

GE Network Technologies

III

ISSDU