3GPP TSG-SA WG1 Meeting #107 	S1-242100

Maastricht, The Netherlands, 19-23 August 2024	(revision of S1-24xxxx)

Title:	6G Migration and Stage 1

Agenda Item:	9 (Other technical contributions)

Source:	Samsung

Document for:	Discussion, Endorsement

Contact:	erik.guttman@samsung.com

Abstract: This discussion paper considers the overall state of stage 1 specifications and recommends actions we can take now, in structuring the 6G study and TR, and eventually, when Rel-21 specifications are created for 6G. Early discussion of these topics may lead to the ability to provide input to RAN and SA in time for the 3GPP Workshop in March 2025.

## Introduction

One of the key areas for discussion in any new generation is backwards compatibility. This has the potential to scope the studies of RAN and Network architecture. Though SA1 will not discuss architecture evolution and targets in technical terms, it is entirely appropriate that SA1 discuss 6G in terms of services and functionality supported, as well as compatibility with existing standards. This is captured in the SA1 ToR:

[...]In addition, SA1 considers the requirements for different services and their interworking as the system continues to evolve. Interworking aspects include both the interworking of services offered by 3GPP standards and the interworking with networks and standards external to 3GPP. SA WG1 also considers service operation, charging and accounting aspects.
[...]

•	Identification of requirements for service interworking.

•	Identification of requirements for service interoperability between networks.

This discussion paper suggests three ways in which critical review of legacy specifications under SA1's control can be taken into account in order to frame the 6G study. Firm decisions on backward compatibility will contribute to reduce the complexity of the 6G system.

## Discussion

1. Starting point assumptions for 6G study and normative specification

It is too early to decide but SA1 must determine how to structure the Rel-21 TS. Does it refer to 22.278, 22.261 and merely enhance from there? Is it worth considering giving up on 22.278 as the basis for mobile broadband service and revisit the text there?

Some provisions in 22.278 are clearly outdated, e.g.

-	CS (27 uses), interworking with other systems (IMT-2000 3GPP2 access, WiMAX, more)

-	Support of UTRAN (throughout)

-	Multi-access and seamless mobility (out of date after advances in 5G)

Other sections are in need of review given advances in 5G and some features never being carried forward in stage 2 and stage 3 (e.g. for ProSe - some Range Class functionality.)

The implication for 6G study is this:

-	Redefining mobile broadband service in a clearly access independent manner, with constrained interworking.

-	Refinement of use of multiple access and service continuity to be independent from 4G terminology and concepts.

2. Many stage 1 specifications are carried forward that should be stopped

There is work that can be done immediately to clean up the status of legacy stage 1 specification. If we can agree to this in principle now, this can be communicated to RAN and SA to help them prepare for the 6G workshop. Though these specifications are often obscure and seldom referenced, removing them from the breadth of 6G requirements now and clearly indicating what remains will help with scoping the objectives of 6G studies in RAN and SA.

SA1 should review whether to continue 'generation-independent' specifications at all, and if so, how to remove legacy parts, or clearly indicate that they apply to older systems (e.g. they fulfill requirements for 22.278 or for GPRS, and clearly state which provisions are not relevant for the 6G system.) Good example we can draw inspiration from: TS 22.280 scope concludes with the words "The requirements in this specification do not apply to GSM or UMTS."

2a.	The following specifications have neither been maintained in decades nor even advanced to have a 5G logo.

| 22.001 | Principles of circuit telecommunication services supported by a Public Land Mobile Network (PLMN) |
| --- | --- |
| 22.002 | Circuit Bearer Services (BS) supported by a Public Land Mobile Network (PLMN) |
| 22.003 | Circuit Teleservices supported by a Public Land Mobile Network (PLMN) |
| 22.022 | Personalisation of Mobile Equipment (ME); Mobile functionality specification |
| 22.024 | Description of Charge Advice Information (CAI) |
| 22.034 | High Speed Circuit Switched Data (HSCSD); Stage 1 |
| 22.057 | Mobile Execution Environment (MExE); Service description; Stage 1 |
| 22.060 | General Packet Radio Service (GPRS); Service description; Stage 1 |
| 22.066 | Support of Mobile Number Portability (MNP); Service description; Stage 1 |
| 22.067 | enhanced Multi Level Precedence and Pre-emption service (eMLPP); Stage 1 |
| 22.072 | Call Deflection (CD) service description; Stage 1 |
| 22.076 | Noise suppression for the AMR codec; Service description; Stage 1 |
| 22.078 | Customised Applications for Mobile network Enhanced Logic (CAMEL); Service description; Stage 1 |
| 22.079 | Support of Optimal Routeing (SOR); Service definition; Stage 1 |
| 22.091 | Explicit Call Transfer (ECT) supplementary service; Stage 1 |
| 22.093 | Completion of Calls to Busy Subscriber (CCBS); Service description, Stage 1 |
| 22.094 | Follow Me service description; Stage 1 |
| 22.096 | Name identification supplementary services; Stage 1 |
| 22.097 | Multiple Subscriber Profile (MSP) Phase 2; Service description; Stage 1 |
| 22.129 | Service aspects; Handover requirements between UTRAN and GERAN or other radio systems |
| 22.135 | Service aspects; Handover requirements between UTRAN and GERAN or other radio systems |
| 22.240 | Service requirements for 3GPP Generic User Profile (GUP); Stage 1 |
| 22.242 | Digital Rights Management (DRM); Stage 1 |
| 22.243 | Speech recognition framework for automated voice services; Stage 1 |
| 22.246 | Multimedia Broadcast/Multicast Service (MBMS) user services; Stage 1 |
| 22.259 | Service requirements for Personal Network Management (PNM); Stage 1 |
| 22.279 | Combined Circuit Switched (CS) and IP Multimedia Subsystem (IMS) sessions; Stage 1 |
| 22.346 | Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1 |
| 22.519 | Business communication requirements |

These should be reviewed and as appropriate not advanced to Rel-21.

2b.	The following specifications have been advanced to 5G but appear to be of historic importance only.

Legacy services

| 22.140 | Multicall; Service description; Stage 1 |
| --- | --- |
| 22.141 | Multimedia Messaging Service (MMS); Stage 1 |
| 22.142 | Presence service; Stage 1 |
| 22.146 | Value Added Services (VAS) for Short Message Service (SMS) requirements |
| 22.174 | Push Service; Service aspects; Stage 1 |
| 22.233 | Transparent end-to-end packet-switched streaming service; Stage 1 |
| 22.340 | IP Multimedia Subsystem (IMS) messaging; Stage 1 |

These should be reviewed and as appropriate not advanced to Rel-21.

2c.	The following 'live specifications' and 'seems like essential legacy specifications' that are used to specify 'cross-generation' requirements that are a mess

These specifications contain many provisions that do not apply to 5G let alone 6G. Perhaps it is time to consider not supporting 'all generations' from some of our central specificiations (22.011, 22.101, etc.) 2G, 3G, 4G are all still supported in existing frozen versions of these specifications.

Mention of deprecated specs and references to them were supposed to be cleaned up in Rel-15!

A few comments were added below by the author of this tdoc.

These should be reviewed in detail and cleaned up if we decide to break with 'generation-independent' specifications - which this paper recommends, since 2G and 3G specification is closed. A change to any text that impacts 2G or 3G is not permitted by 21.900.

| TS # | Specification Title | Legacy Issues (partial list) - text shown appears in spec that could be reviewed... |
| --- | --- | --- |
| 22.011 | Service accessibility | GPRS services not allowed in this PLMN In E-UTRAN it shall be possible to support ... SSAC CSFB from CDMA2000 Does EAB still exist in 5G and beyond access control? ACDC applies to UTRAN PS Domain and E-UTRAN SoLSA |
| 22.016 | International Mobile station Equipment Identities (IMEI) | "IMSI detach" |
| 22.030 | Man-Machine Interface (MMI) of the User Equipment (UE) | standard GSM SS procedure support by the MS Registration of new password supplementary service control via NAS is assumed Follow Me e-marking presentation annex B validate (with CT1?) Table B.1 codes are still relevant annex C bearer services |
| 22.038 | (U)SIM Application Toolkit (USAT); Service description; Stage 1 | GSM is presented as the 'scope of this TS', though the requirements are also applicable to UMTS... mention of I-WLAN (deprecated!) PDP context parameters Bearer Independent Protocol SMS via GPRS etc etc |
| 22.041 | Operator Determined Barring (ODB) | the served subscriber in the serving MSC/VLR. circuit oriented services I-WLAN (deprecated!) |
| 22.042 | Network Identity and TimeZone (NITZ); Service description; Stage 1 | "MS" we no longer use this term, instead User Equipment |
| 22.071 | Location Services (LCS); Service description; Stage 1 | the standard shall support NG-RAN, E-UTRAN, GERAN and UTRAN CAMEL based Location Services |
| 22.090 | Unstructured Supplementary Service Data (USSD); Stage 1 | MSC, VLR, HLR, MExE servers, CSEs and proxy servers. ...it shall forward a message to the HLR in general USSD should be reviewed... |
| 22.101 | Service aspects; Service principles | I-WLAN mention (deprecated!) It shall be capable of delivering audio, text, video and graphics direct to people... we have higher aims now 3GPP specifications include UTRA(N) radio interface SIPTO this is a 4G feature 3GPP specifications aim to be compatible with IMT-2000 standards Evolved Packet System UDC 3GPP specifications shall be capble of supporting 2G services... " Information quality a characterises the bit integrity and delay requirements of the applications. Other parameters may be needed." supported bit rates, Range of QoS requirements, end-user performance expecations old entries follow! specifications handover between the GERAN and UTRAN IP-CAN old terminology the user experience shall be the same as across UTRAN and GERAN access etc |
| 22.104 | Service requirements for cyber-physical control applications in vertical domains | no issues identified |
| 22.105 | Services and service capabilities | bearer services supplementary service this is now considered an aspect of IMS in the architecture - definition is stale home environment (HE) terminal adaptation function (TAF) SAT [5], MExE [6], IN and CAMEL [8] |
| 22.115 | Service aspects; Charging and billing | Home Environment I-WLAN (deprecated!) text specific to E-UTRA(N) may not be applicable to 6G The user’s user equipment that is incurring the charge shall provide the following information to the serving network review if complete and correct IN or CAMEL services VAS information GSM Optimal Routing SCUDIF changin a voice call to a CS multimedia call CSG cell IOT small data (S)Gi-LAN Automatic Roaming Agreements, Release 99 |
| 22.119 | Maritime communication services over 3GPP system | EPS and 5GS specified in 3GPP may modify for 6G |
| 22.125 | Unmanned Aerial System (UAS) support in 3GPP | no issues identified |
| 22.137 | Integrated Sensing and Communication | no issues identified |
| 22.153 | Multimedia priority service | CS Fallback NSWO the terminology has changed in 5G? CS Domain bearer service EPC / EPS text applicable to 4G, 5G, but 6G? |
| 22.156 | Mobile Metaverse Services | no issues identified |
| 22.173 | IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1 | mobile CS networks Data (CS) Interworking with CS domain |
| 22.182 | Customized Alerting Tones (CAT) requirements; Stage 1 | reference to GUP is not used CAT functionality in the CS domain Interworking betwene CS and IMS based CAT services Roaming to 2G or 3G networks |
| 22.183 | Customized Ringing Signal (CRS) requirements; Stage 1 | reference to GUP is not used CS domain functionality Interworking betwene CS and IMS based CAT services Roaming to 2G or 3G networks |
| 22.185 | Service requirements for V2X services | Requirements on "The E-UTRA(N)" are obsolete? |
| 22.186 | Service requirements for enhanced V2X scenarios | Requirements on "The E-UTRA(N)" are obsolete? |
| 22.220 | Service requirements for Home Node B (HNB) and Home eNode B (HeNB) | support for HNB (defined in the 3G / UTRAN architecture) and HeNB (defined in 4G / E-UTRAN architecture) - is this relevant to 6G? If so what needs to be changed? Is CSG as specified relevant to 6G? CS-to-IMS interworking as specified in IMS Centralized services [see above, TS 22.101 is a venerable spec] SIPTO is a 4G service Annex A (informative) use cases describes mainly legacy scenarios |
| 22.226 | Global Text Telephony (GTT); Stage 1 | Circuit switched multimedia Circuit switched voice telephony CS Multimedia Virtual Home Environment toolkit CS networks for interworking with CS Multimedia terminals CS multimedia with text according to ITU-T T.140 [4]. Annex A informative - review text for current state, e.g. of Total Conversation specs and requirements. |
| 22.228 | Service requirements for the Internet Protocol (IP) multimedia core network subsystem (IMS); Stage 1 | OMA Parlay (OSA), CAMEL, MExE, SAT, GUP (e.g. E-UTRAN, UTRAN, GERAN, fixed lines, LAN, DOCSIS®, WiMAX™ and cdma2000® access). reference to TS 22.173 with CS aspects TISPAN interworking a distinction to be made between voice over E-UTRAN and non-voice over E-UTRAN traffic EPC access Note: an example for applicability of multicast services could be the IMS based "Push to talk over Cellular" service, that is being standardised by OMA. IP-CAN terminology Interworking with PLMN CS Domain etc. |
| 22.261 | Service requirements for the 5G system | no issues identified |
| 22.262 | Message service within the 5G System (5GS); Stage 1 | no issues identified |
| 22.263 | Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA) | no issues identified |
| 22.268 | Public Warning System (PWS) requirements | eDRX (4G feature), other terms needed? NodeBs, RNCs, from LTE onwards |
| 22.278 | Service requirements for the Evolved Packet System (EPS) | Please see discussion point (1) above. |
| 22.280 | Mission Critical Services Common Requirements (MCCoRe); Stage 1 | minor alignment needed for references (GSCS LTE has been renamed, for example.) |
| 22.281 | Mission Critical (MC) video | no issues identified |
| 22.282 | Mission Critical (MC) data | [R-7.1-001] Interworking between the MCData SDS and TETRA Short Data Service [5] shall be supported. Editor's Note: interworking with other non-3GPP systems is FFS. It is time to remove this EN. |
| 22.289 | Mobile communication system for railways | no issues identified |
| 22.368 | Service requirements for Machine-Type Communications (MTC); Stage 1 | has this been superceded by 22.261? EAB (4G specific) 3G example (SGSN-GGSN) |
| 22.369 | Service requirements for Ambient power-enabled IoT | no issues identified |
| 22.468 | Group Communication System Enablers (GCSE) | no issues identified |

3. Supported scenarios, when considered at the service layer, could be discussed in SA1

One of the most common concerns about the 5G system is that it is very complex and Release 15 was phased with different options. The different deployment options were discussed in RAN and SA and the decisions had wide ranging consequences. From a service perspective, it is possible to discuss and even specify:

-	Backwards compatibility, including service continuity

-	Support (or lack of support) for specific access technologies

-	Essential features vs. Optional features

-	Service compatibility roaming considerations

Though in the end RAN and SA will determine the scope and objectives of 6G, it would be advantageous if SA1 could provide advice and insights into how this can be done. The technical questions of architecture of the core and radio network are out of scope for SA1, however this leaves significant opportunities for discussion of migration and backwards compatibility to be discussed at the service level.

## Conclusion

Recommendation for endorsement:

1)	SA1 agrees to remove 2G and 3G aspects (at least) from stage 1 specifications that will bear the 6G logo.

2)	SA1 agrees to remove 'deprecated' text and references (to I-WLAN).

3)	SA1 agrees to review whether to either:

- 	discontinue use of 'generation independent TSs' and instead include only the provisions that apply to the current supported generation(s); or

-	clearly distinguish in text of specifications that are 'generation independent' those provisions that are not relevant to the 6G system.

4)	SA1 agrees to identify some specifications (to be determined) to not advance to Rel-21 as these will not be part of 6G and, further, are no longer being actively enhanced or maintained.

NOTE:	The process by which (3) will be undertaken will require further discussion, since the action can only be taken by the Specification Manager. Sometimes discontinuation of specifications is requested by a working group secretary, other times this is decided by TSG.

5) the text below is provided for the 6G SID, TR

Proposed action for SID - add the justification and objectives to other text. [This is not a 'complete justification' nor 'complete set of objectives' - rather this is intended to supplement text supplied by others.]

## Justification A critical review of legacy specifications under SA1's control can be taken into account in order to frame the 6G study. Firm decisions on backward compatibility will contribute to reduce the complexity of the 6G system. It could be useful to define mobile broadband data service freshly in 6G, to decouple it with 3G and 4G terminology and concepts. There are a number of specifications that include provisions which are no longer supported or maintained, e.g. as they refer to 2G and 3G functionality. A careful review of references and specifications carried forward into 6G will reduce the complexity of the overall stage 1 specification. Reduction of complexity of 6G compared to prior releases will benefit all other efforts, and consideration of service interworking is a core responsibility of SA1. For this reason, identification of backwards compatibility support and required features for 6G will benefit all further work in 3GPP on 6G. ## Objective - Consider whether existing specification of mobile broadband service [22.278] is sufficient, or if there are gaps or changes to make for specification of this essential service. - Identify if current specifications are suitable for reference for 6G, whether specifications are historical and should not be advanced to release 21, or whether changes are needed to align to 6G. - Study backwards compatibility and migration scenarios to identify potential requirements for 6G and interworking with previous generation specifications. - Study 6G service requirements to identify which are essential and which are optional, in order to clarify the requirements and achieve coherent and complete specification of features. Conclusions can be added incrementally at any time during the study when consensus can be achieved.

Proposed action for TR skeleton [This is not a complete skeleton - it offers additional information to add to the skeleton.]

## X.Y General Section Editor's Note: Agreements concerning migration, backwards compatibility, supported scenarios and essential features can be added in sub-clauses of this clause. # Z Conclusions Editor's Note: Conclusions can be added incrementally at any time during the study when consensus can be acheived.

## Annex - Specification Analysis

| S1 specifications |  |  |  |  |
| --- | --- | --- | --- | --- |
| Possibly historic specifications |  | Last change | Action? | primary group |
| 22.001 | Principles of circuit telecommunication services supported by a Public Land Mobile Network (PLMN) | Rel-6 | 2G - 3G only -> discontinue? | S1 |
| 22.002 | Circuit Bearer Services (BS) supported by a Public Land Mobile Network (PLMN) | Rel-4 | 2G - 3G only -> discontinue? | S1 |
| 22.003 | Circuit Teleservices supported by a Public Land Mobile Network (PLMN) | Rel-5 | 2G, 3G, 4G only -> discontinue? | S1 |
| 22.022 | Personalisation of Mobile Equipment (ME); Mobile functionality specification | Rel-6 | Rel-7,8,9 corrections | S1 |
| 22.024 | Description of Charge Advice Information (CAI) | Rel-6 | 2G, 3G, 4G only -> discontinue? | S1 |
| 22.034 | High Speed Circuit Switched Data (HSCSD); Stage 1 | Rel-4 | 2G - 3G only -> discontinue? | S1 |
| 22.057 | Mobile Execution Environment (MExE); Service description; Stage 1 | Rel-5 | 2G, 3G, 4G only -> discontinue? | S1 |
| 22.060 | General Packet Radio Service (GPRS); Service description; Stage 1 | Rel-5 | 2G - 3G only -> discontinue? | S1 |
| 22.066 | Support of Mobile Number Portability (MNP); Service description; Stage 1 | Rel-6 | 2G, 3G, 4G only -> discontinue? | S1 |
| 22.067 | enhanced Multi Level Precedence and Pre-emption service (eMLPP); Stage 1 | Rel-11 | 2G - 3G only -> discontinue? | S1 |
| 22.072 | Call Deflection (CD) service description; Stage 1 | Rel-99 | 2G - 3G only -> discontinue? | S1 |
| 22.076 | Noise suppression for the AMR codec; Service description; Stage 1 | NONE | 2G, 3G, 4G only -> discontinue? | S1 |
| 22.078 | Customised Applications for Mobile network Enhanced Logic (CAMEL); Service description; Stage 1 | Rel-10 | 2G - 3G only -> discontinue? | S1 |
| 22.079 | Support of Optimal Routeing (SOR); Service definition; Stage 1 | Rel-99 | 2G - 3G only -> discontinue? | S1 |
| 22.091 | Explicit Call Transfer (ECT) supplementary service; Stage 1 | Rel-99 | 2G - 3G only -> discontinue? | S1 |
| 22.093 | Completion of Calls to Busy Subscriber (CCBS); Service description, Stage 1 | Rel-99 | 2G - 3G only -> discontinue? | S1 |
| 22.094 | Follow Me service description; Stage 1 | Rel-5 | 2G - 3G only -> discontinue? | S1 |
| 22.096 | Name identification supplementary services; Stage 1 | Rel-99 | 2G - 3G only -> discontinue? | S1 |
| 22.097 | Multiple Subscriber Profile (MSP) Phase 2; Service description; Stage 1 | Rel-4 | 2G, 3G, 4G only -> discontinue? | S1 |
| 22.129 | Service aspects; Handover requirements between UTRAN and GERAN or other radio systems | Rel-4 | 2G, 3G, 4G -> discontinue? | S1 |
| 22.135 | Service aspects; Handover requirements between UTRAN and GERAN or other radio systems | Rel-4 | 3G, 4G only -> discontinue? | S1 |
| 22.240 | Service requirements for 3GPP Generic User Profile (GUP); Stage 1 | Rel-6 | minor correction r9,11,16: 2G, 3G, 4G only -> discontinue? | S1 |
| 22.242 | Digital Rights Management (DRM); Stage 1 | Rel-6 | ref to OMA: 3G, 4G only - discontinue? | S1 |
| 22.243 | Speech recognition framework for automated voice services; Stage 1 | Rel-6 | 2G, 3G, 4G only -> discontinue? | S1 |
| 22.246 | Multimedia Broadcast/Multicast Service (MBMS) user services; Stage 1 | Rel-10 | 2G, 3G, 4G only -> discontinue? | S1 |
| 22.259 | Service requirements for Personal Network Management (PNM); Stage 1 | Rel-9 | Rel-16 inclusive language: 2G, 3G, 4G -> discontinue? | S1 |
| 22.279 | Combined Circuit Switched (CS) and IP Multimedia Subsystem (IMS) sessions; Stage 1 | Rel-7 | IMS, 3G, 4G -> discontinue? | S1 |
| 22.346 | Isolated Evolved Universal Terrestrial Radio Access Network (E-UTRAN) operation for public safety; Stage 1 | NONE | 3G, 4G only -> discontinue? | S1 |
| 22.519 | Business communication requirements | NONE | 3G, 4G only -> discontinue? | S1 |
| Legacy services - are these still supported? |  |  |  |  |
| 22.140 | Multicall; Service description; Stage 1 | Rel-6 | IMS, 2G, 3G, 4G -> continue? | S1 |
| 22.141 | Multimedia Messaging Service (MMS); Stage 1 | Rel-7 | IMS, 2G, 3G, 4G -> continue? | S1 |
| 22.142 | Presence service; Stage 1 | Rel-9 | 2G - 3G only -> discontinue? | S1 |
| 22.146 | Value Added Services (VAS) for Short Message Service (SMS) requirements | Rel-11 | 2G, 3G, 4G only -> discontinue? | S1 |
| 22.174 | Push Service; Service aspects; Stage 1 | Rel-6 | IMS, 2G, 3G, 4G -> continue? | S1 |
| 22.233 | Transparent end-to-end packet-switched streaming service; Stage 1 | Rel-10 | 2G, 3G, 4G only -> discontinue? | S1 |
| Legacy but still relevant specifications (MMTEL, IMS related) |  |  |  |  |
| 22.004 | General on supplementary services | Rel-10 | 2G, 3G, 4G only -> what to do? | S1 |
| 22.081 | Line Identification supplementary services; Stage 1 | Rel-11 | 2G - 3G only -> what to do? | S1 |
| 22.082 | Call Forwarding (CF) Supplementary Services; Stage 1 | Rel-10 | 2G - 3G only -> what to do? | S1 |
| 22.083 | Call Waiting (CW) and Call Holding (HOLD); Supplementary Services; Stage 1 | Rel-4 | 2G - 3G only -> what to do? | S1 |
| 22.084 | MultiParty (MPTY) supplementary service; Stage 1 | Rel-4 | 2G - 3G only -> discontinue? | S1 |
| 22.085 | Closed User Group (CUG) supplementary services; Stage 1 | Rel-10 | 2G - 3G only -> discontinue? | S1 |
| 22.086 | Advice of Charge (AoC) supplementary services; Stage 1 | Rel-6 | 2G - 3G only -> discontinue? | S1 |
| 22.087 | User-to-User Signalling (UUS); Service description; Stage 1 | Rel-99 | 2G - 3G only -> discontinue? | S1 |
| 22.088 | Call Barring (CB) supplementary services; Stage 1 | Rel-11 | 2G - 3G only -> discontinue? | S1 |
| 22.250 | IP Multimedia Subsystem (IMS) Group Management; Stage 1 | NONE | IMS, 2G, 3G, 4G -> continue? | S1 |
| Other specification that appear crucial to continue, though not actively maintained |  |  |  |  |
| 22.016 | International Mobile station Equipment Identities (IMEI) | Rel-8 | Rel-16 inclusive language: 5G spec => should continue? | S1 |
| 22.030 | Man-Machine Interface (MMI) of the User Equipment (UE) | Rel-13 | UI display requirements: 2G, 3G, 4G only -> discontinue? | S1 |
| 22.038 | (U)SIM Application Toolkit (USAT); Service description; Stage 1 | Rel-8 | 2G, 3G, 4G only -> continue? | S1 |
| 22.041 | Operator Determined Barring (ODB) | Rel-11 | 2G, 3G, 4G only -> discontinue? | S1 |
| 22.042 | Network Identity and TimeZone (NITZ); Service description; Stage 1 | Rel-5 | Corr Rel-16, Rel18 2G, 3G, 4G only -> continue? | S1 |
| 22.090 | Unstructured Supplementary Service Data (USSD); Stage 1 | Rel-7 | 2G - 3G only -> continue? | S1 |
| 22.340 | IP Multimedia Subsystem (IMS) messaging; Stage 1 | Rel-8 | IMS, 2G, 3G, 4G -> continue? | S1 |
| Living specifications |  |  |  |  |
| 22.011 | Service accessibility | Rel-19 | 2G, 3G, 4G => add 5G? CHECK | S1 |
| 22.071 | Location Services (LCS); Service description; Stage 1 | Rel-19 | IMS, 2G, 3G, 4G, 5G | S1 |
| 22.101 | Service aspects; Service principles | Rel-18 | IMS, 3G, 4G, 5G | S1 |
| 11.104 | Service requirements for cyber-physical control applications in vertical domains | Rel-19 | 5G | S1 |
| 22.105 | Services and service capabilities | Rel-12 | IMS, 2G, 3G, 4G => add 5G? CHECK | S1 |
| 22.115 | Service aspects; Charging and billing | Rel-19 | IMS, 2G, 3G, 4G => add 5G? CHECK | S1 |
| 22.119 | Maritime communication services over 3GPP system | Rel-16 | 4G, 5G | S1 |
| 22.125 | Unmanned Aerial System (UAS) support in 3GPP | Rel-19 | 5G | S1 |
| 22.137 | Integrated Sensing and Communication | Rel-19 | 5G | S1 |
| 22.153 | Multimedia priority service | Rel-19 | 5G | S1 |
| 22.156 | Mobile Metaverse Services | Rel-19 | 5G | S1 |
| 22.173 | IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1 | Rel-17 | IMS, 2G, 3G, 4G => add 5G? CHECK | S1 |
| 22.182 | Customized Alerting Tones (CAT) requirements; Stage 1 | Rel-17 | IMS, 2G, 3G, 4G => add 5G? CHECK | S1 |
| 22.183 | Customized Ringing Signal (CRS) requirements; Stage 1 | Rel-17 | IMS, 2G, 3G, 4G => add 5G? CHECK | S1 |
| 22.185 | Service requirements for V2X services | Rel-14 | 4G => add 5G? | S1 |
| 22.186 | Service requirements for enhanced V2X scenarios | Rel-16 | 5G | S1 |
| 22.220 | Service requirements for Home Node B (HNB) and Home eNode B (HeNB) | Rel-11 | 3G => add 4G? needs to be updated reconsider and discontinue? | S1 |
| 22.226 | Global Text Telephony (GTT); Stage 1 | Rel-5 | 2G, 3G, 4G regulatory requirement | S1 |
| 22.228 | Service requirements for the Internet Protocol (IP) multimedia core network subsystem (IMS); Stage 1 | Rel-17 | 2G, 3G, 4G => add IMS, 5G! | S1 |
| 22.268 | Public Warning System (PWS) requirements | Rel-19 | 2G, 3G, 4G, 5g | S1 |
| 22.278 | Service requirements for the Evolved Packet System (EPS) | Rel-17 | 4G - what to do about this? | S1 |
| 22.261 | Service requirements for the 5G system | Rel-19 | 5G - what to do about this? | S1 |
| 22.262 | Message service within the 5G System (5GS); Stage 1 | Rel-16 | 5G | S1 |
| 22.263 | Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA) | Rel-17 | 5G | S1 |
| 22.280 | Mission Critical Services Common Requirements (MCCoRe); Stage 1 | Rel-19 | 4G, 5G | S1 |
| 22.281 | Mission Critical (MC) video | Rel-16 | 4G, 5G | S1 |
| 22.282 | Mission Critical (MC) data | Rel-16 | 4G, 5G | S1 |
| 22.289 | Mobile communication system for railways | Rel-19 | 4G, 5G | S1 |
| 22.368 | Service requirements for Machine-Type Communications (MTC); Stage 1 | Rel-15 | IMS, 2G, 3G, 4G => add 5G? | S1 |
| 22.369 | Service requirements for Ambient power-enabled IoT | Rel-19 | 5G | S1 |
| 22.468 | Group Communication System Enablers (GCSE) | Rel-15 | 4G, 5G | S1 |
| Specifications under control of other groups - possibly historical & should not be advanced to 6G |  |  |  |  |
| 22.031 | 3G Security; Fraud Information Gathering System (FIGS); Service description; Stage 1 | NONE | 3G security - definitely old 2G, 3G, 4G only -> discontiune? | SA3 |
| 22.032 | Immediate Service Termination (IST); Service description; Stage 1 | NONE | 2G, 3G, 4G only -> discontinue? | SA3 |
| 22.053 | Tandem Free Operation (TFO); Service description; Stage 1 | Rel-4 | 2G - 3G only -> discontinue? | S4 |