3GPP TSG-SA WG1 Meeting #107 	    S1-242122

# Maastricht, NL, 19 - 23 Aug 2024

# Source:	ZTE Corporation, vivo, AsianInfo

# Title:	MiniWID on combined QoS configuration and monitoring

# Document for:	Agreement

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: MiniWID on Combined QoS configuration and monitoring

Acronym: 	CombQoS

Unique identifier:

Potential target Release:	Rel-20

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  | X | X |  |
| No |  | X |  |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a

|  | Study |
| --- | --- |
| X | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

Dependency on non-3GPP (draft) specification:

3	Justification

With the diversity application growing, multiple sensors, cameras, UAVs etc., can be grouped to work together for one application. It is comprised of multiple communication connections with different QoS demands and these communication connections each other have relationship according to the application demand.

3GPP has specified group communication and management related requirements in different specifications(e.g. TS 22.468, TR 22.874, TR 22.876, TS 22.261 ), which mainly focus on forming a collection of devices by the application layer to operate, e.g., a collection of smart home equipment or wearable equipment. However, considering the variable network conditions and status inside/outside the group during the operation, following challenges need be further considered:

- For one application, when communication service is provided, there are multiple session flows related with selected UEs which are constructed as a working group,  thus, not only the QoS demand for each session flow but also the overall QoS demand from the application aspect need to be considered e.g., combined latency requirement from the application aspect.

- In general, it requires not only to monitor the communication QoS status of the multiple PDU sessions within the above working group, but also the combined communication QoS status from the application aspect.

Supporting combined QoS can’t be simply equal to support the QoS of every session flow in the working group. For example, refer the following figure 1, from the application aspect, the combined transmission latency, is the time duration when the camera begins to upload data, and ends when the robot receives the data. And, the Flow1 which is used for the data transmission from camera to network with the requirement of Latency t1 and Flow2 which is used for the subsequent action from network to robot with the requirement of Latency t2. In this process, the combined latency for the application is comprised of t1, t2 and t3 not only the t1+t2.

Figure 1. Example of data transmission between entities

Each of these segments contributes to the overall combined latency, which must adhere to the stipulated requirements to ensure timely and efficient data delivery.

Figure 2. Example of data transmission ways

In addition, network may also use the required combined latency requirement to select the data transmission ways. As shown in figure 2, data keeps forwarding from camera to robot via network by using both Flow1 and Flow2. Due to some reason(e.g. Network overload), new latency t3 is much larger than original t3. Though Latency t1 and Latency t2 are met the QoS requirement for Flow1 and Flow2 individually, the combined latency requirement can’t be met(e.g., t1+t2+t3*>>combined latency requirement). In this case, to fulfill the combined latency requirement, network may find a new network entity to handle the data processing in 5G network. Thus the new combined latency is t1+t2+t4 which fulfills the requirement again.

With such combined QoS policy and monitoring, the following advantages could be achieved:

- The combined latency can further guarantee the transmission of intermediate entities involved in the data transmission process, preventing the scenario where each QoS flow involved meets its individual latency requirement, yet the overall combined latency requirement from application aspect among UEs is not met.

- The network can flexibly select the data transmission ways based on the combined latency requirement and the latency requirement information for each related QoS flows.

4	Objective

The objective of this work item is to address the relevant requirements for the 5G system:

- To enable an authorized 3rd party to provide combined QoS policies together with QoS policies for associated QoS flows

- To provide communication service based on  QoS policy for an application together with the QoS policies for associated QoS flows.

- To support and expose the combined QoS status for one application together with QoS status for associated QoS flows.

5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 22.261 | Add requirements on QoS and QoS monitoring to support combined QoS configuration and monitoring | TSG#105 |  |

6	Work item Rapporteur(s)

Yansheng Liu, ZTE , liu.yansheng@zte.com.cn

7	Work item leadership

SA1

8	Aspects that involve other WGs

None identified yet

9	Supporting Individual Members

Supporting IM name

ZTE Corporation

vivo

AsianInfo