3GPP TSG-SA1 Meeting #107	S1-242138

Maastricht, NL, 19-23 August 2024	(revision of S1-24XXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.153 | CR | 0064 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MPS Subscription Alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs, CISA ECD, AT&T, Verizon, T-Mobile US |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS2 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Stage 2/3 feature was added to cover use of network provided information indicating that the UE has a subscription for MPS in addition to the case of using the information stored on the UE. Specifically, TS 24.501 § clause 4.5.2 indicates the following: “Access identity 1 is valid when: - the USIM file EFUAC_AIC indicates the UE is configured for access identity 1 and the selected PLMN, if a new PLMN is selected, or RPLMN is the HPLMN (if the EHPLMN list is not present or is empty) or EHPLMN (if the EHPLMN list is present), or a visited PLMN of the home country; - the UE receives the 5GS network feature support IE with the MPS indicator bit set to "Access identity 1 valid" from the RPLMN as described in subclause 5.5.1.2.4 and subclause 5.5.1.3.4; or - the UE receives the Priority indicator IE with the MPS indicator bit set to "Access identity 1 valid" from the RPLMN as described in subclause 5.4.4.3.” Stage 1 text only covers the case of using the information stored on the UE and need to be aligned with the stage 2/3 features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text changes to cover the use of network provided information of the UE subscription for MPS to align with stage 2/3 features. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 misalignment with stage 2/3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.8.1, 9.1.1, 9.2.1, and 9.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START of CHANGE***

*** START of 1st CHANGE***

## 5.1	General

The system shall provide preferential treatment based on the subscription for MPS for:

-	An authorized Service User using a UE with an MPS subscription,

NOTE 1:	The MPS subscription related information is associated with the UE’s HPLMN subscription and is either stored in the UE and the HPLMN, or only in the HPLMN.  In the case that the MPS subscription is stored in the UE, the UE’s membership in a special access class as per TS 22.011 [6] is used for E-UTRAN access to the EPC and membership in Access Identity 1 as per TS 22.261 [7] is used for 5GS (NR and E-UTRA access to the 5GC). In the case where the MPS subscription is stored only in the HPLMN (5GC), the HPLMN (5GC) can provide MPS indication during the UE registration.

-	An authorized Service User using a UE that does not have an MPS subscription, and

NOTE 2:	In this case, the Service User’s MPS subscription information (e.g., credentials) is not associated with the UE’s HPLMN subscription. For example, the Service User borrows a UE that does not have an MPS subscription.

-	An authorized IoT device with an MPS subscription.

NOTE 3:	The MPS subscription related information is associated with the IoT device’s HPLMN subscription and is either stored in the IoT device and the HPLMN, or only in the HPLMN to allow invocation/revocation of MPS through machine interactions possibly without involving a human.

Upon invocation of MPS, the system shall provide preferential treatment for access and core network resources associated with the session (i.e., signalling and media bearer related resources within a domain and across domains), including, when applicable, priority treatment towards an enterprise network supporting MPS.

NOTE 4: ‘enterprise network’ (mentioned above, and in other following sections) may be a 3GPP or non-3GPP network.

A Service User is assigned a priority level by a regional/national authority i.e., agency authorised to issue priority levels. Upon MPS invocation the calling Service User's priority level is used to identify the priority to be used for the session being established.

Pre-emption of active sessions shall be subject to regional/national regulatory requirements.

Subject to regional/national regulatory policy, a PLMN should have the capability to retain public access as a fundamental function. Therefore, MPS traffic volumes should be limited (e.g. not to exceed a regional/national specified percentage of any concentrated network resource, such as eNodeB capacity), so as not to compromise this function.

*** END of 1st CHANGE***

*** START of 2nd CHANGE***

### 5.8.1	Roaming within the home country

The system shall support MPS when a UE with an MPS subscription is roaming within the home country and the visited and home network support MPS, and roaming agreements are in place for MPS.

NOTE 1:	Roaming within a home country is subject to national/regional rules and operator policy.

NOTE 2:	The MPS subscription related information associated with the UE’s HPLMN subscription is used for roaming within the home country.  Membership in a special access class as per TS 22.011 [6] is used for EUTRAN access to the EPC and membership in Access Identity 1 as per TS 22.261 [7] clause 6.22.2.2 is used for 5G (NR and E-UTRAN access to the 5GC).

*** END of 2nd CHANGE***

*** START of 3rd CHANGE***

### 9.1.1	General

MPS for MMTEL voice provides the Service User with priority voice communication sessions in periods of severe network congestion during which normal commercial voice service is degraded.

The system shall support:

-	MPS for MMTEL voice and voice conference calls for an authorized Service User using a UE with a subscription for MPS,

NOTE 1:	The MPS subscription related information is associated with the UE’s HPLMN subscription and is either stored in the UE and the HPLMN, or only in the HPLMN.  In the case that the MPS subscription is stored in the UE, the UE’s membership in a special access class as per TS 22.011 [6] is used for EUTRAN access to the EPC and membership in Access Identity 1 as per TS 22.261 [7] is used for 5G (NR and E-UTRAN access to the 5GC). In the case where the MPS subscription is stored only in the HPLMN (5GC), the HPLMN (5GC) can provide MPS indication during the UE registration.

-	MPS for MMTEL voice and voice conference calls for an authorized Service User using a UE that does not have an MPS subscription, and

NOTE 2:	In this case, the Service User’s MPS subscription information (e.g., credentials) is not associated with the UE’s HPLMN subscription. For example, the Service User borrows a UE that does not have an MPS subscription.

-	MPS for all participants of an authorized MMTEL voice conference call.

NOTE 3:	MPS for all participants of an MMTEL voice conference call allows a Service User (i.e., the host) to establish an MMTEL voice conference with MPS for all participants, or upgrade on ongoing MMTEL voice conference to MPS for all participants regardless of their subscription to MPS.

*** END of 3rd CHANGE***

*** START of 4th CHANGE***

### 9.2.1	General

The primary purpose of MPS for MMTEL video is to provide the Service User with priority video communication sessions in periods of severe network congestion during which normal commercial video service is degraded.

The system shall support:

-	MPS for MMTEL video and video conference calls for an authorized Service User using a UE with a subscription for MPS,

NOTE 1:	The MPS subscription related information is associated with the UE’s HPLMN subscription and is either stored in the UE and the HPLMN, or only in the HPLMN.  In the case that the MPS subscription is stored in the UE, the UE’s membership in a special access class as per TS 22.011 [6] is used for EUTRAN access to the EPC and membership in Access Identity 1 as per TS 22.261 [7] is used for 5G (NR and E-UTRAN access to the 5GC). In the case where the MPS subscription is stored only in the HPLMN (5GC), the HPLMN (5GC) can provide MPS indication during the UE registration.

-	MPS for MMTEL video and video conference calls for an authorized Service User using a UE that does not have an MPS subscription, and

NOTE 2:	In this case, the Service User’s MPS subscription information (e.g., credentials) is not associated with the UE’s HPLMN subscription. For example, the Service User borrows a UE that does not have an MPS subscription.

-	MPS for all participants of an authorized MMTEL video conference call.

NOTE 3:	MPS for all participants of an MMTEL video conference call allows a Service User (i.e., the host) to establish an MMTEL video conference with MPS for all participants, or upgrade on ongoing MMTEL video conference to MPS for all participants regardless of their subscription to MPS.

*** END of 4th CHANGE***

*** START of 5th CHANGE***

### 9.3.1	General

MPS for Data Transport Service (DTS) is a generic priority packet transport service that applies independently of the specific data application being used.  It is a specific example of Priority Data Bearer Service described in clause 5.4a. MPS for DTS provides the Service User with priority for applications using the default bearer upon request (in the case of EPS), or QoS flow associated with the default QoS rule upon request (in the case of 5GS),  to one or more selected active Packet Data Networks (PDNs)/Data Networks (DNs), in periods of severe network congestion during which normal commercial data service is degraded. MPS for DTS may also apply to other bearers (in the case of EPS) and other QoS Flows (in the case of 5GS) based on operator policy and regulatory rules.

NOTE 1:  Service Data Flows (SDFs) previously mapped to the default bearer (in case of EPS) / QoS flow associated with the default QoS rule (in case of 5GS), are upgraded to MPS priority.

The system shall support:

-	MPS for DTS for an authorized Service User using a UE with a subscription for MPS, and

NOTE 2:	The MPS subscription related information is associated with the UE’s HPLMN subscription and is either stored in the UE and the HPLMN, or only in the HPLMN.  In the case that the MPS subscription is stored in the UE, the UE’s membership in a special access class as per TS 22.011 [6] is used for EUTRAN access to the EPC and membership in Access Identity 1 as per TS 22.261 [7] is used for 5G (NR and E-UTRAN access to the 5GC). In the case where the MPS subscription is stored only in the HPLMN (5GC), the HPLMN (5GC) can provide MPS indication during the UE registration.

-	MPS for DTS for an authorized Service User using a UE that does not have an MPS subscription.

NOTE 3:	In this case, the Service User’s MPS subscription information (e.g., credentials) is not associated with the UE’s HPLMN subscription. For example, the Service User borrows a UE that does not have an MPS subscription.

MPS for DTS may be used to by a Service User to obtain priority access (e.g., Virtual Private Network (VPN) access) to an enterprise network (e.g., a private corporate network or private Government Agency network to download data and/or access enterprise applications such as email, text/chat/presence, voice and video).

The system shall support for an authorised Service User, an end-to-end MPS for DTS connection between the MPS subscribed UE and an associated enterprise network supporting MPS on demand based on operator policy and when agreements are in place for MPS.

NOTE 4:	The end-to-end MPS for DTS connection between the UE and an associated enterprise network is supported either using a web browser (e.g., use of a predetermined address (i.e., URL) in a web browser) or using a user-selectable option provided as part of the VPN client in the UE with an MPS subscription.

MPS for DTS provides an IoT device with subscription to MPS with priority for applications using a specific default bearer (in the case of EPS), or default QoS flow (in the case of 5GS), towards a single specified PDN/DN;in the case of IoT connectivity to an enterprise, MPS also provides priority for the terminating access network (connected to the enterprise).

The system shall support:

-	MPS for DTS for an authorized IoT device with a subscription for MPS, and

-	an end-to-end MPS for DTS connection between the MPS subscribed IoT device and an associated enterprise network supporting MPS on demand based on operator policy when agreements are in place for MPS.

NOTE 5:	An MPS specialized application on the MPS subscribed IoT device is used to support MPS for DTS through machine interactions that may not directly involve a human Service User.

An authorized enterprise network supporting MPS uses MPS for DTS to activate priority for remote MPS subscribed UEs and MPS subscribed IoT devices associated with the enterprise network based on operator policy when agreements are in place for MPS.

The system shall support:

-	for an authorized enterprise network supporting MPS, activation of MPS for DTS for remote MPS subscribed UEs associated with the enterprise network on demand based on operator policy when agreements are in place for MPS, and

-	for an authorized enterprise network supporting MPS, activation of MPS for DTS for remote MPS subscribed IoT devices associated with the enterprise network on demand based on operator policy when agreements are in place for MPS.

NOTE 6:	MPS for DTS activation by an authorized enterprise network supporting MPS involves prearrangement and configuration (i.e., subscription arrangement identifying the enterprise network authorization for remote activation of DTS in IoT devices) within the MPS Service Provider PLMN.

*** END of 5th CHANGE***

*** END CHANGE***