3GPP TSG- SA1 Meeting # 107	S1-242196

Maastricht, The Netherlands, 19-23 August 2024	(revision of S1-24xxxx)

# Source:	CATT, China Unicom, AsiaInfo, Honor

# Title:	New WID on Supporting Service Continuity of Positioning Services for a MUSIM UE

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Supporting Service Continuity of Positioning Services for a MUSIM UE

Acronym: POS4MUSIM

Unique identifier:

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x |  | x |  |
| No | x |  | x |  |  |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
| x | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 760006 | Study on positioning use cases(FS_5G_HYPOS) | Rel-16 study on 5G positioning service |
| 800049 | 5G positioning services(5G_HYPOS) | Rel-16, Stage 1 of 5G_HYPOS |
| 830019 | Study on Support for Multi-USIM Devices(FS_MUSIM) | Rel-17, study the use cases and potential requirements for Multi-USIM devices in EPS and 5G system |
| 840040 | Support for Multi-USIM Devices(MUSIM) | Rel-17, Stage 1 of MUSIM |

Dependency on non-3GPP (draft) specification:

3	Justification

Location based service and associated service requirements were introduced in 3G. As 3GPP system evolves to 5G, positioning service becomes a critical feature to serve use cases of different verticals (e.g. smart factories, V2X) and applications (e.g. wearable devices, eHealth) as well as assist in the network operation. TS 22.071 has mentioned that LCS feature can be supported for all UEs while positioning shall be supported for all UEs regarding emergency services. TS22.261 inherits all the requirements and extends the capabilities to high accuracy and diversified configuration e.g. event-based, periodicity-based.

To cater to the need of consumer market, the support of MUSIM UE was studied and introduced in Rel-17, which will have more than one USIM and be able to maintain separate registration state with a PLMN for each USIM over 3GPP access. Based on the user preference and service policy, MUSIM UE can autonomously switch USIMs for different services while the network can coordinate the service delivery for each USIM. However, when MT and regulatory services related use cases and requirements were studied (as TS22.101 and TS22.278), the consideration about positioning service was missing.

For example, when emergency service (e.g. voice call, SMS) is initiated, the location service will be triggered as well. Even if emergency call or the transmission of short message have finished, sometimes the periodic location reporting session will keep for a while. For a MUSIM UE, there is a risk that the measurement and ongoing positioning based on one USIM will be interrupted by the swap of USIMs without any coordination of MUSIM UE and the network. Thus, the service continuity of positioning service need to be addressed by enhanced 5GS, especially when MUSIM UE switches the USIMs provided by the same operator.

4	Objective

This work item intends to specify the service requirements to support service continuity for positioning services applied on a MUSIM UE during the USIMs swap:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 22.261 | Additional requirements on supporting service continuity of positioning services for a MUSIM UE | TSG#105 |  |

6	Work item Rapporteur(s)

Hui Xu, CATT, xuhui@catt.cn

7	Work item leadership

SA WG1

8	Aspects that involve other WGs

9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

CATT

China Unicom

AsiaInfo

Honor