3GPP TSG-SA WG1 Meeting #107 	S1-242235

Aug 19 – 23, 2024, Maastricht, Netherlands

Source:	Fraunhofer IIS, vivo

pCR Title:	pCR on IMS Voice Call using GEO satellite access data rates

Draft Spec:	3GPP TR 22.887

Agenda item:	7.3

Document for:	Agreement

Abstract: This document proposes a text update for the use of IMS voice calls using GEO satellite access.

1. Introduction

This contribution proposes clarifications to the requirements of the GEO IMS voice call use case relative to TR 22.887 v0.1.0 (S1-241422).

2. Reason for Change

TR 22.887  v0.1.0 contains a requirement for the optimization of IMS voice call:

[PR 5.1.6.002] The 5G system shall provide mechanisms to optimize IMS voice call setup considering the transmission data rate provided by the GEO satellite access technologies.

Considering the data rate over GEO satellites, optimization should not just entail the call setup but also the media transmission stream (as illustrated in clause 5.1.1 of TR 22.887). To optimize for the GEO satellite access technologies it is thus a necessity lower the data rate of the RTP streams with the voice media. This should consist of

- Reduced data rate for speech codec payload

- Reduced data rate for protocol overhead

According to S1-241071, the expected data rate for the speech codec is in the range of 0.4 – 1.2 kbps. The protocol header overhead (RTP+UDP+IP with RoHC and PDCP+RLC+MAC) for the transmission is 56 bits, which is equivalent to 2.8 kbps for the regular VoIP transmission intervall of 20 ms. The ratio between payload and protocol overhead is thus in the worst case 0.4 kbps to 2.8 kbps meaning the protocol overhead is seven times higher than the actual payload.

In comparision, the worst case currently available in 3GPP (AMR at 4.75 kbps plus RTP payload header according to RFC4867 which totals to 5.6 kbps) shows a ratio of 2 for payload / protocol overhead, meaning the protocol overhead is half the codec payload.

To ensure a healthy ratio of protocol overhead vs. real payload results from the optimization process, some guidelines are proposed.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.887 v0.1.0.

* * * First Change * * * *

### 5.1.6	Potential New Requirements needed to support the use case

#### 5.1.6.1	Potential Service Requirements

[PR 5.1.6.001] A 5G system with GEO satellite access shall be able to provide IMS voice call service as defined in TS 22.228 [3].

[PR 5.1.6.002] The 5G system shall provide mechanisms to optimize IMS signalling and transport (e.g. protocol overhead) of voice calls considering the transmission data rate provided by the GEO satellite access technologies. The transport optimization shall enable a higher data rate for the payload in comparison to the protocol overhead.

[PR 5.1.6.003] The 5G system with GEO satellite access shall be able to support Lawful Interception for IMS voice services.

* * * End of Change * * * *