3GPP TSG-SA WG1 Meeting #107	S1-242267

Maastricht, The Netherlands, 19-23 August 2024						(revision of S1-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0032 | rev | - | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of requirements and gap analysis for multi-train voice communication for Drivers and Ground FRMCS User(s) using Ad hoc Group Communications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS_Ph6 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update the use case according to the latest versions of FRMCS specifications, including Ad hoc Group Communications in the gap analysis. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Multi-train voice communication based on Ad hoc Group Communication with criteria |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TR22.989 is not aligned with latest versions of FRMCS specifications but also with the current specification work in the subsequent groups |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 6.2.1, 6.2.2.3, 6.2.2.5, 6.2.2a.5, 6.2.3.5, 6.2.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Addressed area: The multi-train voice communication and REC (Railway Emergency communication) are based on the concept of predefined initiating area and predefined addressed area. Based on this initiating area, the FRMCS system will determine a corresponding predefined addressed area where the users are eligible to be addressed. The eligible users will be addressed depending on their location and functional identity. Associated to this addressed area, a list of predefined additional users (e.g. controllers) will be also addressed.

Automatic Train Operation (ATO): Automatic Train Operation applications are responsible for acceleration to the permitted speed, speed reduction where necessary due to speed restrictions and stop at designated stations in the correct location.

Automatic Train Protection (ATP): Automatic Train Protection applications are responsible for giving Limit of Movement Authority to a train based on the train’s current speed, its braking capability and the distance it can go before it must stop.

Balise: An electronic beacon or transponder placed between the rails of a railway as part of an automatic train protection or operation (ATP/ATO) system.

Business communication applications: communication applications that support the railway business operation in general, such as wireless internet, etc.

Controller (Train Controller): A Ground FRMCS User provided with special capabilities by the FRMCS System.

Driver (Train Driver): A Mobile FRMCS User provided with special capabilities by the FRMCS System.

External System(s): A general category of stationary FRMCS Users. For example, External Systems could be systems monitoring for trains passing a red light to initiate a railway emergency call.

FRMCS Application: The application on a 3GPP UE offering railway specific communication services to the FRMCS User by making use of the communication capabilities offered by the 3GPP UE and the 3GPP network.

FRMCS Equipment Identity: The identity by which a FRMCS equipment can be addressed.

FRMCS Equipment Type: Indicates the purpose the FRMCS equipment is being used for, FRMCS equipment of different equipment types do have different capabilities.

FRMCS Equipment: The FRMCS Equipment consists of a 3GPP UE and a FRMCS Application residing on it. It may be combined with legacy railway communication equipment (e.g. GSM-R or TRS)

FRMCS Functional Identity: The identity related to a user or related to the equipment, as specified in 9.3 "Role management and presence" indicating its special Role (e.g. as Driver of a specific train, usually a train number) can be addressed.

FRMCS Network: this is a sub-part of the FRMCS System.

FRMCS Roaming: The ability for a FRMCS User to make use of FRMCS Applications in a Visited (FRMCS) Network.

FRMCS System: The system providing railway specific communication constituted of the FRMCS Equipment, the 3GPP transport and the application servers in the network. Legacy networks are not included in the FRMCS System.

FRMCS User Identity: The identity by which a FRMCS User can be addressed.

FRMCS User: A human user or a machine making use of the railway specific communication. FRMCS Users can be connected via 3GPP RAT, wired connectivity or other radio technology

Ground FRMCS User: A general category of FRMCS Users that are predominantly stationary. Mostly they are connected via wired connectivity but may be using also wireless in certain conditions.

Home FRMCS Network: The Home FRMCS Network is the network in which the FRMCS User is engaged in a subscription.

Initiating area: Predefined geographical area where the initiator of a communication is located and which determines the addressing rules used for the selection of the users to be included in the communication. The initiating area is used for the establishment of a multi-train or a REC communication.

Mobile FRMCS User: A general category of FRMCS Users that are mobile. Thus, they are connected via wireless connectivity all the time.

Mobile Intelligent Assistant: 5G enabled robot with autonomous movements and artificial intelligence to support passengers in the Railway Smart Station.

Off-Network communication: direct communication between FRMCS Users in proximity.

On-Network communication: indirect communication between FRMCS Users connected to FRMCS Network(s).

Performance communication applications: applications that help to improve the performance of the railway operation, such as train departure, telemetry, etc.

Radio Block Centre (RBC): A train sends its position and speed information periodically to the RBC. The RBC uses the received information to decide movement authority of the train.

Rail Infrastructure Manager: A company that owns or manages rail infrastructure; within this document the Rail Infrastructure Manager owns, administrates and operates the FRMCS Network.

Railway Smart Station: a train station where the 5G-based services such as IoT and AI, are used for providing assisting railway services.

Railways Undertaking: A company that offers train freight or passenger transportation services, making use of FRMCS network for their operational communication needs that is operated by a Rail Infrastructure Manager.

Role (Functional Role): The function a FRMCS User or a FRMCS Equipment is currently performing. Examples of Roles are Driver, Controller or shunting staff, etc. This is indicated by the FRMCS Functional Identity.

Shunting: manoeuvring trains in order to change their location or composition.

Trackside staff: Staff working as trackside maintenance and/or shunting members

Trainborne equipment: FRMCS Equipment which is physically embedded in train

Visited (FRMCS) Network: A Visited (FRMCS) Network can be either another FRMCS Network than the Home FRMCS Network, or a Public Land Mobile Network (PLMN).

Zone: A 2-dimensional region of a pre-determined size.

Zone resolution: The pre-determined size of the given zone.

Next change

## 6.2	Multi-train voice communication for Drivers and Ground FRMCS User(s)

### 6.2.1	Introduction

In this chapter the use cases related to the function of Multi-train voice communication from the Drivers and/or a Ground FRMCS User towards the Driver and/or Ground FRMCS Users are defined. Ground FRMCS Users may include Controllers. The following use cases are defined:

- Initiation of Multi-train voice communication for Drivers and Ground FRMCS User(s) communication

- Join a Multi-train voice communication for Drivers and Ground FRMCS User(s) communication

- Termination of Multi-train voice communication for Drivers and Ground FRMCS User(s) communication

- Interworking GSM-R and FRMCS for Multi-train voice communication for Drivers and Ground FRMCS User(s) communication

NOTE 1:	The assumption is that Ad hoc Group Calls are used for all multi-train voice communications.

Next change

#### 6.2.2.3	Service flows

The Driver and/or Ground FRMCS User initiates the voice communication to the (other) Driver(s) and/or Ground FRMCS Users. The priority of the communication is managed by the prioritisation application. The voice communication has the priority which matches the application category of CRITICAL VOICE (see 12.10) within the FRMCS System.

The FRMCS System determines the Driver(s) and the Ground FRMCS User(s) to be included in the communication, based on:

- location information of all users provided by the locations services application, and/or

- functional identity of all users provided by the Role management and presence application.

- System configuration on which Ground FRMCS User is responsible for which part of the track/station/etc.

- System configuration related to Predefined Area Definition which includes:

- Definition of initiating and addressed area

- Preconfigured association of Controllers to addressed areas

The FRMCS System establishes the voice communication within a setup time specified as NORMAL (see 12.10). The information from the Role management and presence application is used to present the identities for both Driver(s) and Ground FRMCS User(s). The initiating Driver is indicated to the Ground FRMCS User(s). Also the location of the Driver(s) in the voice communication is presented to the Ground FRMCS User(s) which is retrieved from the location services application.

If the Driver and/or Ground FRMCS User is connected to more than one Drivers and/or Ground FRMCS Users, the multiuser talker control application is used.

The precedence of the incoming voice communication at the Driver and the Ground FRMCS User is managed by the prioritisation application.

The voice communication is recorded by the FRMCS System voice recording and access application.

Next change

#### 6.2.2.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.2.2-001] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), the FRMCS Users shall be able to initiate the voice communication to FRMCS Users in trains or on ground. | A | 22.280 22.179 | TS 22.179 Floor Control clause 6.2.3.2 Req #1, TS 22.280 clause 5.1 Req # 2 TS 22.280 [R-6.15.5.2-001]. Initiation of multi-train voice communication by an authorised user using Ad hoc Group Calls. |
| [R-6.2.2-002] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s) the application layer priority of the communication shall be managed by the prioritisation application. | A | 22.280 | 6.8.7 Application layer priority, 7.6 MCX Service priority requirements |
| [R-6.2.2-003] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), the FRMCS System shall be able to determine the FRMCS User(s) to be included in the voice communication, based, amongst others, on the following criteria: location information of the initiator, speed and direction of travel provided by the locations services application, and/or functional identity provided by the Role management and presence application. System configuration of addressed areas with preconfigured association of these areas to the responsible Ground FRMCS User (e.g., Train Controller). | A | 22.280 | 6.4.9 req #6, 5.17 req # 3, 6.1 req #5, R-6.1.005, R6.6.5.2-00X 5.9a-006, 5.9a-008a [R-6.15.5.2-001]-Note. Determine the participants of a multi-train voice communication using Ad hoc Group Calls based on criteria. |
| [R-6.2.2-004] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s) the FRMCS System shall be able to add or remove FRMCS User from the communication once criteria are met or no more met, e.g. by a FRMCS User entering or leaving an area | A | 22.280 | 6.4.9 req #6, 5.17 req # 3, 6.1 req #5, R-6.1.005, R6.6.5.2-00X 5.9a-006, 5.9a-008a[R-6.15.5.2-014a] using Ad hoc Group Calls criteria mechanisms to update the list of participants of a multi-train communication. |
| [R-6.2.2-005] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), the FRMCS System shall establish the communication within a setup time specified as NORMAL (see 12.10). | A/T | N/A | See section 12.10 below |
| [R-6.2.2-006] | The FRMCS System shall be able to mutually present the identities of all communication partners involved in a Multi-train voice communication for Drivers and Ground FRMCS User(s). | A | 22.280 | 5.3 req #2, 5.7 req # 3, 6.4.3 req # 1 & #2 Presentation of the id of the speaker is sufficient See reqs in 6.4.5 & [R-5.9a-001a] |
| [R-6.2.2-006a] | The FRMCS System shall be able to present the location of the Driver(s) to the Ground FRMCS Users involved in a Multi-train voice communication for Drivers and Ground FRMCS User(s). | A | 22.280 | 5.11 001, 008, 015 6.12 001,006,007 6.4.5 001, 003, 004 [R-6.15.5.2-015]. Participant’s location provided to an authorized participant of the same Ad hoc Group. |
| [R-6.2.2-006b] | The FRMCS System shall be able to update the presentation of the location of the Drivers as they move. | A | 22.280 | 5.11 007, 009, 013 6.12 006 |
| [R-6.2.2-007] | A Multi-train voice communication for Drivers and Ground FRMCS User(s) always includes more than two participants. If only two participants remain, the communication shall be treated as a user-to-user communication. | A | N/A | This is not a service requirement. It is up to 3GPP SA6 to decide whether a communication with only two has to change the mode of operation to 1-2-1 communication. |
| [R-6.2.2-008] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), multiuser talker control shall be used (See "9.7 Multiuser talker control related use cases"). | A | 22.179 | 6.2.3.7.1 – Note. Multi-user talker control also applicable to MCPTT Ad hoc Groups. |
| [R-6.2.2-009] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), on the application layer the precedence of the incoming voice communication at the Driver and the Ground FRMCS User shall be managed by the prioritisation application. | A | N/A | See section 9.16 below |
| [R-6.2.2-010] | The FRMCS System shall be able to make available the speech and communication related data of a Multi-train voice communication for Drivers and Ground FRMCS User(s) for recording. | A | 22.280 | On-Net:6.15.4-001 – 010 Off-Net: Not implemented |

Next change

#### 6.2.2a.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.2.2a-001] | The FRMCS System shall be able to notify the user application of an authorised FRMCS User about an ongoing Multi-train voice-communication for Drivers and Ground FRMCS User(s) when conditions based on location and/or functional identities are met. | A | 22.280 22.179 | TS 22.179 Floor Control clause 6.2.3.2 Req #1, TS 22.280 clause 5.1 Req # 2, 6.4.9 req #6, 5.17 req # 3, 6.1 req #5 [R-6.15.5.2-014a]. Updated list of participants based on Ad hoc group Communications. |
| [R-6.2.2a-002] | When the user application of an authorised FRMCS User receives a notification of an ongoing Multi-train voice-communication for Drivers and Ground FRMCS User(s), it shall be able to request automatically to the FRMCS System a connection to the communication. | A | 22.280 | 6.4.9 req #6, 5.17 req # 3, 6.1 req #5, 5.9a-006, 5.9a-008a [R-6.15.5.4-001]. Notification and acknowledgement for Ad hoc Group Communications. |
| [R-6.2.2a-003] | When the user application of an authorised FRMCS User receives a notification of an ongoing Multi-train voice-communication for Drivers and Ground FRMCS User(s) that has been previously left, it shall be able to request to the FRMCS System a connection to the communication again | A | 22.280 | 6.4.9 req #6, 5.17 req # 3, 6.1 req #5, R6.6.5.2-00X 5.9a-006, 5.9a-008a |
| [R-6.2.2a-004] | When a FRMCS User joins a Multi-train voice communication for Drivers and Ground FRMCS User(s), the application layer priority of the communication shall be managed by the prioritisation application. | A | 22.280 | 6.8.7 Application layer priority, 7.6 MCX Service priority requirements |
| [R-6.2.2a-005] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), the FRMCS System shall be able to determine the FRMCS User(s) to be joined in the ongoing voice communication, based, amongst others, on the following criteria: location information, speed and direction of travel provided by the locations services application, and/or functional identity provided by the Role management and presence application. | A | 22.280 | 6.4.9 req #6, 5.17 req # 3, 6.1 req #5, R-6.1.005, 5.9a-006, 5.9a-008a [R-6.15.5.2-014a]. The criterion (e.g., location) is continuously evaluated by the MCX server to update the list of participants to include in a multi-train voice communication based on Ad hoc Group Calls. |
| [R-6.2.2a-006] | When a FRMCS User joins a Multi-train voice communication for Drivers and Ground FRMCS User(s), the FRMCS System shall be able to mutually present the identities of all communication partners involved in the communication. | A | 22.280 | 5.3 req #2, 5.7 req # 3, 6.4.3 req # 1 & #2 Presentation of the id of the speaker is sufficient See reqs in 6.4.5& [R-5.9a-001a] |
| [R-6.2.2a-007] | The FRMCS System shall be able to present the location of the newly added Driver(s) to the Ground FRMCS Users involved in a Multi-train voice communication for Drivers and Ground FRMCS User(s). | A | 22.280 | 5.11 001, 008, 015 6.12 001,006,007 6.4.5 001, 003, 004 [R-6.15.5.2-015]. Provision of location information of a participant of Ad hoc Group Call to an authorised user |
| [R-6.2.2a-008] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), multiuser talker control shall be used for the newly added FRMCS Users (See "9.7 Multiuser talker control related use cases"). | A | 22.179 | 6.2.3.7.1 – Note. Multi-user talker control also applicable to MCPTT Ad hoc Groups. |
| [R-6.2.2a-009] | When a FRMCS User joins an on-going Multi-train voice communication for Drivers and Ground FRMCS User(s), the precedence of the incoming voice communication at the Driver and the Ground FRMCS User shall be managed by the prioritisation application. | A | N/A | See section 9.16 below |
| [R-6.2.2a-010] | When a FRMCS User joins an on-going Multi-train voice communication for Drivers and Ground FRMCS User(s), the FRMCS System shall be able to make available the speech and communication related data for recording. | A | 22.280 | On-Net:6.15.4-001 – 010 Off-Net: Not implemented |

Next change

#### 6.2.3.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.2.3-001] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), a Driver or a Ground FRMCS User shall be able to put the voice communication on hold. The voice communication between the remaining users shall not be affected by a Driver or Ground FRMCS User putting the voice communication on hold. | A | 22.280 | 5.4.1, 5.4.2, 5.1.5 R-6.4.4-003, R-6.4.4-004 The affiliation mechanism is considered sufficient to mimic the desired behaviour. Put a multi-train voice communication on hold based on Ad hoc Group Calls is not covered. (CR may be needed based on enhancement of voice arbitration use cases) |
| [R-6.2.3-002] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), a Driver shall be able to leave the voice communication at any time. After a Driver has left the communication, the remaining users are informed. | A | 22.280 | TS 22.280: R-6.4.4-003, R-6.4.4-004 The affiliation mechanism is considered sufficient to mimic the desired behaviour. All other use cases of leaving, without criterion evaluation, need queueing mechanisms which are not defined yet for Ad hoc Group Calls. (CR may be needed based on enhancement of voice arbitration use cases) |
| [R-6.2.3-003] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s) the Driver is able to terminate the communication based on configuration (authorised FRMCS User). | A | 22.179 22.280 | TS 22.179: 6.2.3.4, 6.2.3.5, 6.2.4 TS 22.280: R-6.4.4-003, R-6.4.4-004, [R-6.15.5.2-003] based on Ad hoc Group Calls. |
| [R-6.2.3-005] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), the FRMCS System terminates the voice communication if the last Driver has left (although (multiple) Ground FRMCS Users are still active in the communication). All involved users are informed. | A | 22.280 22.179 | 22.179 6.2.4 008 22.280 6.4.9 001 CR 22.280 0048 rev1: R-6.1-006, R-6.4.9-001 [R-6.15.5.2-003] & [R-6.15.5.2-004] based on Ad hoc Group Calls |
| [R-6.2.3-006] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), a Ground FRMCS User shall be able to leave the communication. After a Ground FRMCS User has left the communication, the remaining users are informed. | A | 22.280 | 6.4.4 002; 6.4.5 001; 5.1.5 003-008 R-6.4.4-003, R-6.4.4-004 The affiliation mechanism is considered sufficient to mimic the desired behaviour. Same comment as for [R-6.2.3-002], also applicable to controllers. |
| [R-6.2.3-007] | For a Multi-train voice communication for Drivers and Ground FRMCS User(s), authorisedController shall be able to terminate the communication. All involved users are informed. | A | 22.280 | [R-6.15.5.2-003]. Termination of Ad hoc Group Calls by an authorised user &[R-6.15.5.2-005]. The Ad hoc Group shall not persist, when the Ad hoc Group Communication has terminated. |

Next change

#### 6.2.4.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.2.4-001] | The FRMCS System shall provide the necessary means to allow FRMCS Users to be reachable from a legacy GSM-R system. Interworking between FRMCS and GSM-R shall not require any changes on GSM-R network side. | A | 22.179 22.280 | TS 22.179: Covered in 6.18.4.2 TS 22.280: R-6.17.3.1-001 to 005 |
| [R-6.2.4-001a] | The FRMCS System shall provide the necessary means to Driver attached to the GSM-R system to set up a Multi-train voice communication for Drivers and Ground FRMCS User(s) for users attached to the FRMCS system. Interworking between FRMCS and GSM-R shall not require any changes on GSM-R network side | A | 22.179 22.280 | TS 22.179: Covered in 6.18.4.2 TS 22.280: R-6.17.3.1-001 to 005 |
| [R-6.2.4-002] | The FRMCS System shall provide the necessary means to FRMCS Users to set up a Multi-train voice communication for Drivers and Ground FRMCS User(s) also to users in legacy GSM-R system. Interworking between FRMCS and GSM-R shall not require any changes on GSM-R network side. | A | 22.179 | Covered in 6.18.4.2 |
| [R-6.2.4-003] | For Multi-train voice communication for Drivers and Ground FRMCS User(s), when the GSM-R UE becomes detached from the GSM-R system, e.g. due to coverage problems, the end user device, if capable of making use of the FRMCS System, shall be able to set up the communication on the FRMCS System. An interruption of voice communication is acceptable. Note 1: This use case only applies to end user devices containing a FRMCS UE and a GSM-R UE. It is assumed the FRMCS Application on the FRMCS Equipment will have some control of the GSM-R part of the UE which is referred to here as end user device. | A | 22.280 | Covered in [R-6.15.5.2-001] MCX service Ad hoc Group Communication |
| [R-6.2.4-004] | For Multi-train voice communication for Drivers and Ground FRMCS User(s), when the FRMCS Equipment becomes detached from the FRMCS System, e.g. due to coverage problems, the end user device, if capable of making use of legacy GSM-R, shall be able to set up the communication on legacy GSM-R. An interruption of voice communication is acceptable. Note 2: This use case only applies to end user devices containing a FRMCS Equipment and a GSM-R UE. It is assumed the FRMCS Application on the FRMCS Equipment will have some control of the GSM-R part of the UE which is referred to here as end user device. | A | N/A | Covered by GSM-R “Late Entry |

.

End of changes