3GPP TSG-SA WG1 Meeting #107 	 S1-242274

Maastricht, The Netherlands, 19-23 August 2024

Title:	Use case FR2 broadcast/multicast downlink with a FR1 NGSO uplink feedback for reliable communications

Agenda Item:	7.3

Source:	Viasat Inc., Inmarsat, Novamint, SES

Contact:		Nandan Das Nandan.das@viasat.com
Jim Petranovich jim.petranovich@viasat.com

Abstract:  In this note we present multiple use cases for multi-orbit satellite systems.  We consider satellites in geostationary, i.e. geosynchronous equatorial plane, orbits (GEO) orbits as well as non-geostationary (NGSO) orbits.  NGSO orbits could include medium (MEO), low (LEO), and highly elliptical (HEO) Earth orbits.  These use cases are envisaged for a system that has multiple satellites across different orbits, potentially with different characteristics and potentially using different frequency bands, coexisting in one network.

# Introduction:

Different satellite orbits bring different characteristics that prove advantageous in different applications.  E.g. GEO orbits, where the satellite has a field of view (FOV) of about 1/3 of the earth’s surface area is great for coverage and for flexibly moving bandwidth and capacity from areas of low demand to areas of high demand in real time. LEO orbits with an altitude of a few hundred km are useful for latency sensitive applications and in some cases can provide line-of-sight (LoS) advantages, link budget advantages (this may or may not be the case depending on the satellite size and orbit altitude) and in some cases polar or high latitude coverage depending on the constellation configuration.  HEOs are very effective at covering latitudes close to the polar regions, and typically cover polar regions of the Earth very effectively.   A system that combines some or all these orbits in a multi-orbit satellite network will be able to take advantage of these characteristics to cover a wide range of use cases that would be unlikely to be satisfied using only one orbit.

In this paper, we describe one of the multiple such use cases.

---------- Use Case template ----------

## 2.3	FR2 GEO downlink for broadcast/multicast with FR1 LEO uplink for reliable communications

### 2.3.1	Description

FR2 typically has a lot more bandwidth than FR1.  Thus enables users to have true broadband connectivity on the downlink over FR2.  GEO satellites can thus use their large FOV together with more available BW to do broadcast/multicast to a large number of users spread over a large geographical area.

Due to typical asymmetrical data usage, the return link (uplink), can be accommodated by a FR1 link.  This link may be over a LEO satellite.

The use case envisages a FR2 GEO satellite (e.g. a Ka-band GEO satellite) co-existing in a network with a FR1 LEO satellite (e.g. L-band LEO).  This also reduces the cost of a UE, since the narrowband FR1 uplink RF path is typically much cheaper than a broadband FR2 uplink RF path.

### 2.3.2	Pre-conditions

- The network supports both GEO and NGSO satellites

- The network supports handover between GEO satellite(s) and NGSO satellite(s)

- UEs can close a link to both FR2 GEO (downlink) and FR1 NGSO (uplink) satellites

- UEs are able to handle the Doppler of NGSO satellites

- UEs are able to tolerate a differential delay inherent in the GEO and NGSO networks

- GEO satellites provide broad coverage via (potentially) spot beams or regional beams

- NGSO satellites with focused coverage via on-demand spot beams

- A network plan that deconflicts frequency allocations between the GEO and NGSO satellites.

- The network is able to determine the location of a user to a sufficient granularity in order to be able to activate or redirect the user to the correct NGSO coverage beam or cell

- The different satellite orbits employed by the network may be supported by the same or different RAT types

- The network is designed to send downlink data packets over a GEO link and receive the uplink data packets via the LEO link

- The network is able to create multicast/broadcast groups which UEs can join.

### 2.3.3	Service Flows

- The network shall determine the routing needed in order to send downlink data packet to the UE via the GEO path, while receiving uplink data packets through the LEO path.

- Creation of multicast/broadcast groups

- UE joins multicast/broadcast group

- Control and Management traffic similarly needs to accommodate the two paths.

### 2.3.4	Post-conditions

The UE is able to receive high throughput broadband data via a multicast/broadcast group on a GEO link while using a LEO for the uplink.

### 2.3.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 2.3.6	Potential New Requirements needed to support the use case

[PR 5.x.6.001] A 5G system with satellite access supporting multi orbit operation, shall be able to support a mechanism for

[PR 5.x.6.002] downlink transmission via a GEO link and an uplink transmission via a NGSO link.