3GPP TSG-SA WG1 Meeting #107 	 S1- 242275

Maastricht, The Netherlands, 19-23 August 2024

Title:	Use Case:  Multi-orbit backhaul

Agenda Item:	7.3

Source:	Viasat Inc., Inmarsat, Novamint, SES

Contact:		Nandan Das Nandan.das@viasat.com
Jim Petranovich jim.petranovich@viasat.com

Abstract:  In this note we present multiple use cases for multi-orbit satellite systems.  We consider satellites in geostationary, i.e. geosynchronous equatorial plane, orbits (GEO) orbits as well as non-geostationary (NGSO) orbits.  NGSO orbits could include medium (MEO), low (LEO), and highly elliptical (HEO) Earth orbits.  These use cases are envisaged for a system that has multiple satellites across different orbits, potentially with different characteristics and potentially using different frequency bands, coexisting in one network.

# Introduction:

Different satellite orbits bring different characteristics that prove advantageous in different applications.  E.g. GEO orbits, where the satellite has a field of view (FOV) of about 1/3 of the earth’s surface area is great for coverage and for flexibly moving bandwidth and capacity from areas of low demand to areas of high demand in real time. LEO orbits with an altitude of a few hundred km are useful for latency sensitive applications and in some cases can provide line-of-sight (LoS) advantages, link budget advantages (this may or may not be the case depending on the satellite size and orbit altitude) and in some cases polar or high latitude coverage depending on the constellation configuration.  HEOs are very effective at covering latitudes close to the polar regions, and typically cover polar regions of the Earth very effectively.   A system that combines some or all these orbits in a multi-orbit satellite network will be able to take advantage of these characteristics to cover a wide range of use cases that would be unlikely to be satisfied using only one orbit.

In this paper, we describe one of the multiple such use cases.

---------- Use Case template ----------

## 2.3	GEO backhauling traffic from NGSO orbit.

### 2.3.1	Description

### NGSO satellites have a small FOV and often do not find a GW to download data to a GW. This is especially true when the NGSO satellite is over oceans.  Some NGSO orbits (e.g. sun-synchronous orbit (SSO)) are such that the gateways are placed only in the polar regions.  These orbits are especially popular for imaging, reconnaissance, and weather satellites.  For expediency of data transfer the NGSO satellite might need to offload data when there are no GWs in its FOV.

### In this case we envisage an use-case where the NGSO satellite offloads its data to a GEO satellite, and the GEO satellite downloads it to a GW.  This is a backhaul application.

### This could also have in the reverse direction, where some information has to be uploaded to the NGSO satellite when there are no GWs in its FOV.

### 2.3.2	Pre-conditions

- The network supports both GEO and NGSO satellites

- The network supports handover between GEO satellite(s) and NGSO satellite(s)

- UEs can close a link to both FR2 GEO (downlink) and FR1 NGSO (uplink) satellites

- UEs are able to handle the Doppler of NGSO satellites

- UEs are able to tolerate a differential delay inherent in the GEO and NGSO networks

- GEO satellites provide broad coverage via (potentially) spot beams or regional beams

- NGSO satellites with focused coverage via on-demand spot beams

- A network plan that deconflicts frequency allocations between the GEO and NGSO satellites.

- The network is able to determine the location of a user to a sufficient granularity in order to be able to activate or redirect the user to the correct NGSO coverage beam or cell

- The different satellite orbits employed by the network may be supported by the same or different RAT types

- The NGSO satellite is able to connect to a GSO satellite over an RF or Optical link.

- The system is able to expect a backhaul between a NGSO satellite and GSO satellite.

### 2.3.3	Service Flows

- The NGSO satellite determines that it needs to offload data to a GEO satellite (perhaps because there are no GWs in its FOV, or due to other reasons (e.g. memory limitations on the NGSO satellite))

- The NGSO satellite establishes a link to the GEO satellite

- The NGSO satellite transfers its data to the GEO satellite

- The GEO satellite downloads the data to the GW

### 2.3.4	Post-conditions

The GEO satellite backhauls the NGSO satellites data in either direction.

### 2.3.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 2.3.6	Potential New Requirements needed to support the use case

[PR 5.x.6.001] A 5G system with satellite access supporting multi orbit operation, shall be able to support a mechanism for

[PR 5.x.6.002] GEO backhaul of NGSO data