3GPP TSG-SA WG1 Meeting #107 	 S1- 242276

Maastricht, The Netherlands, 19-23 August 2024

Title:	Use Case:  Application aware traffic steering/switching between satellites in different orbits.

Agenda Item:	7.3

Source:	Viasat Inc., Inmarsat, Novamint, SES

Contact:		Nandan Das Nandan.das@viasat.com
Jim Petranovich jim.petranovich@viasat.com

Abstract:  In this note we present multiple use cases for multi-orbit satellite systems.  We consider satellites in geostationary, i.e. geosynchronous equatorial plane, orbits (GEO) orbits as well as non-geostationary (NGSO) orbits.  NGSO orbits could include medium (MEO), low (LEO), and highly elliptical (HEO) Earth orbits.  These use cases are envisaged for a system that has multiple satellites across different orbits, potentially with different characteristics and potentially using different frequency bands, coexisting in one network.

# Introduction:

Different satellite orbits bring different characteristics that prove advantageous in different applications.  E.g. GEO orbits, where the satellite has a field of view (FOV) of about 1/3 of the earth’s surface area is great for coverage and for flexibly moving bandwidth and capacity from areas of low demand to areas of high demand in real time. LEO orbits with an altitude of a few hundred km are useful for latency sensitive applications and in some cases can provide line-of-sight (LoS) advantages, link budget advantages (this may or may not be the case depending on the satellite size and orbit altitude) and in some cases polar or high latitude coverage depending on the constellation configuration.  HEOs are very effective at covering latitudes close to the polar regions, and typically cover polar regions of the Earth very effectively.   A system that combines some or all these orbits in a multi-orbit satellite network will be able to take advantage of these characteristics to cover a wide range of use cases that would be unlikely to be satisfied using only one orbit.

In this paper, we describe one of the multiple such use cases.

---------- Use Case template ----------

## 2.3	Application aware traffic steering/switching between different satellites in different orbits.

### 2.3.1	Description

### Satellites in different orbits have different characteristics.  E.g. GEOs have relatively higher latency than LEOs.  FR2 GEOs will typically have much higher bandwidth than FR1 NGSOs.

### Different user applications also have different QoS and BW requirements.  Texting/email (not latency sensitive) etc., is very different than real time video conferencing (delay sensitive).  For large file transfer we want high bandwidth and don’t care so much about latency, and so on.

### The use case here envisages a QoS and traffic aware scheduler scheduling data via different orbits depending on the requirements of the application.  In addition, this scheduler will also be aware of availability of the link due to congestion, rain fade and other parameters.  High BW, latency tolerant data may be sent via a GEO path, whereas latency intolerant data may be sent via a NGSO path.

### 2.3.2	Pre-conditions

- The network supports both GEO and NGSO satellites

- The network supports handover between GEO satellite(s) and NGSO satellite(s)

- UEs can close a link to both FR2 GEO (downlink) and FR1 NGSO (uplink) satellites

- UEs are able to handle the Doppler of NGSO satellites

- UEs are able to tolerate a differential delay inherent in the GEO and NGSO networks

- GEO satellites provide broad coverage via (potentially) spot beams or regional beams

- NGSO satellites with focused coverage via on-demand spot beams

- A network plan that deconflicts frequency allocations between the GEO and NGSO satellites.

- The network is able to determine the location of a user to a sufficient granularity in order to be able to activate or redirect the user to the correct NGSO coverage beam or cell

- The different satellite orbits employed by the network may be supported by the same or different RAT types

- An application and QoS aware scheduler that routes traffic down the satellite that is best matched for its QoS needs.

### 2.3.3	Service Flows

- Scheduler determines application and its necessary QoS

- Scheduler evaluates link parameters of each of the links available (BW, latency etc).

- Scheduler determines optimal routing and routes traffic appropriately.

### 2.3.4	Post-conditions

A better user experience as traffic is routed via the optimal route that is best suited for the QoS needs of that particular application.

### 2.3.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 2.3.6	Potential New Requirements needed to support the use case

[PR 5.x.6.001] A 5G system with satellite access supporting multi orbit operation, shall be able to support a mechanism for

[PR 5.x.6.002] Splitting data plane traffic via different paths based on QoS