3GPP TSG-SA WG1 Meeting #107 	 S1-242280

Maastricht, The Netherlands, 19-23 August 2024

Title:	Use case on using GEO for low rate true broadcast or massive multicast communications

Agenda Item:	7.3

Source:	Viasat Inc., Inmarsat, Novamint, SES

Contact:		Nandan Das Nandan.das@viasat.com
Jim Petranovich jim.petranovich@viasat.com

Abstract:  In this note we present multiple use cases for multi-orbit satellite systems.  We consider satellites in geostationary, i.e. geosynchronous equatorial plane, orbits (GEO) orbits as well as non-geostationary (NGSO) orbits.  NGSO orbits could include medium (MEO), low (LEO), and highly elliptical (HEO) Earth orbits.  These use cases are envisaged for a system that has multiple satellites across different orbits, potentially with different characteristics and potentially using different frequency bands, coexisting in one network.

# Introduction:

Different satellite orbits bring different characteristics that prove advantageous in different applications.  E.g. GEO orbits, where the satellite has a field of view (FOV) of about 1/3 of the earth’s surface area is great for coverage and for flexibly moving bandwidth and capacity from areas of low demand to areas of high demand in real time. LEO orbits with an altitude of a few hundred km are useful for latency sensitive applications and in some cases can provide line-of-sight (LoS) advantages, link budget advantages (this may or may not be the case depending on the satellite size and orbit altitude) and in some cases polar or high latitude coverage depending on the constellation configuration.  HEOs are very effective at covering latitudes close to the polar regions, and typically cover polar regions of the Earth very effectively.   A system that combines some or all these orbits in a multi-orbit satellite network will be able to take advantage of these characteristics to cover a wide range of use cases that would be unlikely to be satisfied using only one orbit.

In the related tdocs, we describe one of multiple such use cases.

---------- Use Case template ----------

## 2.4	Use case on using GEO for low rate true broadcast or massive multicast communications

### 2.4.1	Description

With a wide FOV, GEO satellites are perfect for broadcast (or multicast) to a large number of UEs.  A multi-orbit system with both broadcast/multicast and unicast traffic can take advantage of this by using the GEO for broadcast/multicast and using the NGSO for unicast traffic.

### 2.4.2	Pre-conditions

- The network supports both GEO and NGSO satellites

- The network supports handover between GEO satellite(s) and NGSO satellite(s)

- UEs can close a link to both GEO and NGSO satellites

- UEs are able to handle the Doppler of NGSO satellites

- UEs are able to tolerate a differential delay inherent in the GEO and NGSO networks

- GEO satellites provide broad coverage via (potentially) spot beams or regional beams

- NGSO satellites with focused coverage via on-demand spot beams

- A network plan that deconflicts frequency allocations between the GEO and NGSO satellites.

- The network is able to determine the location of a user to a sufficient granularity in order to be able to activate or redirect the user to the correct NGSO coverage beam or cell

- The different satellite orbits employed by the network may be supported by the same or different RAT types

- The UE must be able to simultaneously receive both a GEO link and a NGSO link.  Alternatively the UE must be able to receive the GEO link, move to a NGSO link and then move back at the appropriate time to the GEO

### 2.4.3	Service Flows

- UE request for specific multicast/unicast services

- Network determines which satellite systems support these services

- UE establishes connection with appropriate GEO satellite system

- Connection is established between UE and GEO satellite system while maintaining existing NGSO connection

### 2.4.4	Post-conditions

UE has connectivity for unicast over NGSO and for multicast/broadcast of GEO

### 2.4.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 2.4.6	Potential New Requirements needed to support the use case

[PR 5.x.6.001] A 5G system with satellite access supporting multi orbit operation, shall be able to support a mechanism to associate specific multicast/broadcast services with specific satellite systems