3GPP TSG- SA1 Meeting # 107	S1-242311

Maastricht, The Netherlands, 19-23 August 2024	(revision of S1-242249, SP-240199)

# Source:	UIC

# Title:	Revised SID: Study on FRMCS Phase 6

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on FRMCS Phase 6

Acronym :	FS_FRMCS_Ph6

Unique identifier :	1030043

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X | MCX (Application layer) |
| No | X |  | X |  |  |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

| X | Study |
| --- | --- |
|  | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 750004 | Study on Future Railway Mobile Communication System | Preceding Rel-15 study |
| 760005 | Study on Future Railway Mobile Communication System2 | Preceding Rel-16 study |
| 840043 | Study on Future Railway Mobile Communication System3 | Preceding Rel-17 study |
| 900026 | Study on FRMCS Evolution | Preceding Rel-18 study |
| 950007 | Study on FRMCS Phase 5 | Preceding Rel-19 study |
| 1000031 | FRMCS_Ph5 | WID with normative impact in Rel-19, as an outcome of the related SID: FS_FRMCS_Ph5 |

# 3	Justification

In the framework of the UIC FRMCS program stable draft specifications are released on a regular basis, including revisions of the Functional Use Cases (FU-7110), the Functional Requirements Specifications (FRS, FU-7120), as further steps in the evolution of the FRMCS specifications.

The UIC FRMCS 3GPP Task Force is constantly analyzing impact of FRMCS specifications on existing stage 1 use cases and requirements collected in TR 22.989, from Rel-18 onwards. Our intention is that outcome of this task will also be collected in TR22.989, deriving potential requirements to be added in Rel-20.

The UIC FRMCS 3GPP Task Force analysis has concluded that refining existing use cases, defining new use cases, and deriving potential new requirements will be necessary to align FRMCS specifications and 3GPP standards as part of Release 20 timeframe. As an example, in case of Rel-19, the derived requirements as part of the WID FRMCS_Ph5 allowed to enhance Ad hoc Group Alert and Communication requirements in TS22.280.

# 4	Objective

The objective of this study is to update the TR22.989 to add new use-cases and enhance the existing ones as well as to derive new potential requirements to stage 1 TSs (e.g., 5GS, MCX), if any.

Below is the list of use cases identified in the scope of this study:

-	Call forwarding for Ad hoc Group Call

-	Merging of two Ad hoc Group calls (multi-train voice communications) by the Train Controller or by the Driver (e.g., banking voice use case)

-	Merging of two Ad hoc Group Emergency alerts and associated REC (Railway Emergency Communications) voice communications by the Train Controller

-	The real-time automatic translation of languages uses cases in the context of:

-	MC interconnection between MC users of two MC systems

-	MC migrated user in a Partner MC system

-	Presence: the capability of the FRMCS system to provide the status of a user to interested users currently logged in the FRMCS system

-	Ability of an authorized user to indicate temporary unavailability in an on-going Ad hoc Group call (for operational reasons e.g. to initiate in a private mode, a Controller to Driver communication). Other participants of the Ad hoc Group call need to be notified of start and stop of the temporary unavailability of this authorized user.

# 5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TR22.989 | Specification to be updated (refine existing use cases, define new uses, and derive new potential requirements to stage 1 TS e.g. 5GS, MCX). | TSG#107(March 25) | None |

# 6	Work item Rapporteur(s)

Nikolopoulou, Vassiliki, UIC, nikolopoulou@uic.org

# 7	Work item leadership

SA1

# 8	Aspects that involve other WGs

# 9	Supporting Individual Members

Supporting IM name

UIC

Nokia

Nokia Shanghai Bell

Ericsson

Huawei

Kontron Transportation France