3GPP TSG-SA1 Meeting #107	S1-242317

Maastricht, NL, 19-23 August 2024	(revision of S1-242049)

# Source:	China Telecom

# Title:	New WID on efficient signaling for N3GPP access

# Document for:	Approval

# Agenda Item:	4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: New WID on efficient signaling for N3GPP access

Acronym: ES_N3GPP

Unique identifier:	XX

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
| X | Normative – Stage 1 |
|  | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A | N/A | N/A | N/A |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

3	Justification

Network operators with both fixed broadband and mobile communication facilities aim to leverage fixed broadband connections to enhance mobile communication service delivery. In TS 23.501, the support of the N3GPP access requires both the 5G network and the UE to support N3IWF.

However, current and planned commercial deployments of N3IWF are very limited for two primary reasons:

(a) 5G chipsets from major vendors do not support N3IWF;

(b) operators are reluctant to deploy N3IWF.

For operators/chipsets not supporting N3IWF, N3GPP access cannot be utilized. Therefore, another option is needed to enable operators/chipsets without N3IWF support to utilize N3GPP access.

The requirements in TS 22.261 clause 6.3 (multiple access technologies) focus on optimizing user data transmission when UE have multiple accesses. It is missing to allow multi-access capable UEs to transmit signaling only over 3GPP access while utilizing N3GPP access for data traffic.

4	Objective

Stage 1 specifications to be updated to support the transmission of data traffic over N3GPP access and the associated signaling over 3GPP access for UEs with multiple accesses to 5G. This new requirement enables operators/chipsets without N3IWF support to utilize N3GPP access.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 22.261 | Update the stage 1 specifications to support the transmission of data traffic over N3GPP access and the associated signaling over 3GPP access for UEs with multiple accesses to 5G. | TSG#105 (Sept 2024) |  |

6	Work item Rapporteur(s)

Yinglin Chen, China Telecom, chenyl37@chinatelecom.cn

7	Work item leadership

SA1

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Telecom