3GPP TSG SA WG 1 Meeting #107 	S1-242319

Maastricht, The Netherlands, 19-23 August 2024	(revision of S1-242074)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0795 | rev | 1 | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New requirements for satellite access network sharing via Indirect Network Sharing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, China Unicom, LG Uplus |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NetShare_SAT |  |  |  |  |  | Date: |  |  | 2024-08-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The satellite network and the terrestrial network can effectively complement each other when they can’t offer available services in their respective areas. The sharing of satellite based NG-RAN via Indirect Network Sharing enables new business models and cooperation between terrestrial network operators and satellite MNOs (as defined in TR28.844). However, compared with the terrestrial network, the coverage of the satellite NG-RAN is managed in larger granularity of geographic region so that one cell or tracking area will cover the area expected or allowed to be shared and the areas not expected or allowed to be shared. Thus, this CR intends to address such network access control issue to facilitate the deployment of Indirect Network Sharing in different SLAs and business models. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR clarifies the definition of Shared NG-RAN in 3.1, and introduces new requirements to support the use case of satellite based NG-RAN sharing via Indirect Network Sharing in 6.21. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are potential issues when MNOs and satellite network operators cooperate by sharing satellite based NG-RAN via Indirect Network Sharing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 6.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0 S1-242074 – original version Rev1 S1-242319 – reword the NOTE, isolate new requirements from the legacy requirements. |  |  |  |  |  |  |  |  |

* * * First Change * * * *

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [29] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [29].

Shared NG-RAN: as defined in 3GPP TS 22.101 [6].

NOTE X: Shared NG-RAN can be a shared satellite NG-RAN.

* * * Second Change * * * *

6.21	NG-RAN Sharing

6.21.1	Description

The increased density of access nodes needed to meet future performance objectives poses considerable challenges in deployment and acquiring spectrum and antenna locations. RAN sharing is seen as a technical solution to these issues.

In RAN Sharing operations, NG-RAN resources can be used by multiple network operators. For the NG-RAN resources deployed on satellites, they can be used by both satellite mobile network operators and terrestrial network operators together by proper network operations. Indirect Network Sharing is one of the possible sharing methods.

During NG-RAN sharing, the security and privacy of shared networks, non-shared networks, and subscribers need to be maintained without negative effects. Especially in the case of Indirect Network Sharing, where the involvement of the core network of the hosting operator e.g. for signalling exchange between the users and the core network of the participating operator could cause exposure of the subscriber’s information to the hosting network, an extra scrutiny of the security mechanism is expected to avoid sharing the information that is not needed for the Indirect Network Sharing operation (e.g. network topology) and protect the information that is needed for the Indirect Network Sharing operation between the hosting operator and the participating operator.

6.21.2	Requirements

6.21.2.1	General

Requirements related to NG-RAN sharing are described in 3GPP TS 22.101 [6], mainly in clause 28.2.

A 5G satellite access network shall support NG-RAN sharing.

6.21.2.2	Indirect network sharing

The 5G system shall be able to support Indirect Network Sharing between the Shared NG-RAN and one or more Participating NG-RAN Operators’ core networks, by means of the connection being routed through the Hosting NG-RAN Operator’s core network.

NOTE 1:	Requirements of Indirect Network Sharing assume no impact on UE.

NOTE 2: 	For more information on Indirect Network Sharing see Annex I.

The following requirements apply to Indirect Network Sharing:

Indirect Network Sharing shall be transparent to the user.

NOTE 3:	This requirement is aligned with the existing requirement in 3GPP TS 22.101 [6] clause 4.9.

The following existing service requirements related to network sharing in 3GPP TS 22.101 [6] apply:

-	clause 4.2.1,

-	clause 28.2.3, and

-	clause 28.2.5.

Subject to the agreement between the hosting and participating operator, the 5G system shall support a means to

enable a UE of the Participating NG-RAN Operator to:

-	access their subscribed PLMN services when accessing a Shared NG-RAN, and/or,

-	obtain its subscribed services, including Hosted Services, of participating operator via a Shared NG-RAN.

Based on operator policy, the 5G system shall support a mechanism to enable an authorized UE with a subscription to a Participating Operator to select and access a Shared NG-RAN.

Based on operator policy, the 5G system shall support access control for an authorized UE accessing a Shared NG-RAN and be able to apply differentiated access control for different Shared NG-RANs when more than one Shared NG-RAN are available for the Participating Operator to choose from.

Based on operator policy, the 5G network shall be able to apply different access control for a UE connecting to Participating Operator’s core network when the UE is located in different geographic areas of the coverage of one shared satellite NG-RAN

Based on operator policy, the 5G system shall enable the Participating Operator to provide steering information in order to assist a UE with access network selection amongst the Hosting Operator’s available Shared RAN(s).

Subject to the agreement of operators, the 5G network shall enable the Participating Operator to allow a UE accesing a suitable RAN when this UE is located in the same coverage of one shared satellite NG-RAN and non-Shared RANs.

The 5G system shall support service continuity for UEs that are moving between different Shared NG-RANs and/or between a Shared NG-RAN and a non-Shared NG-RAN network.

The 5G system shall be able to provide a UE accessing a Shared NG-RAN network with positioning service in compliance with regulatory requirements.

Subject to regulatory requirements and mutual agreement between the participating operators and the hosting operator, the requirements to support regulatory services, e.g., PWS or emergency calls apply to Indirect Network Sharing.

In case of Indirect Network Sharing and subject to agreement between operators the 5G system shall enable the Shared NG-RAN of a hosting operator to provide services for inbound roaming users.

The 5G core network shall be able to support collection of charging information associated with a UE accessing a Shared NG-RAN using Indirect Network Sharing, which refers to the resource usage of hosting operator’s core network.

NOTE4: When Shared NG-RAN is a shared satellite NG-RAN, the charging information can include the information specific to the shared satellite NG-RAN, e.g. the location of UE, involved satellites.

* * * End of Changes * * * *