3GPP TSG SA WG 1 Meeting #107 	S1-242349

Maastricht, The Netherlands, 19-23 August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.101 | CR | 0597 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification and alignment on NTN reception of TV transport services via MBS broadcast without authentication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Novamint, SES, EBU, Thales, ESA, Inmarsat, Viasat, EchoStar, JSAT, TNO, Airbus, Eutelsat Group |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18; 5MBS_ph2 |  |  |  |  |  | Date: |  |  | 2024-08-21 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some network operators supporting satellite access are interested in deploying broadcast services over satellite access. In Release 18, SA2 defined a feature to enable reception of TV transport services via MBS broadcast without authentication. This feature in SA2 is equally applicable to terrestrial access network and satellite access network. This contribution proposes alignment and clarification to the related SA1 service requirement to match the solution in SA2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification is added to the requirement on reception of TV transport services via MBS broadcast without authentication: - applies to NTN and TN; - applies to eMBMS (LTE) and MBS (5G); and - applies to network operators other than MNOs. Mirror CR of the CR proposed for Release 18 as this provides alignment with the first release in which reception of TV transport services via MBS broadcast without authentication was introduced in SA2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will remain ambiguous if the requirement on reception of TV transport services via MBS broadcast without authentication applies only to terrestrial access networks or also to satellite access networks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 32.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 32.2.1	General requirements

The 3GPP network shall be able to support on-demand network capacity assignment for TV services.

The 3GPP system shall allow the MNO to implement a network supporting downlink only broadcasting based on a set of EPS functionalities and entities which are required to offer linear TV services.

The 3GPP network shall be able to support on-demand network broadcast service area coverage management for TV transport services.

The 3GPP network shall support content delivery up to UHD resolution.

The 3GPP network shall allow the MNO to authenticate a subscriber before providing TV transport services via unicast.

The 3GPP network shall allow the MNO to authenticate a subscriber before providing TV transport services via eMBMS broadcast.

The 3GPP network shall enable a network operator to allow a UE to receive TV transport services via eMBMS/MBS broadcast without authentication.

NOTE: This requirement applies to both terrestrial access networks and satellite access networks.

The 3GPP network shall be able to provide FTA TV services receivable by UEs subscribed to the MNO and roaming UEs of other MNOs. A UE supporting enhanced TV transport services and in limited service state shall be able to receive eMBMS broadcast TV transport services.

The 3GPP network shall provide the ability for an MNO to meet regulatory requirements for privacy and non-identification of a receiving user by entities outside of the MNO.

NOTE: The MNO is responsible for maintaining the privacy of personal usage data collected as a result of providing access to the TV services.

The 3GPP network shall provide mechanisms to restrict the reception of some or all Subscribed TV services to groups of subscribers (e.g. based on the recipients of the services are subscribers of the MNO, roaming subscribers of other MNOs, or not subscribed to any MNO).

The quality of experience of the TV service shall be independent from the size of the concurrent audience. While receiving TV services in normal service state, subscribers of the MNO and roaming subscribers shall be able to access other subscribed services such as voice, data or SMS.

The 3GPP network shall support combinations of SD, FHD and UHD resolution TV transport services.

The 3GPP network shall allow the MNO to provide a TV service in a resource efficient manner over a large area up to the size of a country.

The 3GPP network shall enable the quality of experience of the TV service to be ensured throughout the coverage area defined for the TV service.

* * * End of Changes * * * *