3GPP TSG-SA WG1 Meeting #107 	S1-242061

Maastricht, The Netherlands, 19-23 August 2024	(revision of S1-242061)

Title:	Use Case on GEO assisted fast network entry to a multi-orbit Satellite Access System

Agenda Item:	7.3

Source:	SES, Novamint, Thales, TNO, JSAT

Contact:	joel.grotz@ses.com, stavros.domouchtsidis@ses.com, abbas.karaki@ses.com

Abstract: This document proposes a use case on GEO assisted network entry to a multi-orbit Satellite Access System above 10GHz and potential requirements for TR22.887 v0.1.0 (FS_5GSA_ph4)

## x.1	Use case on GEO assisted fast network entry to a multi-orbit Satellite Access System

### x.1.1	Description

Cell search and camping can be a tedious process for a UE served by NTN payloads in NGSO when operating in frequencies above 10GHz.

The link budget in the mentioned operating frequencies, require highly directional antennas both on the NTN payload and on the VSAT UE. Therefore, the UE after being switched ON, must scan not only the frequency raster but also the sky to discover possible NTN cells to camp to. However, scanning the sky is impeded by the high orbital speed in NGSO, as well as the steering delay of the VSAT antenna. More specifically, the orbital speed in LEO can be more than 7.5km/s while the steering of a beam is in the order of seconds for a mechanical steering antenna or milliseconds for an electronic one.

In legacy non-3GPP SANs, VSATs are either installed by technicians or perform when switched ON an extensive sky scan which can take many minutes until a connection to a satellite is established.

Within this use case, a multi-orbit Satellite Access System consisting of GSO and NGSO NTN payloads is envisioned to enable fast connection establishment for VSATs by exploiting the fixed position, relevant to earth, of GEO satellites.

The fixed position and number of GEO satellites, enables the pre-provision of their ephemeris since it requires significantly less storage than an NGSO constellation via e.g. a USIM. Alternatively, if the latter is not possible scanning the GEO arc is much more efficient than searching for serving satellites in NGSO because of their fixed position.

This use case describes a satellite access system where it has two configured cell types. Low capacity GSO cells for fast network entry, and high capacity NGSO cells for serving user traffic once the connection is established. These two types of cells may be served by different operators.

### x.1.2	Pre-conditions

The 5G satellite operator has a network served by NTN payloads in GSO and NGSO.

The UE supports multi-orbit access (GSO and NGSO) provided by the same 5G satellite operator.

The UE supports using ephemeris not only for Uplink timing and frequency synchronisation but also for steering its antenna for Downlink synchronisation.

The 5G satellite network has the following policies and configuration:

- Cells with low capacity to allow fast network entry via GEO satellite link.

- Cells in GSO that broadcast the ephemeris of overlapping cells served by NGSO satellites.

- Cell with high capacity for increased data traffic via LEO/MEO satellite link.

### x.1.3	Service Flows

According to this use case:

- The VSAT UE is turned ON and it acquires a valid GNSS position.

- If pre-provisioned with relevant ephemeris data it points its beam towards the GEO satellite that serves its location, or else it scans the GSO arc for the serving GEO satellite.

- The UE performs the process to identify a suitable physical cell ID and establish a connection to the NTN cell, as already specified.

- Once camped/connected to a cell served by a GSO NTN payload, the UE receives and decodes SIB19 that shall include ephemeris data of the satellites in NGSO that serve its location.

- The UE (in Idle Mode)/ network (in Connected Mode) triggers mobility towards a cell served by NGSO payload which offers significantly more capacity and lower latency.

### x.1.4	Post-conditions

The VSAT UE connects in matter of seconds to the satellite network, rather than minutes. After the initial connection establishment to the GEO cell the handover to the LEO/MEO cell is seamless as the VSAT uses the acquired ephemeris to quickly steer its beam towards the LEO/MEO serving satellite.  The same mechanism can be applied to a handover from a GEO cell to a roaming partner’s NGSO cell.

### x.1.5	Existing features partly or fully covering the use case functionality

Requirements in 22.261 Rel19 that partly cover the described use case are:

- A 5G system providing service with satellite access shall be able to support GEO based satellite access with up to 285 ms end-to-end latency.

NOTE 1:	 5 ms network latency is assumed and added to satellite one-way delay.

- A 5G system providing service with satellite access shall be able to support MEO based satellite access with up to 95 ms end-to-end latency.

NOTE 2:	 5 ms network latency is assumed and added to satellite one-way delay.

- A 5G system providing service with satellite access shall be able to support LEO based satellite access with up to 35 ms end-to-end latency.

NOTE 3:	 5 ms network latency is assumed and added to satellite one-way delay.

### x.1.6	Potential New Requirements needed to support the use case

[PR x.1.6-001] A 5G system with multi-orbit satellite access shall be able to support idle mode via a GSO space segment layer and connected mode via a lower altitude NGSO space segment layer.

[PR x.1.6-002] A 5G system shall support mobility between GSO and NGSO satellites serving different PLMNs.