3GPP TSG-SA WG1 Meeting #107 	 S1-242370

Maastricht, The Netherlands, 19-23 August 2024	(revision of S1-242290)

Title:	Use case on GEO downlink and NGSO uplink

Agenda Item:	7.3

Source:	Viasat Inc., Inmarsat, Novamint, SES

Contact:		Nandan Das Nandan.das@viasat.com
Jim Petranovich jim.petranovich@viasat.com

Abstract:  In this note we present multiple use cases for multi-orbit satellite systems.  We consider satellites in geostationary, i.e. geosynchronous equatorial plane, orbits (GEO) orbits as well as non-geostationary (NGSO) orbits.  NGSO orbits could include medium (MEO), low (LEO), and highly elliptical (HEO) Earth orbits.  These use cases are envisaged for a system that has multiple satellites across different orbits, potentially with different characteristics and potentially using different frequency bands, coexisting in one network.

# Introduction:

Different satellite orbits bring different characteristics that prove advantageous in different applications.  E.g. GEO orbits, where the satellite has a field of view (FOV) of about 1/3 of the earth’s surface area is great for coverage and for flexibly moving bandwidth and capacity from areas of low demand to areas of high demand in real time. LEO orbits with an altitude of a few hundred km are useful for latency sensitive applications and in some cases can provide line-of-sight (LoS) advantages, link budget advantages (this may or may not be the case depending on the satellite size and orbit altitude) and in some cases polar or high latitude coverage depending on the constellation configuration.  HEOs are very effective at covering latitudes close to the polar regions, and typically cover polar regions of the Earth very effectively.   A system that combines some or all these orbits in a multi-orbit satellite network will be able to take advantage of these characteristics to cover a wide range of use cases that would be unlikely to be satisfied using only one orbit.

In this paper, we describe one of the multiple such use cases.

* * * First Change * * *

## 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

satellite access type in the context of this study, it refers to a satellite access with a specific set of characteristics (including orbit type, orbital characteristics, satellite capabilities, IoT or NR, …)

* * Next Change * * *

---------- Use Case template ----------

## 2.3	Use case on GEO downlink with LEO uplink

### 2.3.1	Description

There are many applications with asymmetrical data usage with high bandwidth forward link (downlink) requirements to a user or a group of users, with minimal return link (uplink) speed requirements.  Examples of this include unicast streaming video transmission to a user device, or multicast streaming video transmission to a large group of user devices.  The return link might be utilized to support reliable data communication and/or video content selection.  The low bandwidth return link  can be accommodated by a low bandwidth uplink link.  This link may be over a LEO satellite.  The use case envisages a GEO satellite (e.g. a Ka-band GEO satellite) co-existing in a network with a LEO satellite (e.g. L-band LEO).  This also reduces the cost of a UE, since the narrowband uplink RF path is typically much cheaper than a broadband uplink RF path.  Similar principles might apply in reverse with a low rate forward link and high rate return link.

### 2.3.2	Pre-conditions

- The network consists of two separate satellite systems with different satellite access types

- The network supports downlink with one satellite access type and uplink with a different satellite access type

- The network is able to determine the location of a user to a sufficient granularity in order to be able to activate or redirect the user to the correct NGSO coverage beam or cell

### 2.3.3	Service Flows

- The network directs all downlink data destined for the user over the first satellite access network

- The terminal directs all uplink data destined for the network over the second satellite access network

### 2.3.4	Post-conditions

The UE is able to receive maintain a connection using these multiple satellite networks

### 2.3.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 2.3.6	Potential New Requirements needed to support the use case

[PR 5.x.6.001] A 5G system with multi orbit satellite access shall be able to support downlink connectivity via one satellite access type and uplink connectivity via a different satellite access type.