3GPP TSG-SA WG1 Meeting #107 	 S1-242375

Maastricht, The Netherlands, 19-23 August 2024	(revision of S1-242369)

Title:	Use case on using GEO for sleep/power management

Agenda Item:	7.3

Source:	Viasat Inc., Inmarsat, Novamint, SES

Contact:		Nandan Das Nandan.das@viasat.com
Jim Petranovich jim.petranovich@viasat.com

Abstract:  In this note we present multiple use cases for multi-orbit satellite systems.  We consider satellites in geostationary, i.e. geosynchronous equatorial plane, orbits (GEO) orbits as well as non-geostationary (NGSO) orbits.  NGSO orbits could include medium (MEO), low (LEO), and highly elliptical (HEO) Earth orbits.  These use cases are envisaged for a system that has multiple satellites across different orbits, potentially with different characteristics and potentially using different frequency bands, coexisting in one network.

# Introduction:

Different satellite orbits bring different characteristics that prove advantageous in different applications.  E.g. GEO orbits, where the satellite has a field of view (FOV) of about 1/3 of the earth’s surface area is great for coverage and for flexibly moving bandwidth and capacity from areas of low demand to areas of high demand in real time. LEO orbits with an altitude of a few hundred km are useful for latency sensitive applications and in some cases can provide line-of-sight (LoS) advantages, link budget advantages (this may or may not be the case depending on the satellite size and orbit altitude) and in some cases polar or high latitude coverage depending on the constellation configuration.  HEOs are very effective at covering latitudes close to the polar regions, and typically cover polar regions of the Earth very effectively.   A system that combines some or all these orbits in a multi-orbit satellite network will be able to take advantage of these characteristics to cover a wide range of use cases that would be unlikely to be satisfied using only one orbit.

In this paper, we describe one of the multiple such use cases.

---------- Use Case template ----------

## 2.5	Use case on using GEO for sleep/power management

### 2.5.1	Description

Many UEs are power limited and need to conserve power to extend battery life.  In some cases, UEs receive (or have to send) packets very infrequently.  In these cases, it is wasteful of the UE to continuously maintain and establish sync with a NGSO satellite that needs to be tracked through the sky.  A GEO satellite can be very useful for these UEs.  The UE would only try to connect to a NGSO satellite if the GEO satellite told it to do so.  The GEO satellite would only tell the UEs to connect to the NGSO if the network informed the GEO that packets were waiting for the UE.  In cases where the UE needs to send a packet, it would either send it directly to the GEO, or request (via the GEO) resources from the network on a NGSO satellite.

### 2.5.2	Pre-conditions

- The network supports both GEO and NGSO satellites

- The network supports handover between GEO satellite(s) and NGSO satellite(s)

- A network plan that deconflicts frequency allocations between the GEO and NGSO satellites.

- The network is able to determine the location of a user to a sufficient granularity in order to be able to activate or redirect the user to the correct NGSO coverage beam or cell

- The different satellite orbits employed by the network may be supported by the different satellite access network types

- The network should be aware of when to tell the UE to move to a NGSO satellite connection.

### 2.5.3	Service Flows

- UE establishes a connection with a GEO satellite

- UE enters sleep mode

- UE wakes ups periodically and re-establishes connection with a GEO

- GEO informs UE of whether it needs to wake up

- If there is no need to wake up, UE goes back to sleep

- If there is a need to establish a connection, UE exists wakes up, and establishes a connection with NGSO instead of the GEO

### 2.5.4	Post-conditions

UE is in appropriate sleep mode on GEO or active communications mode on NGSO

### 2.5.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 2.5.6	Potential New Requirements needed to support the use case

[PR 5.x.6.001] Subject to regulatory requirement and operator’s policy, a 5G system with multi orbit satellite access shall be able to support a mechanism for the network to inform a UE when it can be active and on which satellite system.

[PR 5.x.6.002] Subject to regulatory requirement and operator’s policy, a 5G system with multi orbit satellite access shall be able to support a mechanism for the GEO satellite access to inform specific UE or group of UEs on whether there is available data for it/them.