3GPP TSG-SA WG1 Meeting #107	S1-242543

Maastricht , The Netherlands, 19-23 August 2024	(revision of S1-24517)

Title:	Energy efficient, carbon aware, content download

Agenda Item:	7.2

Source:	TNO, KPN, China Mobile, ZTE, Telefonica

Contact:	Toon Norp

Abstract: the downloading of attachments (e.g., music, videos, or files) exchanged via instant messaging or email is scheduled during time periods when both a green energy mix and favourable signal conditions occur at the serving site in order to minimize EC and CO2e.

Background

In this meeting 3 discussion papers are issued, that serve in the background as support for this use case:

- The (future) energy market (See S1-242109).

The discussion paper shows how dynamic the future energy market will become, that lowering peaks in supply and demand are of importance, that day, hour and second trading is important to maintain grid stability.

Acknowledgements

This work has been supported by the European Union’s HORIZON-JU-SNS-2023 research and innovation programme under EXIGENCE project (grant 10113920). For more info see www.projectexigence.eu.

---------- First change ----------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.882: "Study on Energy Efficiency as a service criteria".

[3]	3GPP TS 22.261: "Service requirements for the 5G system".

[x]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[y]	3GPP TR 23.700-66: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[z]	E. D. Fitkov-Norris and A. Khanifar, "Dynamic pricing in mobile communication systems," First International Conference on 3G Mobile Communication Technologies, London, UK, 2000, pp. 416-420, doi: 10.1049/cp:20000083.

---------- Second change ----------

## 5.x	Use case on Green social media & email content download

### 5.x.1	Description

With 7.9 billion users accessing instant messaging and email services worldwide, with increasing attachment sizes (e.g., WhatsApp’s expanded message size from 100MB to 2GB) and with the volume of data consumed expected to grow exponentially, it is essential to address the challenges which this imposes for achieving carbon neutrality, in support of a global green digital future vision.

This use case aims to reduce the carbon impact of instant messaging and email services by postponing the download of attachments (i.e., the bulk of the data) to a moment in time when both the energy mix is and/or the radio signal conditions are favourable, considering that  radio signal conditions have a major impact on the EC needed for RF communications.

When sharing media via instant messaging or email apps (such as WhatsApp, Facebook Communicator, Outlook, or Gmail), users can subscribe through their apps to only download shared music, videos, or files specifically during periods of a high renewable energy ratio and favourable radio signal conditions at the serving site. In addition to the satisfaction of contributing to reducing the carbon footprint, this use case could provide dynamic CO2 credit incentives (or “environmental points”) to the end-user. These credits can be applicable, for example, toward service discounts.

Similarly, other non-time critical data transfers, such as firmware downloads, uploads of social media or email attachments (e.g., recorded videos) and downloads from OTT video providers for offline viewing could be scheduled using the same constraints.

### 5.x.2	Pre-conditions

A Service Provider (e.g., an email or IM provider) provides an option in its App for configuring green transfer of non-critical data and the End-User has selected this option in that App.

A PLMN is able to compute the renewable energy ratio at the applicable serving cell sites.

### 5.x.3	Service flows

- A message with attachment(s) is sent to the End-User through social media or email.

- The message body is instantly delivered by the Mobile Network to the End-User’s UE, but its attachments are scheduled for (delayed) green delivery instead.

- The Mobile Network regularly evaluates the renewable energy ratio at the serving cell site, as well as the energy consumption parameters relating to radio signal conditions experienced at the UE.

- When the renewable energy ratio at the serving site is sufficiently high and/or the energy consumption parameters relating to radio signal conditions are favourable, the attachments are also delivered to the UE.

- The MNO provides incentives to the End-User that has (delayed) green download enabled. These incentives may be based on the EC and/or CO2e gains obtained through (delayed) green delivery.

### 5.x.4	Post-conditions

This use case has the potential to enhance the creation of new business opportunities by MNOs and to provide incentives for users to reduce their energy consumption and carbon footprint. Renewable energy (including energy self-produced by MNOs) is utilized more efficiently which is instrumental in achieving carbon neutrality goals and energy efficiency is improved by scheduling transmissions when signal conditions are favourable.

### 5.x.5	Existing feature partly or fully covering use case functionality

Providing (monetary) incentives at cell level (using cell broadcast channels) has been proposed by implementing dynamic tariff offers in 3G networks [z] to encourage users to increase traffic in cells with low utilization (e.g., 3GPP TS 23.041 [x]).

Some of the procedures and network functions proposed in 3GPP TR 23.700-66 [y] can be used to realize part of the service flow described in Section 5. x.3 above.

### 5.x.6	Potential new requirements needed to support the use case

[PR.5.x.6-1] The 5G system shall be able to expose information to an Application Function in the network on the (expected) energy related characteristics applicable to the serving site/network.

NOTE:	This information can be used by applications in the Application Function to schedule non time-critical data transfer to or from the UE.