3GPP TSG-SA WG1 Meeting #107 	S1-242548

Maastricht, The Netherlands, 19-23 August 2024     (revision of S1-242540,2490,2327)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.261 | CR | 0806 | rev | 4 | Current version: | 19.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new service requirements for exposing VN information and capability to authorized third party |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | VNExposure |  |  |  |  |  | Date: |  |  | 2024-08-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | (1) The authorized third-party industry customers need to obtain information reporting at the granularity of 5G VN from the 5G system, so as to make necessary business adjustments and troubleshoot based on these information.In TS 22.261 6.26.2.3, the definition of "for a specific network" and the scope of "information" in "configure and receive information regarding the achieved performance for a specific network" are unclear and needs more clarification. (2) Industry customers require adjustments to 5G LAN groups based on business needs, which may involve changes in VN network configurations. This necessitates manual intervention by operators, resulting in a lengthy business adjustment cycle and leads to high communication costs between the enterprise AF and NEFs. To address these issues, operators should just make the subscription and exposure VN management service, which is closely tied to business requirements, to authorized third-party industry customers. The industry customers could perform timely operation within the subscribed 5G LAN. This will enable a "one-time service activation, self-adjustment by customers" model, improving the efficiency of 5G VN service adjustments. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update and add service requirements for VN information and capabilities to authorized third party. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Authorized third-party industry customers are unable to obtain sufficient information to support their necessary business adjustments and troubleshooting. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.26.2.3, 6.26.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## *****************   Start of First Change ***********************

6.26.2.3	Creation and management

The 5G network shall enable the network operator to scale up/down a 5G LAN-VN, e.g. the coverage, capacity for efficient consumption of network resources.

The 5G network shall enable the network operator to create, manage, and remove 5G LAN-VN including their related functionality (subscription data, routing and addressing functionality).

The 5G network shall enable the network operator to add one or more authorized UEs to an existing 5G LAN-VN.

NOTE 1:	A UE needs to be authorized by the MNO to use 5G LAN-type service before it can be added to any 5G LAN-VN.

NOTE 2: 	some use cases will require user permission for a UE to be added to a 5G LAN-VN.

The 5G system shall enable the network operator to add an authorized UE to multiple independent 5G LAN-VNs.

The 5G network shall enable the network operator to remove one or more UEs from an existing 5G LAN-VN.

NOTE 3:	Removing a UE from a 5G LAN-VN does not have impact on other 5G LAN-VNs that the UE is a member of.

The 5G system shall enable the network operator to configure a 5G LAN-VN that is available only within a geographical area.

Based on MNO policy, the 5G network shall provide suitable means to allow an authorised third party to

-	monitor changes in QoS policy that pertains to LAN-VN performance;

-	configure and receive information regarding the achieved performance for a specific UE;

-	configure and receive information regarding the achieved performance for a specific network (e.g, number of active UEs, UE status (connection status) change events in 5G LAN-VN);

-	receive notification of changes in specific configuration aspects of the UE in the VN (e.g., changes in group membership information.)

## *****************  Start of Next Change ***********************

6.26.2.9	Service exposure

Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third-party to create/remove a 5G LAN-VN.

Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third-party to manage a 5G LAN-VN dedicated for the usage by the trusted third-party, including the address allocation.

Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third-party to authorize/deauthorize UEs to access a specific 5G LAN-VN managed by the trusted third-party.

Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third-party to add/remove an authorized UE to/from a specific 5G LAN-VN managed by the trusted third-party.

Based on MNO policy, the 5G network shall provide suitable APIs to allow a trusted third-party to dynamically adjust partial configuration (i.e. create, manage, and remove) of the 5G LANs for the authorized 5G LAN-VN capable UEs.

NOTE:	5G LAN-VN capable UEs means the operator authorizes UEs to use 5G LAN-type service. The joining to the 5G LAN-VN group and the configuration to the 5G LAN-VN can be dynamically configured by an trusted third party.

## *****************   End of Changes ***********************