3GPP TSG-SA WG1 Meeting #108 	S1-244024

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Title:	Network assisted 3D-mobility

Agenda Item:	8.1.2 Integrated Sensing and Communication

Source:	Ericsson

Contact:	Daniel Lönnblad (Daniel.lonnblad at ericsson.com)

Abstract: Use cases to introduce Network Assisted 3D Mobility in TR 22.870.

***********   Start  change  *********

# 6	Integrated Sensing and Communication

Editor's Note: "Integrated Sensing and Communication" facilitates new applications and services that require sensing capabilities. It offers wide area multi-dimensional sensing that provides spatial information about unconnected objects as well as connected devices and their movements and surroundings.

## 6.1	Use Case 1

## 6.x	Use cases on Network Assisted 3D Mobility

### 6.x.1	Description

Network Assisted 3D Mobility refers to the use case where the mobility of persons or connected vehicles such as cars or drones is assisted by the network. The assistance can be in the form of help of e.g., sensing, navigating and positioning of humans or vehicles. Examples can be found in Autonomous Drone Transport, Smart Intersections, and Assisted Vehicles.

Figure 1: Network-Assisted 3D Mobility Example Scenario

Autonomous Drone Transport

In autonomous drone transport, flying drones are carrying goods in urban areas. The drones are equipped with sensors (camera, GPS, etc.) and a processor. Through the onboard 6G device they get a reliable communication link and that allows them to be positioned by the network. Over the communication channel the drone can receive timely information about nearby flight paths and activities and get recommended actions for fast and high energy efficient paths. The drone can also receive a corrected position as well as map data from the network. It can also, depending on need, offload tasks such as image processing from the camera feed and share the resulting data on the surrounding environment with the network. The users and deliverers of the drone transport service can count on a reliable, efficient, just-in-time delivery that can be tracked at any time, and people in the streets can trust that no collisions will occur by the drones passing over their heads.

Smart Intersections

Mobile communication infrastructure featuring sensing capabilities would enable a wide-coverage and well-connected radar mesh. Such network can be seen as a set of wide area sensors, that will add value to its users at nearly no extra cost. Safety and efficiency of urban traffic can be improved by means of real-time spatial and temporal sensing of critical roads and intersections to support the vehicle when its onboard sensors do not have line of sight. For example, even before a vehicle gets to a junction, it will be made aware of the conditions around the junction (such vulnerable pedestrians or other vehicles coming around the corner) so that the vehicle can take the right precautions to advert collision or accidents. Different from monitoring with video cameras, such sensing approach would allow perception even in complete darkness, or in adverse weather conditions like rain or fog (with certain performance limitations). Also, privacy concerns for citizens would be reduced as no actual video or picture would be used. Finally, sharing the sensed real-time data with traffic control centres can help to improve traffic flow management in smart cities.

Assisted Vehicles

Vehicles today have many onboard sensors to support applications such as cruise control, pedestrian detection etc. In the 6G era, the enabling of advanced automotive features such as autonomous driving and autonomous coordinated manoeuvring is envisioned through leveraging the wide area sensors from the network, in addition to on-board vehicle sensors. Autonomous driving would allow vehicles to be navigated through challenging traffic conditions and terrains without the need for human interaction.

In addition, autonomous coordinated manoeuvring feature would allow multiple vehicles to autonomously navigate through roads and highways in a coordinated fashion to ease traffic congestion and improve the traffic flow. These advanced automotive features would require a comprehensive detailed knowledge of the environment surrounding the vehicles as well as high precision localization and positioning. This would be enabled by the fusion of the wide-area sensor information provided by the network, vehicle on-board sensors and/or even sensors embedded in the transport infrastructure.

Key Value impact analysis

Energy resources: Less energy/fuel would be consumed due to driving via optimized routes, however the additional sensing equipment will require energy therefore energy consumption needs to be considered while developing the required solutions.

Material resources: Need for waste treatment is expected to reduce due to decreased number of accidents. In addition, if the application of sensing technology leads to producing less or smaller vehicles, less material will be needed.

Emissions: Improving the traffic flow and assisted driving should lead to reduction of emissions.

Health: Reduced transport-related accidents should lead to enhanced safety and well-being,

Trustworthiness: Preserved privacy is enabled through network-assisted mobility compared to video-based solutions, however there are still potential risks for privacy intrusion associated to localization data, which can be mitigated with the help of privacy preserving technologies, and an increased risk of cyber-attacks or unintended errors which may lead to accidents.

Work & income: With self-driving vehicles the job opportunities for drivers are reduced. Potential improved efficiency from freeing resources e.g. by with providing the ability of working during transport/driving.

Infrastructure: Safer road transport systems is a result of the use-case and the access to reliable transports increases. However, building network infrastructure to meet the requirements for high reliability of services can be costly.

### 6.x.2	Pre-conditions

-	Vehicles and drones using the service are equipped with UEs.

-	The service is offered in a well-defined area/space (service space).

### 6.x.3	Service Flows

In this use case, illustrated in Figure 1, vehicles (cars, AGVs, drones, etc.) are relying on the network nodes and devices for localization of connected and unconnected objects and for determination of their properties such as size and velocity, contextual info, trajectories etc. Vehicles have a reliable connection to the network, with services available in a well-defined service area. Networks measure the physical environment in traffic scenarios and analyse to detect objects, and aggregate data of device positions and from this large data set extract information to relay to vehicles. Position data shared with vehicles can include vulnerable road users (VRUs), such as pedestrians and cyclists.

This can be done on several levels; networks can provide raw or processed environment and position data, or navigational support ranging from collecting and sharing of data, over navigation assistance, to full operation, leveraging on beyond-communication capabilities. The network can thereby support vehicles with different levels of autonomy and different modes of operation and enable smart transport in urban areas.

Furthermore, there is a possibility to use the physical environment understanding around nodes and devices to improve communication to vehicles by tailoring beams, avoid blockers, etc.

Some examples of the services provided by networks are: network assistance info (warnings, trajectories, object locations), network assistance map (digital 3D map data), network navigation (route recommendations), full operation (remote control of vehicles) and context-aware communication (path selection, scheduling).

### 6.x.4	Post-conditions

Not applicable.

### 6.x.5	Existing features partly or fully covering the use case functionality

-	Network positioning available in 5G standard as in TS 22.261.

-	Sensing requirements for 5G as specified in TS 22.137. Dependent on what object to give assistance to the requirement on the sensing service will vary. For the drone/vehicle use case described above sensing service category 2 or 3 specified in Table 6.2-1 performance requirements of 5G Wireless sensing TS 22.137 would be required.

Table 6.2-1:	Performance requirements for 5G Wireless sensing

| Scenario | Sensing service category | Confidence level [%] | Accuracy of positioning estimate by sensing (for a target confidence level) |  | Accuracy of velocity estimate by sensing (for a target confidence level) |  | Sensing resolution |  | Max sensing service latency [ms] | Refreshing rate [s] | Missed detection [%] | False alarm [%] | Sensing service description in a target sensing service area |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Horizontal [m] | Vertical [m] | Horizontal [m/s] | Vertical [m/s] | Range resolution [m] | Velocity resolution (horizontal/ vertical) [m/s x m/s] |  |  |  |  |  |
| Object detection and tracking | 2 | 95 | 2 | 5 | 1 | N/A | 1 | 1 | 1000 | 0.2 | 0.1 to 5 | 5 | Outdoor (e.g., detection of human, UAV) |
|  | 3 | 95 | 1 | 1 | 1 [3], [4] | 1 | 1 [3], [4] | 1 x 1 [3] | 100 [2], or 1000 (NOTE 3); 5000 for detection in highway | 0.05 to 1 | 2 | 2 | Indoor/outdoor (e.g., detection and tracking of human, animal, UAV) |

### 6.x.6	Potential New Requirements needed to support the use case

The following requirements are derived from the use cases above. These requirements need to be fulfilled within the service volume, a defined volume where the service is offered, that the specific use case would require (e.g. an airspace volume for drones, ground level for cars).

The 6G system shall be able to predict network conditions to ensure UE’s can take actions if communication connection should be bad or lost.

The 6G system shall support confidentiality, integrity protection, and privacy of the 3GPP mobility data that will be collected and processed.

The 6G system shall support following KPI’s simultaneously:

- For the communication part:

  - User experienced data rate in the range of 1-10Mb/s, depending on use case. Lower for sending warnings and assistance and higher for digital maps.

  - Round trip latency in the range of 40ms.

  - Service availability for communication 99.99% within service volume.

  - Connection density in the range of 104 devices/km2 (all served vehicles on ground and in the air).

- For the spatial data part:

  - Location accuracy in 3D, 1m with 90% probability and within 99% of the service volume.

    - 20-120m above ground the required accuracy is 10m.

    - >120m above ground the required accuracy is 50m.

  - Sensing of objects in line with category 2 or 3 in Table 6.2-1 in TS 22.137, within 99% of the service volume.

    - A reduced accuracy can be required from >20m above ground.