3GPP TSG-SA WG1 Meeting SA1#108 	S1-244090

Orlando, USA, 18th- 22nd November, 2024

Title:	New requirement on UAC for RedCap devices

Agenda Item:	5

Source:	Jio

Contact:	Anupkumar Pandey

Anupkumar.pandey@ril.com

Abstract: This contribution discusses the use cases for Redcap devices and proposes the potential requirement on UAC for Redcap.

# 1. Introduction

According to the current 3GPP specifications, network operators have the option to bar Reduced Capability (RedCap) User Equipment (UE) by utilizing specific information elements in System Information Block Type 1 (SIB1). This is achieved through the cellBarredRedCap-r17 parameter, which includes the elements cellBarredRedCap1Rx-r17 and cellBarredRedCap2Rx-r17.

However, this barring mechanism using the SIB1 information element cellBarredRedCap-r17 applies uniformly to all RedCap UEs within the cell. In the event of network congestion, activating this barring parameter will indiscriminately prevent all RedCap devices from accessing the network, regardless of their service requirements or priority levels.

This approach poses a significant issue for critical services that rely on RedCap UEs requiring continuous connectivity. Essential applications provided by always-on RedCap devices—such as emergency services, healthcare monitoring, or industrial control systems—would be disrupted if the network bars RedCap UEs using the current mechanism. Consequently, the inability to differentiate between RedCap devices based on their service criticality may lead to unintended service interruptions for vital use cases.

# 2. Discussion

According to the current 3GPP specifications, network operators have the option to bar Reduced Capability (RedCap) devices by configuring specific parameters in the System Information Block Type 1 (SIB1). These parameters are cellBarredRedCap1Rx-r17 and cellBarredRedCap2Rx-r17, collectively referred to as cellBarredRedCap-r17. This mechanism allows operators to control network access for RedCap devices under certain conditions, such as during network congestion or maintenance periods.

RedCap devices, introduced in Release 17, are a class of User Equipment (UE) designed for Internet of Things (IoT) applications that require lower complexity, reduced power consumption, and support for moderate data rates. They are intended to serve a wide range of use cases, from non-critical consumer applications to essential services like healthcare monitoring, industrial automation, and critical infrastructure management.

However, the current barring mechanism using the SIB1 Information Element cellBarredRedCap-r17 applies uniformly to all RedCap devices within a network cell. This means that when the network sets this parameter to bar RedCap devices—perhaps in response to congestion—all RedCap devices are prevented from accessing the network without exception. There is no provision to differentiate between devices based on their service criticality or specific use cases.

This indiscriminate barring poses significant challenges:

Disruption of Critical Services: RedCap devices supporting essential or mission-critical services would be barred along with non-critical devices. For example, RedCap devices used in medical monitoring systems, emergency response equipment, or industrial control systems require uninterrupted connectivity. Barring these devices could lead to service outages with severe consequences for safety and operations.

Lack of Granularity: The inability to selectively bar devices based on service priority or application type limits the operator's flexibility in managing network resources. Operators cannot prioritize network access for critical services during congestion, which is essential for maintaining service quality and reliability for vital applications.

Impact on Service Level Agreements (SLAs): Operators may have contractual obligations to ensure a certain level of service availability for critical applications. Uniformly barring all RedCap devices could lead to SLA violations and potential legal or financial repercussions.

User Experience Degradation: End-users relying on critical services provided by RedCap devices would experience unexpected disruptions, leading to dissatisfaction and potential loss of trust in the network provider.

## Need for Enhanced Barring Mechanisms:

To address these challenges, there is a need for more sophisticated barring mechanisms that allow for:

Selective Barring: Enabling operators to bar non-critical RedCap devices while allowing critical ones to maintain network access. This could be achieved by categorizing RedCap devices based on their service priority or application type and applying barring rules accordingly.

Dynamic Resource Management: Implementing network policies that can adjust in real-time to changing network conditions, ensuring that critical services receive the necessary resources even during congestion.

Policy-Based Controls: Utilizing advanced policy frameworks that consider various factors such as device capabilities, subscription profiles, and real-time network status to make informed barring decisions.

## Possible Solutions with Proposals:

Proposal 1: Introduce Service Classes for RedCap Devices

- Service Prioritization: Define multiple service classes or priority levels for RedCap devices within the specifications. Devices can be assigned a priority based on their service criticality.

- Selective Barring Parameters: Extend the SIB1 parameters to include options for barring specific service classes of RedCap devices. This allows operators to bar lower-priority devices while keeping critical services active.

Proposal 2: Enhance Unified Access Control (UAC) for RedCap Devices

- Additional Access Categories for RedCap Devices: Introduce new Access Categories within the existing UAC framework specifically for RedCap devices based on various services. The types of access categories can be further discussed with UE Original Equipment Manufacturers (OEMs) to cover major potential use cases.

- Selective Barring Using UAC:Utilize UAC for RedCap devices to allow operators to differentiate and control access during network congestion or disaster situations. Operators could filter out non-critical services and maintain network access for critical RedCap devices when too many devices are connected to the network.

- Fractional Access Control: Leverage UAC parameters such as uac-BarringFactor and uac-BarringTime to allow a fraction of RedCap devices to access the network. This enables RedCap devices with tolerant service requirements to access the network with a reduced access probability, helping manage network load while maintaining essential services.

Proposal 3: Specify System Information Indications for Selective Barring

- New System Information Indications: Define new system information indications that allow operators to bar specific RedCap devices or use cases from camping on a gNB. This provides more granular control over network access for RedCap devices, ensuring critical services remain connected while non-critical devices can be barred when necessary.

Proposal 4: Utilize Access Class Barring (ACB) Techniques

- Extend ACB to Include RedCap Devices: Utilize or extend existing Access Class Barring techniques to include RedCap devices, enabling more granular control over device access based on predefined access classes.

Proposal 5: Implement Network Slicing and Quality of Service (QoS) Differentiation

- Dedicated Network Slices: Allocate dedicated network slices for critical RedCap services, isolating them from congestion affecting other services.

- QoS Profiles: Assign different QoS profiles to RedCap devices based on their service requirements, ensuring that critical services receive higher priority in resource allocation.

Proposal 6: Introduce Device and Application Signaling Mechanisms

- Application Awareness: Implement mechanisms where RedCap devices can signal their service criticality to the network, enabling the network to make informed barring decisions.

- Subscription Data Enhancements: Include service priority information in the subscription data stored in the Unified Data Management (UDM), allowing the network to consider this information during access control.

Proposal 7: SA1 to study addition of mentioned Technical Specifications to Support Enhanced UAC for RedCap Devices

- Listing Impacted Technical Specifications: To implement enhanced UAC for RedCap devices, modifications are needed in the following technical specifications:

### TS 22.261 - Service Requirements for Next Generation New Services and Markets

- Update Access Identities: Modify Table 6.22.2.2-1 to include new Access Identities for RedCap devices, allowing the network to recognize and handle them differently based on service requirements.

- Update Access Categories: Modify Table 6.22.2.3-1 to define new Access Categories specific to RedCap devices, facilitating more granular access control.

- Table 6.22.2.2-1: Access Identities

| Access Identity number | UE configuration |
| --- | --- |
| 0 | UE is not configured with any parameters from this table |
| 1 (NOTE 1) | UE is configured for Multimedia Priority Service (MPS). |
| 2 (NOTE 2) | UE is configured for Mission Critical Service (MCS). |
| 3 | UE for which Disaster Condition applies (note 4) |
| 4-10 | Reserved for future use |
| 11 (NOTE 3) | Access Class 11 is configured in the UE. [PLMN Use] |
| 12 (NOTE 3) | Access Class 12 is configured in the UE. [Security Services] |
| 13 (NOTE 3) | Access Class 13 is configured in the UE. [Public Utilities] |
| 14 (NOTE 3) | Access Class 14 is configured in the UE. [Emergency Services] |
| 15 (NOTE 3) | Access Class 15 is configured in the UE. [PLMN Staff] |
| NOTE 1: Access Identity 1 is used by UEs configured for MPS, in the networks where the configuration is valid. The networks where the configuration is valid are NPN or HPLMN, PLMNs equivalent to HPLMN, and visited PLMNs of the home country. Access Identity 1 is also valid when the UE is explicitly authorized by the network based on specific configured NPNs or based on specific configured PLMNs inside and outside the home country. NOTE 2: Access Identity 2 is used by UEs configured for MCS, in the networks where the configuration is valid. The networks where the configuration is valid are NPN or HPLMN or PLMNs equivalent to HPLMN and visited PLMNs of the home country. Access Identity 2 is also valid when the UE is explicitly authorized by the network based on specific configured NPNs or based on specific configured PLMNs inside and outside the home country. NOTE 3: Access Identities 11 and 15 are valid in Home PLMN only if the EHPLMN list is not present or in any EHPLMN. Access Identities 12, 13 and 14 are valid in Home PLMN and visited PLMNs of home country only. For this purpose, the home country is defined as the country of the MCC part of the SUPI. NOTE 3a: Access Identities 11 to 15 are valid in an NPN. NOTE 4: The configuration is valid for PLMNs that indicate to potential Disaster Inbound Roamers that the UEs can access the PLMN. See clause 6.31. |  |

- Table 6.22.2.3-1: Access Categories

| Access Category number | Conditions related to UE | Type of access attempt |
| --- | --- | --- |
| 0 | All | MO signalling resulting from paging |
| 1 (NOTE 1) | UE is configured for delay tolerant service and subject to access control for Access Category 1, which is judged based on relation of UE’s HPLMN and the selected PLMN. | All except for Emergency, or MO exception data |
| 2 | All | Emergency |
| 3 | All except for the conditions in Access Category 1. | MO signalling on NAS level resulting from other than paging |
| 4 | All except for the conditions in Access Category 1. | MMTEL voice (NOTE 3) |
| 5 | All except for the conditions in Access Category 1. | MMTEL video |
| 6 | All except for the conditions in Access Category 1. | SMS |
| 7 | All except for the conditions in Access Category 1. | MO data that do not belong to any other Access Categories (NOTE 4) |
| 8 | All except for the conditions in Access Category 1 | MO signalling on RRC level resulting from other than paging |
| 9 | All except for the conditions in Access Category 1 | MO IMS registration related signalling (NOTE 5) |
| 10 (NOTE 6) | All | MO exception data |
| 11-31 |  | Reserved standardized Access Categories |
| 32-63 (NOTE 2) | All | Based on operator classification |

### TS 38.331 - Radio Resource Control (RRC) Protocol Specification

- Modifications in SIB1: Add UAC parameters specific to RedCap devices:

uac-BarringInfo SEQUENCE {

uac-BarringForCommon          UAC-BarringPerCatList         OPTIONAL,

uac-BarringPerPLMN-List       UAC-BarringPerPLMN-List       OPTIONAL,

uac-BarringInfoSetList        UAC-BarringInfoSetList,

uac-AccessCategory1-SelectionAssistanceInfo CHOICE {

plmnCommon                  UAC-AccessCategory1-SelectionAssistanceInfo,

individualPLMNList          SEQUENCE (SIZE (2..maxPLMN)) OF UAC-AccessCategory1-SelectionAssistanceInfo

} OPTIONAL

} OPTIONAL,

  - Introduce New Access Categories:Define new UAC-BarringPerCat entries for RedCap devices in UAC-BarringPerCatList.

  - Define Barring Factors and Times: Specify uac-BarringFactor and uac-BarringTime for RedCap devices in UAC-BarringInfoSetList.

### TS 24.501 - Non-Access-Stratum (NAS) Protocol for 5G System (5GS)

Update Mapping Table for Access Categories: Modify Table 4.5.2.2 to include rules for RedCap devices, ensuring proper mapping between access attempts and Access Categories.

Table 4.5.2.2: Mapping table for access categories

| Rule # |  | Type of access attempt |  | Requirements to be met |  | Access Category |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Response to paging or NOTIFICATION over non-3GPP access; 5GMM connection management procedure initiated for the purpose of transporting an LPP message without an ongoing 5GC-MO-LR procedure; Access attempt to handover of ongoing MMTEL voice call, MMTEL video call or SMSoIP from non-3GPP access |  | Access attempt is for MT access, or handover of ongoing MMTEL voice call, MMTEL video call or SMSoIP from non-3GPP access |  | 0 (= MT_acc) |  |  |
| 2 |  | Emergency |  | UE is attempting access for an emergency session (NOTE 1, NOTE 2) |  | 2 (= emergency) |  |  |
| 3 |  | Access attempt for operator-defined access category |  | UE stores operator-defined access category definitions valid in the current PLMN as specified in subclause 4.5.3, and access attempt is matching criteria of an operator-defined access category definition |  | 32-63 (= based on operator classification) |  |  |
| 3.1 |  | Access attempt for MO exception data |  | UE is in NB-N1 mode and allowed to use exception data reporting (see the ExceptionDataReportingAllowed leaf of the NAS configuration MO in 3GPP TS 24.368 [17] or the USIM file EFNASCONFIG in 3GPP TS 31.102 [22]), and access attempt is for MO data or for MO signalling initiated upon receiving a request from upper layers to transmit user data related to an exceptional event. |  | 10 (= MO exception data) |  |  |
| 4 |  | Access attempt for delay tolerant service |  | (a) UE is configured for NAS signalling low priority or UE supporting S1 mode is configured for EAB (see the "ExtendedAccessBarring" leaf of NAS configuration MO in 3GPP TS 24.368 [17] or 3GPP TS 31.102 [22]) where "EAB override" does not apply, and (b): the UE received one of the categories a, b or c as part of the parameters for unified access control in the broadcast system information, and the UE is a member of the broadcasted category in the selected PLMN or RPLMN/equivalent PLMN (NOTE 3, NOTE 5, NOTE 6, NOTE 7, NOTE 8) |  | 1 (= delay tolerant) |  |  |
|  | 4.1 |  | MO IMS registration related signalling |  | Access attempt is for MO IMS registration related signalling (e.g. IMS initial registration, re-registration, subscription refresh) or for NAS signalling connection recovery during ongoing procedure for MO IMS registration related signalling (NOTE 2a) |  | 9 (= MO IMS registration related signalling) |  |
| 5 |  | MO MMTel voice call |  | Access attempt is for MO MMTel voice call or for NAS signalling connection recovery during ongoing MO MMTel voice call (NOTE 2) |  | 4 (= MO MMTel voice) |  |  |
| 6 |  | MO MMTel video call |  | Access attempt is for MO MMTel video call or for NAS signalling connection recovery during ongoing MO MMTel video call (NOTE 2) |  | 5 (= MO MMTel video) |  |  |
| 7 |  | MO SMS over NAS or MO SMSoIP |  | Access attempt is for MO SMS over NAS (NOTE 4) or MO SMS over SMSoIP transfer or for NAS signalling connection recovery during ongoing MO SMS or SMSoIP transfer (NOTE 2) |  | 6 (= MO SMS and SMSoIP) |  |  |
| 8 |  | UE NAS initiated 5GMM specific procedures |  | Access attempt is for MO signalling |  | 3 (= MO_sig) |  |  |
| 8.1 |  | Mobile originated location request |  | Access attempt is for mobile originated location request (NOTE 9) |  | 3 (= MO_sig) |  |  |
| 8.2 |  | Mobile originated signalling transaction towards the PCF |  | Access attempt is for mobile originated signalling transaction towards the PCF (NOTE 10) |  | 3 (= MO_sig) |  |  |
| 9 |  | UE NAS initiated 5GMM connection management procedure or 5GMM NAS transport procedure |  | Access attempt is for MO data |  | 7 (= MO_data) |  |  |
| 10 |  | An uplink user data packet is to be sent for a PDU session with suspended user-plane resources |  | No further requirement is to be met |  | 7 (= MO_data) |  |  |

# 3. Conclusions and Proposal

Based on the above analysis, it is proposed to agree the following requirements on UAC for Redcaps.

Proposal 1: Introduce Service Classes for RedCap Devices

- Service Prioritization: Define multiple service classes or priority levels for RedCap devices within the specifications. Devices can be assigned a priority based on their service criticality.

- Selective Barring Parameters: Extend the SIB1 parameters to include options for barring specific service classes of RedCap devices. This allows operators to bar lower-priority devices while keeping critical services active.

Proposal 2: Enhance Unified Access Control (UAC) for RedCap Devices

- Additional Access Categories for RedCap Devices: Introduce new Access Categories within the existing UAC framework specifically for RedCap devices based on various services. The types of access categories can be further discussed with UE Original Equipment Manufacturers (OEMs) to cover major potential use cases.

- Selective Barring Using UAC:Utilize UAC for RedCap devices to allow operators to differentiate and control access during network congestion or disaster situations. Operators could filter out non-critical services and maintain network access for critical RedCap devices when too many devices are connected to the network.

- Fractional Access Control: Leverage UAC parameters such as uac-BarringFactor and uac-BarringTime to allow a fraction of RedCap devices to access the network. This enables RedCap devices with tolerant service requirements to access the network with a reduced access probability, helping manage network load while maintaining essential services.

Proposal 3: Specify System Information Indications for Selective Barring

- New System Information Indications: Define new system information indications that allow operators to bar specific RedCap devices or use cases from camping on a gNB. This provides more granular control over network access for RedCap devices, ensuring critical services remain connected while non-critical devices can be barred when necessary.

Proposal 4: Utilize Access Class Barring (ACB) Techniques

- Extend ACB to Include RedCap Devices: Utilize or extend existing Access Class Barring techniques to include RedCap devices, enabling more granular control over device access based on predefined access classes.

Proposal 5: Implement Network Slicing and Quality of Service (QoS) Differentiation

- Dedicated Network Slices: Allocate dedicated network slices for critical RedCap services, isolating them from congestion affecting other services.

- QoS Profiles: Assign different QoS profiles to RedCap devices based on their service requirements, ensuring that critical services receive higher priority in resource allocation.

Proposal 6: Introduce Device and Application Signaling Mechanisms

- Application Awareness: Implement mechanisms where RedCap devices can signal their service criticality to the network, enabling the network to make informed barring decisions.

- Subscription Data Enhancements: Include service priority information in the subscription data stored in the Unified Data Management (UDM), allowing the network to consider this information during access control.

Proposal 7: SA1 to study addition of mentioned Technical Specifications to Support Enhanced UAC for RedCap Devices