3GPP TSG-SA1 Meeting # 108	S1-244123

Orlando, Florida, USA 18-22 November 2024	(revision of S1-24xxxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.153 | CR | 0071 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MPS when access to EPC/5GC is satellite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Peraton Labs [CISA ECD, AT&T, T-Mobile US, Verizon] |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | Dummy |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current requirements in TS.22.153 do not cover MPS when the UE or IoT device has satellite access to the EPC/5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Text modifications and addition of a new clause with normative requirements to support: - MPS for MMTEL voice/video calls when the UE has satellite access to the EPC/5GC, and - MPS for DTS when the UE or IoT device has satellite access to the EPC/5GC. - MPS for Messaging services when the UE or IoT device has satellite access to the EPC/5GC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of MPS support when the UE or IoT device has satellite access to the EPC/5GC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18 and new clause 11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START of 1st CHANGE***

## 5.18	3GPP and non-3GPP accesses

The 3GPP system will select the most appropriate RAT for one or more services active on the UE, when a UE is using one or more of the following:

-	Terresterial 3GPP RATs (i.e., E-UTRA, NR),

-	Terresterial non-3GPP RATs (i.e., WLAN),

-	Satellite 3GPP RATs, and

-	Satellite non-3GPP RATs.

The system shall support MPS when the UE is using one or more of the above RATsconnected to the same EPC.

The system shall support MPS when the UE is using using one or more of the above RATs connected to the same 5GC.

NOTE:	The following conditions apply to the above requirements:

-	MPS support is subject to regulatory rules,

-	MPS is supported by terrestrial operator policy,

-	MPS is supported by satellite operator policy (if the satellite operator is not the terrestrial operator), and

-	MPS is supported by terrestrial operator and satellite operator agreements (if any).

*** END of 1st CHANGE***

*** Start of 2nd CHANGE***

# 11	Satellite access

## 11.1	MPS when access to the EPC/5GC is satellite

MPS can be supported when the access to the EPC/5GC is satellite. Support of MPS when the UE/IoT device has satellite access is important during certain disaster scenarios such as when terrestrial access networks are unavailable or degraded. In some cases, satellite might be the only available access (e.g., remote areas with only satellite coverage or during terrestrial access network failure) and therefore critical for MPS communications.

The following subclauses provide requirements specific to satellite access to EPC/5GC.

NOTE:	The following conditions apply to all requirements in clause 11:

-	MPS support is subject to regulatory rules,

-	MPS is supported by terrestrial operator policy,

-	MPS is supported by satellite operator policy (if the satellite operator is not the terrestrial operator), and

-	MPS is supported by terrestrial operator and satellite operator agreements (if any).

### 11.1.1	MPS for MMTEL voice and video

#### 11.1.1.1	General

For a UE with satellite access to the EPC/5GC, the system shall support the requirements in clauses 9.1 and 9.2 for MMTEL voice and voice conference calls and MMTEL video and video conference calls by an authorized MPS Service User using

-	a UE with a subscription for MPS, and

-	a UE that does not have an MPS subscription.

The system shall support the requirements in clauses 9.1 and 9.2 for MPS for MMTEL voice and voice conference calls and MPS for MMTEL video and video conference calls terminating to a UE with satellite access to the EPC/5GC regardless of the terminating UE subscription for MPS.

#### 11.1.1.2	Authentication and authorization

For a UE with a 3GPP subscription for MPS and with satellite access to the EPC/5GC, the system shall support MPS for MMTEL voice/video authorization based on the UE subscription information.

For a UE that does not have a 3GPP subscription for MPS and with satellite access to the EPC/5GC, the system shall support MPS for MMTEL voice/video authorization based on Service User credentials not associated with the UE (e.g., a calling card number, PIN or security token).

#### 11.1.1.3	Service continuity

The system shall support MPS for MMTEL voice/video continuity when features for service continuity as specified in clause 6.46.3 of TS 22.261 [7] are supported within a satellite access network (e.g., ISLs), between satellite networks, and between a terrestrial access network and a satellite access network.

### 11.1.2	MPS for DTS

#### 11.1.2.1	General

For a UE or IoT device with satellite access to the EPC/5GC, the system shall support the requirements in clause 9.3 on MPS for DTS sessions for:

-	an authorized Service User using a UE with a subscription for MPS,

-	an authorized Service User using a UE that does not have an MPS subscription,

-	an authorized IoT device with a subscription for MPS, and

-	an authorized enterprise network activation of MPS for DTS for remote MPS subscribed UEs/IoT devices associated with the enterprise network.

#### 11.1.2.2	Authentication and authorization

For a UE or IoT device with a 3GPP subscription for MPS and with satellite access to the EPC/5GC, the system shall support MPS for DTS authorization based on the UE subscription information.

For a UE that does not have a 3GPP subscription for MPS and with satellite access to the EPC/5GC, the system shall support MPS for DTS authorization based on Service User credentials not associated with the UE (e.g., a calling card number, PIN or security token).

For an authorized enterprise network that is activating MPS for DTS for a remote UE or IoT device with a 3GPP subscription for MPS, the system shall support MPS for DTS authorization based on the UE or IoT device subscription information.

#### 11.1.2.3	Service continuity

The system shall support MPS for DTS continuity when features for service continuity as specified in clause 6.46.3 of TS 22.261 [7] are supported within a satellite access network (e.g., ISLs), between satellite networks, and between a terrestrial access network and a satellite access network.

### 11.1.3	MPS for Messaging services

#### 11.1.3.1	General

For a UE or IoT device with satellite access to the EPC/5GC, the system shall support the requirements in clause 9.4 on MPS for Messaging supported using IMS Messaging, SMS and/or MMS, or MSGin5G.

#### 11.1.3.2	Authentication and authorization

For a UE or IoT device with a 3GPP subscription for MPS and with satellite access to the EPC/5GC, the system shall support MPS for Messaging authorization based on the UE subscription information.

For an authorized enterprise network that is activating MPS for Messaging for a remote UE or IoT device with a 3GPP subscription for MPS, the system shall support MPS for Messaging authorization based on the UE or IoT device subscription information.

#### 11.1.3.3	Service continuity

The system shall support MPS for Messaging continuity when features for service continuity as specified in clause 6.46.3 of TS 22.261 [7] are supported within a satellite access network (e.g., ISLs), between satellite networks, and between a terrestrial access network and a satellite access network.

*** END of 2nd CHANGE***

*** END of CHANGE***