3GPP TSG SA WG 1 Meeting #108 	S1-244131

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Source:	CATT

pCR Title:	Use Case on Efficient Data Exchange for Cross-domain AI/ML Inference

Draft Spec:	3GPP TR 22.870 v0.0.0

Agenda item:	8.1.7

Document for:	Approval

Contact:	Qing Wan wanqing1@cictmobile.com

Abstract: This contribution introduces one use case about the AI/ML relevant data exchange within 6G system for cross-domain AI/ML inference.

1. Introduction

Beyond 5G, 6G system is expected to support native AI in the initial architecture to address the applications with ML training/inference’s need as well as improve the network operation in a more intelligent manner. This trend will bring out a challenge for the 6G system to manage and execute the in-network data exchange among various AI/ML endpoints efficiently. This contribution proposes a use case about how to exchange the data relevant to AI/ML inference efficiently within 6G system for cross-domain AI/ML inference.

2. Reason for Change

Introduce a use case about efficient data exchange within 6G system for cross-domain AI/ML inference.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR22.870 v0.0.1

* * * First Change * * * *

W	Other Use Cases

W. x	Use case on efficient data exchange for cross-domain AI/ML inference

W.x.1	Description

Different from traditional network operation, AI is data-driven and will introduce vast amount of data exchanged among various types of AI/ML endpoints (e.g. data provider, network entities with AI built-in, data consumer). A typical case, AI/ML model training and inference will include several operations that require data exchange between AI/ML endpoints, such as collecting AI datasets for training, transmission of AI/ML model and intermediate data, transmission of training data and inference result, etc. Such operations raise big challenge to the mobile communication system in transmission cost and complexity.

In addition, some inference requirements of the application cannot be fulfilled with single model or one-time inference due to the data security across the AI/ML endpoints, or the complexity of the request itself, so that there is additional data exchange across the AI/ML endpoints belonging to different inference tasks. For example, the trajectory planning for the vehicle like autonomous driving car, UAV need to consider the regulatory framework, efficiency and security of the road or flight zone. However when the vehicle moves with the need of continuous communication service, the situation becomes complicated in that the communication quality along the planned path will be prioritized to be an important factor to evaluate overall user experience. The inference result based on the AI/ML model used for the transportation management can only be treated as a local optimal result unless verified by the inference result based on network AI/ML model. Thus, the overall optimal inference result may request the collaboration of multiple AI/ML models, and/or multiple inference courses.

This use case describes the cross-domain AI/ML inference with the collabration of cloud AI/ML model and network AI/ML model for different inference tasks. It presents a hybrid data exchange model including the transmission between AI/ML endpoints of the same inference task, and those of different inference task. Different from the legacy user plane path(with mark ①), which is designated for the transmission of user service data, the data pipelines (with mark ② and ③) are carrying a lot of non-user service data(e.g. inference input data, result data) across different AI/ML endpoints such as UEs, base stations, NFs of CN, servers of data network belonging to different security domains. Thus, how to manage data transmission with different transmission characteristics and endpoints should be well defined to pursuit efficency and optimal resource utilization.

Figure W.x.1-1: Efficient Data Exchange for Cross-domain AI/ML Inference

W.x.2	Pre-conditions

The network entities of 6G core network have be deployed network related AI/ML models, e.g. NW Entity#1.

The application server is deployed transportation related AI/ML models, and is mutual trusted with 6G network.

Various UEs are the subscribers of 6G network and authorized to obtain AI service from the network. UEs are also capable of 3GPP sensing and non-3GPP sensing.

W.x.3	Service Flows

- UE sends a request to the Application Server asking for trajectory planning for UAV to the target locations, and work with the UAV to execute reliable video surveillance along the path via communication path ①.

- The application server analyzes the request and identifies two consecutive AI/ML inference tasks. The inference will be executed firstly in the application server locally based on the information and data provided by UE. Then the inference result will be fed to the network AI/ML model via transmission path ② for the following inference

- During the second inference, UAV and/or base stations will provide the information about the signal quality around UAV, detected objects in planned route, network connection status, etc to NW Entity#1 via transmission paths ③, for the inference.

- The NW Entity#1 will transmit the inference result to the Application Server via transmission path ② or control plane path (e.g. NEF)

- UAV gets the planning route from Application Server via communication path ①. And the Application Server responds UE about the task result.

W.x.4	Post-conditions

UAV can fly along the planning route as well as upload the video smoothly.

W.x.5	Existing features partly or fully covering the use case functionality

The legacy user plane connection is efficient to exchange data between UEs and data network.

W.x.6	Potential New Requirements needed to support the use case

[PR-W.x.6-1] Based on application needs, the 6G system shall support an efficient data transmission path between a UE and one core network element, or between RAN and one core network element of the same  operator, or between one network element and the application of trusted 3rd party considering the requested QoS.

[PR-W.x.6-2] Subject to operator’s policy, the 6G network shall support mechanisms to  select proper data transmission path considering  the characteristics of  data (e.g. volume, type, security level).