3GPP TSG-SA WG1 Meeting #108 	S1-244234

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Title:	Use case on active WPT for IoT in home

Agenda Item:	8.1.5

Source:	ZTE Corporation

Contact: 		xu.ling@zte.com.cn,  si.yuan@zte.com.cn, dai.bo@zte.com.cn

Abstract: <provide a short description of the content>

This contribution proposes a new use case for TR22.870, which is about 6G active wireless power transfer (A-WPT) for IoT devices in home.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- N. Ayir, T. Riihonen, M. Allén and M. F. Trujillo Fierro, "Waveforms and End-to-End Efficiency in RF Wireless Power Transfer Using Digital Radio Transmitter," in IEEE Transactions on Microwave Theory and Techniques, vol. 69, no. 3, pp. 1917-1931, March 2021, doi: 10.1109/TMTT.2020.3047654s

* * * Next Change * * * *

3	Definitions, symbols and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3GPP Wireless power transfer (WPT) service: 6G networks transmit energy through wireless signals based on UE request, and UE obtains energy from the wireless signals sent by the 6G network.

Active wireless power transfer (A-WPT): 3GPP IoT device or UE actively request energy, then 3GPP network delivery wireless power through wireless signals to response this request.

* * * Next Change * * * *

Followings are all new texts

9	Massive Communication

9.x	Use case on Active WPT for IoT device in home

9.x.1	Description

Smart homes relies on a large number of smart IoT devices deployed, including sensors, cameras, controller, etc.  These devices are scattered, and the problem of energy supply needs to be solved. The use of battery power requires to replace batteries regularly, which causes great inconvenience to users. In addition, used batteries also bring recycling and pollution problems. The use of traditional wired power supplies affects the free deployment of IoT devices in multiple scenarios, and complex wiring in the room is not only unattractive but also brings security risks.

Ambient energy harvesting is considered to address the issue, e.g., solar-powered, wind-powered and powered by P-WPT (Passive wireless power transfer, UE passively harvests energy based on existing wireless signals in the environment and does not interact with the network.).  But, powered by ambient energy harvesting requires the IoT device located near an energy source, such as heat, wind and base station. In some cases, there may be no energy source in the environment and IoT devices can not harvest energy (e.g., solar-powered devices can not work at night). In other cases, IoT devices may not be able to harvest enough energy from the environment. For example, when using P-WPT,  IoT devices passively receive wireless signals (which may be  a communication signal sent to other users) for charging. The power of passively received signals (e.g., IoT devices and communication target UEs are located in different beam directions) is often low and may not meet the charging power required by IoT devices. In a word, ambient energy harvesting is dependent on the external environment which can not provide a steady supply of energy.

In order to deal with the situation described above, more active power delivery for low-power IoT devices is being developed. The charging station transmit RF energy signals to low-power devices, which receive them using a rectifying antenna and recharge the battery. When it comes to 3GPP, base stations can actively transmit energy signals to the low-power devices to response its request, i.e. A-WPT. The 6G network can provide 3GPP WPT service (e.g., Inform base station/charging station to transmit energy signals) after receiving the charging request from the low-power IoT device. The IoT device can also exchange related charging status information to network to improve charging efficiency (e.g. report received signal strength indicator).

In addition, wireless charging of home IoT devices requires additional security considerations. High intensity electromagnetic radiation may cause harm to human body. Therefore, charging nodes need to have the ability to sense people and stop sending energy signals if human body blocks them.

Following give an introduction of basic A-WPT service flow for low power IoT device in home.

9.x.2	Pre-Conditions

Operator A deployed 3GPP A-WPT service. Bob subscribed this service for his home’s IoT devices.

There are a number of IoT devices in Bob's house,  for example human body sensor, temperature and humidity sensor, wireless camera, etc. which can achieve 3GPP WPT service from operator A.

9.x.3	Service Flows

1. In Bob’s house, the IoT devices e.g. blood pressure measuring machine, temperature and humidity sensor, wireless camera, door and window controller are working.

2. When the battery level of the window controller is lower than the threshold, it sends charge requests to the network. The charging request contains identity information for network authentication. The charge request may include message identifiers, charge duration, etc. Assuming that the charging time is quantified in milliseconds, seconds, minutes, and hours, it would require approximately 31 bits. The number of message identifier bits is approximately 8 bits or more. The reliability and latency requirements for charging requests are similar to the requirements for Building automation in 22.261[X].

3. After authentication, the 6G network select a base station B nearby to send the energy signal to the IoT device.

4. When charging, the window controller continue working and Bob can control the house window this this IoT device and 6G network.

5. When people is detected in the energy delivery area, the base station B stops sending energy signals.

6. After the battery is fully charged, the IoT device sends a charging termination request to the network, and the network controls base station B to stop sending energy signals.

7. The IoT devices can also negotiate with the 6G network to perform periodic charging at specific times (such as midnight).

9.x.4	Post-Conditions

By using A-WPT, sensors can work stably for a long time without battery replacement.

9.x.5	Existing features partly or fully covering the use case functionality

In TS22.261, Table 7.5.2-1, Performance requirements for highly reliable machine type communication can be reused.

9.x.6	Potential New Requirements needed to support the use case

[PR 9.x.6 -1] Subject to operator’s policy, the 6G network shall be able to broadcast that 3GPP WPT service is available.

[PR 9.x.6 -2] Subject to operator’s policy, the 6G system shall be able to support to start, and stop 3GPP WPT service.

[PR 9.x.6 -3] The 6G network shall be able to transmit RF power signals to provide 3GPP WPT service.

[PR 9.x.6 -4] The 6G IoT devices shall be able to request 3GPP WPT service from 6G network with 3GPP WPT service.

[PR 9.x.6 -5] The 6G IoT devices shall be able to receive RF power signals for energy harvesting.

[PR 9.x.6 -6] Subject to operator’s policy, the 6G network shall be able to authenticate 6G IoT device who request 3GPP WPT services.

[PR 9.x.6 -7] Subject to operator’s policy, the 6G network shall be able to detect the human body is in the power delivery area and temporarily stop sending power signal.

[PR 9.x.6 -8] Subject to operator’s policy, the 6G system shall be able to provide 3GPP WPT service for 6G IoT device  with performance specified in Table 9.x.6-1.

Table 9.x.6-1: KPIs for 3GPP WPT for IoT device

| Scenario | Charging Duration | Charging Service Availability | Charging service Reliability | Charging Range | IoT Devices speed |
| --- | --- | --- | --- | --- | --- |
| WPT for IoT device in home | TBD | TBD | TBD | ≤20m indoor | Stationary |
| Note: Charging Duration: The amount of time an IoT device takes from the time it initiates a charge request to the time it completes the charge Charging Service Availability: The proportion of successfully completed charging service times to the total number of services. Charging Service Reliability: The proportion of the time for receiving signal power to reach the charging threshold to the charging duration |  |  |  |  |  |