3GPP TSG SA WG 1 Meeting #108 	S1-244284

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Title:	Use Case on distributed sub-network to support local area SLA

Agenda item:	8.1.1

Source:	Huawei, HiSilicon

Contact:	Alice Li, alice.li1@huawei.com

Abstract: This contribution proposes a new network deployment mode for PLMN network (sub-network) to allow a more flexible, dynamic, and cost-effective way to support diverse vertical service requirements in local zones. This use case illustrates how to offer sub-network services for an industrial park in plug and play manner, and derives the potential system requirements.

1. Introduction

“System and Operational Aspects” has been identified as one of key areas for 6G study. This contribution proposes a new use case on distributed sub-network to support local area SLA.

2. Reason for Change

Millions of local zones in the world are used for special purposes and one benchmark for local zones is the mobile network service, where the easiness to deploy and use, the performance and cost of mobile network service are important factors for the competitiveness of a local zone. The subnetwork is the extension of the PLMN network; it offers the network services with certain-level of autonomy and diverse functionalities in an area during specific time period depending on individual customers’ requirements.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1]	IMT-2030 (6G) Promotion Group White Paper: "6G Vision and Candidate Technologies".

[x2]	Hexa-X in Deliverable D5.1: "Initial 6G Architectural Components and Enablers".

[x3]	ITU-T Recommendation M.2516-0 "Future technology trends of terrestrial International Mobile Telecommunications systems towards 2030 and beyond".

* * * Second Change * * * *

## 5.x	Use case on Distributed sub-network to support local area SLA

### 5.x.1	Description

Millions of local zones in the world are used for special purposes, e.g., an industrial park is an area zoned and planned for the purpose of industrial development, for instance the large number of industrial parks located along the River Thames in the Thames Gateway area. Other examples include shopping centres, power centres, lifestyle centres, factory outlet centres, and festival marketplaces, etc. By providing an integrated infrastructure in one location, many enterprises are attracted to such local zones, including power grid companies, manufacturing companies, banking companies, high-tech companies, and chain stores, etc. One benchmark for local zones is the mobile network service, where the easiness to deploy and use, the performance and cost of mobile network service are important factors for the competitiveness of a local zone. Thus, the landlords or admins of the local zones often need the operators to provide local area network services as subnetworks of the operator’s network, which poses the following challenges:

-	the number of the subnetworks will be millions, considering the large number of local zones and enterprises,

-	the requirements for each subnetwork will be different, considering the diversified requirements for vertical services,

-	the deployment or upgrade of the subnetwork needs to be flexible enough to enable agile service delivery,

-	to protect the business sensitive information for verticals, the traffic will not be exposed outside of the subnetwork,

-	the failure of a subnetwork or the operator’s network won’t affect the others.

The subnetwork is the extension of the PLMN network; it offers the network services with certain-level of autonomy and diverse functionalities in an area during specific time period depending on individual customers’ requirements. The subnetwork can provide services for international or national enterprises, small or large. The similar concepts are already investigated and proposed in other Standards Development Organization(s), like distributed autonomous networks released by IMT-2030 (6G) Promotion Group in white paper on "6G Vision and Candidate Technologies" [x1], Campus/non-public networks for Network of Networks released by Hexa-X in Deliverable D5.1 "Initial 6G Architectural Components and Enablers" [x2], simplified user-centric network released by ITU-R in Report  ITU-R  M.2516-0 "Future technology trends of terrestrial  International Mobile Telecommunications systems towards 2030 and beyond" [x3]. It is generally expected that the subnetwork will contribute to a more flexible, dynamic, distributed, and cost-effective deployment of the PLMN network, aiming to not only improve the global coverage and mobility performance, but also cater for the customized needs from any type of enterprises customers in addition to eMBB users.

This use case describes how to offer sub-network services for an industrial park in plug and play manner.

### 5.x.2	Pre-conditions

PLMN network of MNO X is deployed with modularized function design and in use.

The landlord (Bob) of Industrial Park asks the MNO X for subnetwork services as part of their infrastructure to attract enterprises to settle down.

### 5.x.3	Service Flows

Figure 5.x.3-1: Subnetworks for local zone with diverse offerings

- Bob and MNO X agree to use the PLMN network of MNO X to provide the mobile network services for the Industrial Park. To keep attractiveness of the Industrial Park, Bob and MNO X agree on the following items as part of the service level agreements:

-    Free data connectivity service for basic communication needs within the Industrial Park, and Internet connectivity service for communication needs beyond the Industrial Park,

-   Flexible zero-downtime addition or deletion or update of the network functionalities customized for different enterprises to rapidly onboard their services,

-   Local autonomy for traffic (e.g., signalling, data, subscription) of business sensitive services that require high-level security,

-   Changes or failures for network functionalities dedicated to an enterprise will not affect services for other enterprises.

- The MNO X, as required to better cater for the service level agreements, can deploy more physical RAN and Edge Cloud resources in the close proximity of the Industrial Park. Based on the service level agreements, the subnetwork#1 is automatically created to provide the customized network services for Industrial Park using AI technologies. The subnetwork#1 consists of RAN and the customized CN components, where the detailed topology of the subnetwork#1 is only visible within that subnetwork. The subnetwork#1 determines the required network function modules (e.g., access management, mobility management, HR session management, traffic control etc.) for the access request of any visited users, and orchestrates those modules in a service chain to efficiently handle the received access requests. Via subnetwork#1, any visitors in the Industrial Park can access local "Welcome" portal, and if granted by the portal, the visitors can access their home network to surf the Internet, i.e., subnetwork#1 can route any traffic for Internet services to the PLMN network.

- Bob agrees to provide NB-IoT network service using subnetwork#1 for the power grid companies, water supply companies, and gas supply companies within the Industrial Park. NB-IoT network functionalities are automatically added in the subnetwork#1, and this change won’t affect ongoing services or other (sub)networks. The subnetwork#1 determines the required network function modules (e.g., access management, mobility management, small data transfer, charging etc.) for the access request of any NB-IoT devices, and orchestrates those modules in a service chain to efficiently handle the received access requests. Then the power grid companies, water supply companies, and gas supply companies can enjoy the NB-IoT network services offered by subnetwork#1, e.g., read meters, meter status monitoring, etc.

- A high-tech company Y newly moves into the Industrial Park. To save deployment costs, company Y and landlord Bob agree to use subnetwork#1 to provide the local LAN services at company Y offices. LAN network functionalities are automatically added in the subnetwork#1, and this change won’t affect ongoing services or other (sub)networks. The company Y can timely synchronize the information of their employees, customers, and equipment to the subnetwork#1. The subnetwork#1 determines the required network function modules (e.g., access management, mobility management, LBO or HR session management, LAN management etc.) respectively for the access request from company Y’s employees, customers, and equipment, and then orchestrates the corresponding modules in a service chain to efficiently handle the received access requests.  As a result, any kinds of traffic for LAN services are only routed locally. In addition, the subnetwork#1 can route any traffic for Internet services or new calling services to the PLMN network. The employees can use their phones to simultaneously access the LAN service, and access their home PLMN network to make calls or surf the Internet.

- A banking company Z newly moves into the Industrial Park. Because of the high-level security needs of banking services, the company Z and landlord Bob agree to provide dedicated subnetwork service at its bank branch. Based on this agreement the subnetwork#2 is automatically created with LAN and sensing network functionalities for company Z. The subnetwork#2 consists of RAN and customized CN components, and the detailed topology of subnetwork#2 is only visible within the subnetwork, thus other (sub)networks are not affected. The company Z can timely synchronize the information of their employees, customers, and equipment to subnetwork#2. The subnetwork#2 determines the required network function modules (e.g., access management, mobility management, LBO or HR session management, LAN management, sensing management etc.) respectively for the access request from company Z’s employees, customers, and equipment, and then orchestrates the corresponding modules in a service chain to efficiently handle the received access requests. As a result, any kinds of traffic for LAN services are only routed locally in the subnetwork#2, i.e., the communication within subnetwork#2 is isolated from other subnetworks or PLMN network. In addition, the subnetwork#2 can route any traffic for headquarter bank services to the target subnetworks serving the bank headquarters of banking company Z, and route any traffic for Internet services or new calling services to the PLMN network offering Calling services or Internet services. The security for bank branch is automatically monitored based on the sensing data collected and an alert or alarm will be automatically triggered if any security threats detected.

- The high-tech company Y is going to hold expo exhibition in conference hall within Industrial Park for one week. The company Y and landlord Bob agrees to provide dedicated subnetwork service at the conference hall. Based on this agreement the subnetwork#3 is automatically created with immersive communication, real-time rendering and sensing network functionalities. The subnetwork#3 consists of RAN and customized CN components, and where the detailed topology of subnetwork#3 is only visible within the subnetwork, thus other (sub)networks are not affected. The company Y can timely synchronize the information of event visitors to subnetwork#3. The subnetwork#3 determines the required network function modules (e.g., access management, mobility management, LBO or HR session management, XRM function, sensing management etc.) for the access request from the event visitors, and then orchestrates the corresponding modules in a service chain to efficiently handle the received access requests. As a result, visitors to the expo exhibition can enjoy the high-quality experience via high-resolution professional audio and video, while simultaneously, they can access PLMN network to share experiences on social media or make calls, i.e., the subnetwork#3 can route any traffic for internet services or new calling services to the PLMN network. Meanwhile, the security for expo exhibition is automatically monitored based on the sensing data collected. The subnetwork#3 can monitor the real-time number of visitors, and adjust its capacity automatically without impact to ongoing services or other (sub)networks, e.g., when the number of visitors increases sharply in the daytime or decreases in the midnight. When the expo exhibition finishes, the subnetwork#3 is released.

- A logistics company W newly moves into the Industrial Park. To save deployment costs, the company W and landlord Bob agree to use subnetwork#1 to provide Ambient IoT services at its warehouse. Based on Ambient IoT network functionalities automatically added in the subnetwork#1, and this change won’t affect ongoing services or other (sub)networks. The subnetwork#1 determines the required network function modules (e.g., Ambient IoT management, Ambient IoT ID validation etc.) for the Ambient IoT service request from company W, and then orchestrates those modules in a service chain to efficiently handle the received service requests. Bob provides company W with a sub-portal, and the company W can use the sub-portal to perform inventory and command services at its warehouse area.

- The logistics company W newly offers the UAV parcel delivery service for customers at Industrial Park, and the company W and landlord Bob agree to use subnetwork#1 to offer UAV parcel delivery service at Industrial Park. Based on this agreement UAV and Sensing network functionalities are automatically added in the subnetwork#1, and this change won’t affect ongoing services or other (sub)networks. The company Y can timely synchronize the information of their UAV equipment to subnetwork#1. The subnetwork#1 determines the required network function modules (e.g., access management, mobility management, LBO session management, UAV management, sensing management etc.) for the access request from the UAVs, and then orchestrates the corresponding modules in a service chain to efficiently handle the received access requests. As a result, any parcels collected at or delivered to Industrial Park will be served by the UAVs.

- A failure occurs at the PLMN network (outside of the Industrial Park), and this failure won’t bring any impacts to any subnetworks (e.g., #1, #2) in the Industrial Park, so the subnetwork#1 and #2 can still provide local services for enterprises within the Industrial Park.

- A fire disaster happens at the company Y offices, and this disaster causes the failure of LAN type services for company Y. Some colleagues can still stay at the offices that not affected by the fire disaster, while some colleagues have to work at home due to the burned offices. To keep network resiliency and availability, the subnetwork#1 and the PLMN network will take over the subscribers of company Y (colleagues, equipment, customers) in order to offer the basic connectivity services towards the enterprises servers. In this case, the employees of company Y at offices can access the subnetwork#1 to access the enterprise servers, and the employees of company Y at home can access the PLMN network to remotely access the enterprise server.

### 5.x.4	Post-conditions

Thanks to the subnetwork services offered by MNO X, now mobile network service is one benchmarking infrastructure in Industrial Park, more and more enterprises move into the Industrial Park, the landlord can provide the enterprises separately with their customized mobile network services.

### 5.x.5	Existing features partly or fully covering the use case functionality

Currently, there are several 5G features allowing to deploy network services in a local area to cater for the requirements from different enterprises, this includes Network Slicing, Edge Computing, and Non-public Network. Faced with the diverse use cases and scenarios, the service requirements of campus networks are various. Therefore, the network deployed on campus shall be flexible enough to meet different demands, this includes:

-	the number of the campus networks will be millions, considering the large number of local zones and enterprises,

-	the requirements for each campus network will be different, considering the diversity of requirements for vertical services,

-	the deployment or upgrade of the campus network needs to be flexible enough to enable agile service delivery,

-	to protect the business sensitive information for verticals, the traffic will not be exposed outside of the campus network,

-	the failure of a campus network or the operator’s network won’t affect the others.

Network Slicing is a concept to allow differentiated treatment depending on each customer requirements. With slicing, it is possible for MNOs to consider customers as belonging to different tenant types with each having different service requirements that govern in terms of what slice types each tenant is eligible to use based on SLA and subscriptions. The support of network slicing relies on the principle that traffic for different slices is handled by different PDU sessions. Network can realise the different network slices by scheduling and by providing different L1/L2 configurations. Instances of Network slices may differ for supported features and network functions optimisations. Thus, the network slicing is a means to allow the PLMN network to have differentiated treatments via network configuration, traffic scheduling for users accessing to the PLMN network, however,

-	even though network can support large number of slices (hundreds) as defined in TS 38.300, but this is far behind the target number of millions of local zones and enterprises

-	resources of UE, RAN and Core can be shared among different network slices, instantiation or release or update of one network slicing instance impacts the configuration on the UE, RAN and Core, this is not easy to achieve in short time and may subsequently affect the other services, thus configuring network slicing may not be flexible and agile enough to meet the various dynamical service requirements of different local areas, this is unfavourable for agile service delivery and network resilience.

-	ownership of the network slice belongs to MNO, this is not applicable for sensitive services that required network and data autonomy including subscription, signalling or user traffic

Edge computing enables operators’ and 3rd party’s services to be hosted close to the UE's access point of attachment, so as to achieve an efficient service delivery through the transport network with the reduced end-to-end latency and load. The 5G Core Network selects a UPF close to the UE and forwards traffic to enable the local access to the DN via a N6 interface according to the provided traffic steering rules to the UPF. Thus, edge computing is a means to achieve low latency service delivery by offloading the user plane traffic to the local DN close to the UE for users accessing to the PLMN network, however,

-	the network resources used for edge computing and traffic local offloading enabled for edge computing are all under the control of PLMN network, this is not applicable for sensitive services that required network and data autonomy including subscription, signalling or user traffic

-	edge computing doesn’t address how to support the diversity of requirements for different vertical services and how to enable flexible deployment of new network functions for agile service delivery

Non-public Network allows the network to be deployed for a third party (e.g., Enterprise or Campus) as an NPN which can be a Standalone NPN (SNPN) or Public Network Integrated-NPN (PNI-NPN). SNPN is a network operated by an NPN operator and not relying on network functions provided by a PLMN, while a PNI-NPN is a network deployed with the support of a PLMN (e.g., by using dedicated slices and DNN). However,

-	For SNPN, all network functions are deployed on campus to compose a full-feature network including mobility management, session management, subscription management and so on. Therefore, SNPN can enable a customized isolated network according to the scenarios and service requirements. For instance, dedicated network functions of sensing or Ambient IoT can be further deployed in the SNPN to achieve dedicated use cases. However, the construction and maintenance fee may not be affordable especially for small or medium size Enterprise (SME), and the SNPN network is standalone, which means that the SNPN network is not able to have direct connectivity to the PLMN network by definition, thus the UE accessing the SNPN needs to use the SNPN network as underlay network to access to the PLMN network to simultaneously access PLMN network services and local SNPN network services (see Annex D.3 of TS 23.501), this is cumbersome and hard to apply in reality.

-	Compared with SNPN, PNI-NPN is more cost-effective. However, since the network functions (UDM, AMF, SMF, UPF, NEF) of the PNI-NPN deployed by the public network operator may be in centralized sites, the signalling and traffic may be routed out of the campus, which is roundabout. The centralized network functions are shared by the PLMN network and PNI-NPN networks, there is lack of isolation mechanisms, this may not fulfil high quality of service and security requirement that key data is processed locally in campus.

Therefore, those 5G feature (including Network Slicing, Edge Computing, and Non-public Network) are not an optimized one to fulfil all local area use cases in the future. A more flexible, dynamic, and cost-effective network deployment option should be proposed and considered.

### 5.x.6	Potential New Requirements needed to support the use case

[PR 5.x.6-1] The 3GPP system shall be able to rapidly offer the network service with certain-level of autonomy and diverse functionalities in a given area during specific time period as part of the operator’s PLMN network, i.e., deploy a subnetwork as the extension of the PLMN network in a flexible zero-downtime manner.

[PR 5.x.6-2] The 3GPP system shall support a mechanism to allow a (sub)network to take over the users subscribed to another subnetwork when that subnetwork is not available.

[PR 5.x.6-3] The 3GPP system shall support a mechanism to allow a subnetwork continue operating and providing service for its subscribers when the operator’s network is not available.

[PR 5.x.6-4] Based on the operator’s security policy, the 3GPP system shall support a mechanism to make sure the changes of network functionalities for a (sub)network only visible within that (sub)network, i.e., the changes of network functionalities for a (sub)network are not synchronized between subnetworks (e.g., inter-network topology hiding).

[PR 5.x.6-5] The 3GPP system shall support a mechanism to enable communication beyond one subnetwork, when necessary, e.g., inter-(sub)network direct connectivity and traffic routing for Internet surfing, experience sharing on social media, calling, and banking services across branches and headquarter etc.

[PR 5.x.6-6] The 3GPP system shall support a mechanism to determine the required network functionalities according to user subscription, service type, device type and network status etc, and further to orchestrate such functionalities in a service chain to efficiently handle the service request from the UE or the 3rd party.