3GPP TSG SA WG 1 Meeting #108 	S1-244293

Orlando, Florida, USA, 18-22 November 2024	(revision of S1-24xxxx)

Title:	Use case on V2X infotainment

Agenda item:	8.1.4

Source:	Huawei, HiSilicon

Contact:	Alice Li, alice.li1@huawei.com

Abstract: This contribution provides a new use case on the enhanced in-vehicle experience with the required KPIs.

1. Introduction

“Immersive Communication” has been identified as one of key areas for 6G study. This contribution proposes a new use case on enhanced in-vehicle experience.

2. Reason for Change

Future trend of vehicles becomes more digitized and intelligent, and is often regarded as a new “entertainment space”, where people can fully enjoy infotainment and immersive data via in-vehicle system. This is motivated by increased number of installed screens with larger size (e.g. 32 inch) and higher resolution (e.g. 8K) where passengers can enjoy ultra-high-resolution video contents or interactive applications.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 22.870.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x1] 	https://www.qld.gov.au/transport/safety/road-safety/driving-safely/stopping-distances

[x2] 	https://learn.microsoft.com/en-us/windows/win32/medfound/recommended-8-bit-yuv-formats-for-video-rendering

[x3] 	https://support.apple.com/zh-cn/guide/compressor/cpsrf44b12ac/4.6.6/mac/13.4

[x4] 	https://en.wikipedia.org/wiki/High_Efficiency_Video_Coding#cite_note-HEVCApril2013M1000-133

[x5] 	https://www.midlane.top/wiki/pages/viewpage.action?pageId=49677194

* * * Second Change (new clause) * * * *

## 8.x	Use case on V2X infotainment

### 8.x.1	Description

Future trend of vehicles becomes more digitized and intelligent, and is often regarded as a new “entertainment space”, where people can fully enjoy infotainment and immersive data via in-vehicle system. This is motivated by increased number of installed screens with larger size (e.g. 32 inch) and higher resolution (e.g. 8K) where passengers can enjoy ultra-high-resolution video contents or interactive applications.

Figure 8.x.1-1: Examples of enhanced in-vehicle experience (HD screens equipped on board)

### 8.x.2	Pre-conditions

The vehicle is capable of connecting to cellular network, i.e. a connected vehicle. The connected vehicle is equipped with on-board devices (e.g. screens) allowing passengers to access content data from service providers through cellular network.

Service area:  Open highways and urban lanes

Vehicle density:  Taking ‘Century Avenue Area in Pudong, Shanghai’ as an example, there are three main avenues (12, 10, 8 lanes respectively) intersected, in addition, there are two 6-lane streets and two 4-lane streets within the proximity 1km2 area as shown in figure 8.x.2-1. If 45m inter-vehicle distance plus 5m vehicle length is assumed under 60km/h [x1], there can be 20 vehicles on a 1-km lane. Therefore, there can be around 1000 vehicles within the 1 km2 area in total. More details can be seen in table 8.x.2-1.

Table 8.x.2-1 Total vehicles within ‘Century Avenue Area in Pudong, Shanghai’

| Avenue Name | Number of lanes | Vehicles/lane (1km) | Total vehicles |
| --- | --- | --- | --- |
| Century Avenue | 12 | 20 | 240 |
| Zhangyang Road | 10 | 20 | 200 |
| Dongfang Road | 8 | 20 | 160 |
| Shangcheng Road | 6 | 20 | 120 |
| Weifang Road | 6 | 20 | 120 |
| Laoshan Road | 4 | 20 | 120 |
| Yuanshen Road | 4 | 20 | 120 |
| Total | 50 | 20 | 1000 |

Figure 8.x.2-1: Number of streets (including number of lanes) within 1 km2 in Century Avenue Area in Pudong, Shanghai

HD content: 8 bits per pixel@ YUV coding [x2] [x3] & H.265 [x4] [x5]

DL Data rate: Two types of vehicles are considered here, one is 5-seat version and the other is 7-seat version. Both supports 8K and 4K screens

Table 8.x.2-2 DL data rate for different vehicle versions

| Screen configuration | Type-1: 5-seat version | Type-2: 7-seat version |
| --- | --- | --- |
| Num. of 8K screens @ 90fps | 1 | 1 |
| Num. of 4K screens @ 60fps | 3 | 5 |
| Total DL data rate (Mbps) | 175 | 215 |
| NOTE: For 4K screen @ 60fps: 3840 * 2160(pixels/frame) * 60 fps * 8 (bits/ pixel) *(1/200) ~= 20Mbps For 8K screen @ 90fps: 7680 * 4230 (pixels/frame) * 90 fps * 8 (bits/ pixel) *(1/200) ~= 115 Mbps |  |  |

Figure 8.x.2-2: Service area of enhanced in-vehicle service

### 8.x.3	Service Flows

- Wang plans to visit City A with his 6 friends by car. The vehicle is equipped with one 8K laser projection and five 4K screens. A communication link between the connected vehicle and application server is established

- During the journey, one plans to watch a movie via the 8K laser projection and other five plan to surf short videos via the remaining five 4K screens. Each passenger individually chooses which HD content he/she is interested in;

- Application server identify each passenger’s individual choices and the vehicle location; Application server check if the content is available and/or has permission to be accessed in the region where the connected vehicle is located and if the connected vehicle is authorized to receive the HD content;

- Application server makes the HD content available to individual passengers.

- Each passenger individually plays and stops the HD content at his or her own convenience

Figure 8.x.3-1: Service flow of enhanced in-vehicle service

### 8.x.4	Post-conditions

Thanks to the wide area coverage of cellular network, the passengers on board the connected vehicle can fully enjoy high-definition and immersive entertainment media content while the vehicle is moving.

### 8.x.5	Existing features partly or fully covering the use case functionality

From TS 22.261, performance requirements for high-speed vehicle scenario is provided in table 7.1-1.

3GPP TS 22.261 specifies multiple KPIs for UEs in vehicles, rather than a vehicular UE, which would be equipped with multi-media devices. Experienced data rate can be up to 215Mbps by aggregating multiple HD screens on board (e.g. one 8K + five 4K). Instead of user density, vehicle density should be considered, of which value range could be around 1000 vehicles/km2 during rush hour time for a metropolitan center area, and therefore DL area traffic capacity can be up to 215 Gbit/s/km2.

Considering the increased screens and higher-resolution videos, the experienced DL data rate is over 3 times higher than existing requirement, as well as DL area traffic capacity, and such requirements cannot be fulfilled by existing 5G network.

### 8.x.6	Potential New Requirements needed to support the use case

[PR 8.x.6-1] The 3GPP system shall be able to support the enhanced in-vehicle experience with the following KPI requirements.

|  | Experienced data rate (DL) | Area traffic capacity (DL) | Overall vehicle density | Absolute velocity | Service area |
| --- | --- | --- | --- | --- | --- |
| 5-seat vehicle (note 1) | 175 Mbps | 175 Gbps/km2 | 1 000/km2 (note 3) | up to 250 km/h | Along roads |
| 7-seat vehicle (note 2) | 215Mbps | 215 Gbps/km2 | 1 000/km2 (note 3) | up to 250 km/h | Along roads |
| NOTE 1: For 5-seat vehicle, DL experience data rate can be 175 Mbps assuming one 8K-screen (115 Mbps) and three 4K-screens (20 Mbps) are installed NOTE 2: For 7-seat vehicle, DL experience data rate can be 215 Mbps assuming one 8K-screen (115 Mbps) and five 4K-screens (20 Mbps) are installed NOTE 3: The overall vehicle density is derived assuming a total of 50 lanes within 1km2 and 20 vehicles/lane under 60km/h. |  |  |  |  |  |