3GPP TSG-SA WG1 Meeting #108 	S1-244295

Orlando, USA, 18-22 November 2024	(revision of S1-xxxxxx)

Title:	Use case on wireless power transfer

Agenda Item:	8.1.5

Source:	Huawei, Hisilicon

Contact:	Edward Hall edward.hall@huawei.com

Abstract: This use case covers the on-demand delivery of wireless power to an IoT device.

--- FIRST CHANGE ---

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TR 22.840, Study on Ambient power-enabled Internet of Things, V19.0.0

--- SECOND CHANGE (new clause) ---

## 9.x	Use case on Wireless Power Transfer

### 9.x.1	Description

Power consumption has always been the key problem for the IoT devices, especially those without batteries. Considering the rise of the factory management due to labour and device cost for battery change, the deployed base station could act as steady source of energy via electromagnetic radiation of wireless signals to support operation of those devices, i.e. base station provides wireless power transfer service.

One typical example is the instruments/sensors in the chemical factory to monitor the processing procedure. Currently, these instruments/sensors are wired deployed or equipped with a battery. For the wired deployment, it usually involves the labour cost, plastic pipe cost, construction of shallow trench, etc., which leads to huge deployment cost. On the other hand, for the case of battery equipment, periodic or temporary battery replacement (every two/three years) involves high management cost. Under some hazardous areas/core production areas, wired deployment or battery replacement are even impossible. Within this scenario, the service of wireless power transfer could become really desired.

Although ambient radio energy from the surrounding environment could be harvested, it is opportunistic and random. More importantly, since the radio signals are not for the target sensors, the mismatch exist of the signals beam. On the other hand, the communication waveform is designed for data transmission rather than only energy delivery. Hence, the energy harvesting efficiency from the ambient environment cannot be guaranteed. Wireless Power Transfer can be used alongside ambient energy harvesting, or other power source techniques to ensure a resilient power supply to remote devices.

### 9.x.2	Pre-conditions

Various sensors are widely deployed within industrial buildings/factories to monitor the environment information or machine status, such as temperature/humidity/air/water quality or equipment vibration. These sensors are registered to the network to report real-time information. Instead of equipping batteries, these sensors are equipped with energy storage components, such as capacitance, and these components could be recharged. When run out of energy, these sensors could send a request to the base stations for wireless energy service. Upon the received requires, the base stations emit wireless energy signals.

### 9.x.3	Service Flows

The following introduces the service flows of proactive wireless power transfer, which mainly consists of 5 steps:

- Wireless IoT devices S1 to Sn are deployed within a warehouse/factory to obtain and report environment information, such as smoke/temperature, etc.

- After a period of time continuous working, devices S1 to Sn are almost out of the stored energy and request energy service from the network.

- Upon the requests, the network (core network, such as the energy management function) would authenticate the UE and indicate the base stations (including serving base station and/or neighbour base station) to provide energy service to the devices S1 to Sn.

- The base stations which indicated by the network send wireless energy signals to provide a steady source for RF energy harvesting to the  devices S1 to Sn.

- Upon the received signals with an antenna which can convert the signals to direct current, the devices S1 to Sn store the energy for future usage until the stored energy reach certain threshold.

Figure 9.x.3-1: Example Flow for Wireless Power Transfer

With regarding to the wireless power time (how long the network emit wireless power signals each time) and frequency (how often the network emits wireless power signals), one of the main factors is the device’s capability. If a device has greater energy storage capacity, it would require longer service time and lower service frequency. On the contrary, if the device has smaller energy storage capacity, it generally would require less service time and higher service frequency. Another factor is the operation flow of the device, which means how often the UE would use the stored energy to do transmission operation or sensing operation. More intensive operation flow would lead to higher wireless power transfer frequency. Hence, the wireless power transfer time would generally span from seconds to hours and wireless power transfer service frequency would generally span from several times per minutes to once per day.

### 9.x.4	Post-conditions

Thanks to the proactively wireless power transfer capability of the base station, the sensor based IoT devices could be recharged without involvement of labour effort. The lifespan of the IoT devices could be greatly increased, and hence the factory management cost due to battery replacement is significantly decreased. The network-based wireless power transfer can provide timely, continuous, accurate, and adequate energy, which is a reliable source for IoT devices without batteries.

### 9.x.5	Existing features partly or fully covering the use case functionality

SA1 has studied environment monitoring based on ambient IoT sensors in TR 22.840 [1], especially, use case on ambient IoT sensors in smart homes, descriptions of the scenarios are:

For UEs only supporting any of the following, or a combination of, NB-IoT, GERAN EC-GSM-IoT [18], and  Category M1[13] of E-UTRAN enhanced-MTC, the UE shall interpret the interval value to be between 2 and 240 hours, with a step size of 2 hours between 2 and 80 hours and a step size of 4 hours between 80 and 240 hours.

Monitoring tasks in smart home scenarios can be roughly divided into two types:

- Monitoring of room environment. By deploying specific sensors that enable communication services, people can acquire real-time monitoring data (e.g. temperature, humidity) of their room environment no matter where they are.

- Monitoring of emergency situations. By deploying specific sensors that enable communication services, risk factors such as gas and smoke can be detected in the air and alarmed timely. When the sensor detects that the gas concentration in the home exceeds the threshold, it usually activates a very strong audio signal to alarm people at home. However, in case of people are out of home (e.g. while at work, or on shopping), it cannot reach people on this situation. So, it should be necessary to notify the family members through their phones as well.

In contrast with conventional battery-based sensors, Ambient IoT sensors can obtain and/or store energy from the environment, such as light, heat, wind, and radio waves, which can be converted to useable electrical energy [12]. Different ambient energy harvesting technologies have their own advantages and disadvantages, suitable for use in different environments. Energy harvesting technologies are not in the scope of the study.

The service requirements for the ambient IoT sensors are defined as

The 5G system shall be able to support communication services for Ambient IoT devices.

The 5G system shall support suitable security mechanisms for Ambient IoT devices, including authentication, encryption and data integrity.

The 5G system shall be able to provide the required communication service according to KPI given in table 5.6.6-1.

In this report, the service requirements are defined from the communication aspects. However, one of the most important aspects of the sensor based IoT devices without battery/with limit battery is energy supply. The ambient IoT sensors only consider energy harvesting from the ambient environment. The ambient energy poses the characteristics of random, opportunistic and unsteady. The power supply of the IoT-based sensors cannot be guaranteed. For example, there may be no light at night for indoor scenario, such indoor factory or warehouses. To ensure continuous working of the sensors, proactively wireless power transfer services from the base station should be supported, i.e, the IoT devices could initiate energy service to the network for wireless power transfer. New challenges to the wireless cellular systems are foreseen and need to be addressed.

### 9.x.6	Potential New Requirements needed to support the use case

[PR 6.7.6-001] The 3GPP network shall be able to provide wireless power service for sensor based IoT device which is battery-less or with energy storage capability.

[PR 6.7.6-002] The 3GPP network shall be able to support energy service control and management, e.g., collect energy service request, activate and/or deactivation energy service.

[PR 6.7.6-003] The 3GPP network shall support IoT device authentication and identification with wireless power service and capabilities.

[PR 6.7.6-004] The 3GPP system shall provide the energy service to address the KPIs for the use of sensor based IoT devices in indoor smart warehouses/factories.

Table 9.x.6-1: Potential KPIs for sensor based IoT devices in smart warehouses/factories

| Scenario | Max. allowed end-to-end latency | Typical required received power per device | Communication Service Availability | Reliability | Service Frequency | Message Size | Device density | Communication Range | Service area dimension | Device speed | Transfer interval | Positioning service latency | Positioning service availability | Positioning Accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sensor based IoT devices in smart warehouses/factories | 1 s (note 1) | uW~mW level (note 2) | 99% | NA | Once per day ~ once per minute | NA | < 5000/km2 (note 3) | Indoor: typically 10-30 meters | NA | Stationary | NA | NA | NA | NA |
| NOTE 1: This is the time interval between the IoT device send power transfer request to the IoT device receive the wireless power signal. NOTE 2: The power consumption of the temperature/pressure sensors could reach the level of mW according to [2]. NOTE 3: This density is calculated based on an individual chemical factory, where typically several hundreds of sensors based IoT devices are required to monitor the environment and production procedures. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

--- END OF CHANGES ---