3GPP TSG SA WG 1 Meeting #108 	S1-244337

Orlando, USA, 18-22 November 2024

Source:	Nokia, ETRI

pCR Title:	Pseudo-CR on updates of use case 5.16 to replace handover by mobility and GEO by GSO

Draft Spec:	3GPP TR 22.887-020

Agenda item:	7.3

Document for:	Approval

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: This contribution proposes updates of use case 5.16 in FS_5GSAT_Ph4 TR.

1. Introduction

This contribution proposes to update the description, pre-conditions, service flows and potential new requirements in use case “using GSO for initial network entry and service initialization”.

2. Reason for Change

The term “handover” assumes UE is in connected mode to GSO while searching & selecting NGSO, which is not the intention of this use case. Therefore the term “handover” should be replaced by 'mobility' being the more general term covering both idle/inactive mode and connected mode mobility.

In addition, to be consistent with GSO and NGSO in other use cases, GEO is replaced by GSO.

Also some format cleanup for pre-conditions and service flows.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.887-020.

* * * First Change * * * *

## 5.16	Use case on using GSO for initial network entry and service initialization

### 5.16.1	Description

GSO satellites have a much wider field of view than NGSO satellites and typically have longer active duty cycles than NGSO systems. They can be used to transmit cell broadcast signals and system information on a very large coverage area very efficiently, where a GSO satellite cell coverage area may be of a size that is equivalent to one or several NGSO satellite footprints areas (and thus to several NGSO cells). Cell search and network entry can be a simpler process using GSO satellites, while cell search and network entry can be an extended process for a UE served by NGSO satellites exclusively.

Furthermore, satellites often present significant payload power budget constraints, which can be more severe for NGSO satellites. In such scenarios, an NGSO satellite’s active coverage footprint might be selectively reduced in order to conserve power.  This has however the effect of creating coverage holes under the footprint of the satellite, in which cells may not be activated.

The presence of a GSO satellite coverage overlay on top of NGSO allows to bridge coverage gaps by providing broadcast signal coverage and initial access for the network.  In this mode, we assume that upon power-on, a UE will perform initial cell search, initial access and network registration (also known as “network entry”) into a GSO satellite.  During the initial access and registration procedure the network may choose to serve the UE via an available NGSO satellite.  The NGSO satellite may be instructed by the network to activate a dormant beam or cell with a specific set of resources in the targeted coverage area, or the UE may be instructed to join an existing NGSO beam or cell.

This concept can be generalized from GSO and NGSO to multiple satellite access types but the concept is most easily understood and justified when considering GSO and NGSO.

### 5.16.2	Pre-conditions

- The network supports multiple satellite access types

- The network supports mobility between multiple satellite access types

- A network plan that deconflicts frequency allocations between the component satellite access types.

- The networks support network entry initiation on one satellite access network and network entry completion on a different satellite access network.

### 5.16.3	Service Flows

- UE is turned on and wants to establish a connection

- UE does initial network entry on a GSO satellite (or more generally a satellite of the appropriate satellite access type)

- The system provides the UE enough information to enable network entry completion on a satellite of the NGSO (or more generally a satellite of the appropriate satellite network type)

- UE does network entry complete on an NGSO (or more generally a satellite of the completes

- UE does network entry completion on an NGSO spot beam

### 5.16.4	Post-conditions

The UE is registered on the preferred satellite access network and carries out communication on that satellite network.

### 5.16.5	Existing features partly or fully covering the use case functionality

3GPP Rel 19 NTN features which support the use of satellites shall cover the use case partially.

### 5.16.6	Potential New Requirements needed to support the use case

[PR 5.16.6-001] A 5G system with multi-orbit satellite access shall be able to support network entry initiation via one satellite access type and network entry completion via a different satellite access type.

Editor’s note: this requirement is FFS

[PR 5.16.6-002] A 5G System supporting multi-orbit satellite access shall enable the UE to select the relevant GSO for network entry initiation.

Editor’s note: this requirement is FFS