3GPP TSG SA WG 1 Meeting #108 	S1-244340

Orlando, USA, 18-22 November 2024

Source:	Nokia, ETRI

pCR Title:	Pseudo-CR on updates of use case 5.20 to remove unrelated pre-conditions

Draft Spec:	3GPP TR 22.887-020

Agenda item:	7.3

Document for:	Approval

Contact:	Feifei Lou (feifei.lou@nokia.com)

Abstract: This contribution proposes updates of use case 5.20 in FS_5GSAT_Ph4 TR.

1. Introduction

This contribution proposes to update the pre-conditions, service flows, existing features and potential new requirements in use case “multi-orbit satellite backhauling”.

2. Reason for Change

The second part in pre-conditions has nothing to do with the rest part of the use case. Therefore it is removed. Plus some editorial update.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 22.887-020.

* * * First Change * * * *

## 5.20	Use case on multi-orbit satellite backhauling

### 5.20.1	Description

Backhaul links are used as connectivity between a Core Network and Base Stations (gNBs). Various techniques could be used to provision backhauling, e.g., terrestrial fiber optical-, mmWave- or satellite-links. Simultaneously combining various backhaul link types could be of interest in cases when a single backhaul link cannot meet the prescribed requirements (e.g., capacity, latency). Specific characteristic of each backhaul link needs to be taken into account when combining it with another backhaul link type in order to achieve required QoS of each user within a group of users (aggregate QoS).

### 5.20.2	Pre-conditions

MLK is an airline company known for its high end on-board services. MLK takes good care of its passenger satisfaction and intends to upgrade its on-board entertainment services with on-line messaging, internet access, video streaming and gaming. MLK is looking for connectivity solutions that will enable the upgrade of its on-board entertainment. Cost efficiency of the connectivity solution is of great importance for MLK in order to remain competitive on the tight airline market.

### 5.20.3	Service Flows

A satellite operator SAOP having GEO and NGSO satellites in its satellite fleet offers backhauling connectivity solutions for the airplanes in MLK fleet anywhere around the globe. SAOP offers the most cost efficient solution for MLK as the different service types e.g., messaging, internet, video/gaming, will transfer their traffic over satellite backhauling links optimal for the selected service e.g., GEO, MEO, LEO, respectively.

While on the ground the airplane mounted base station is connected through a terrestrial backhaul. The passengers on board the airplane acquire wide variety of services (latency sensitive, latency tolerant, messaging, video or gaming) through the terrestrial backhaul or terrestrial mobile network.

After the take-off, airplane mounted base station loses the terrestrial backhaul connectivity and switches over to satellite backhauling through multiple satellite orbits i.e. LEO, MEO and/or GEO satellites. Satellite backhauling through distinct orbits have different properties with respect to latency, data rate, spectrum, coverage and availability of the satellites in different orbits. 5G system is capable of differentiating traffic among various service types of the airplane passengers. For example, 5G system will route latency tolerant services (e.g., messaging) through GEO satellite backhaul, latency sensitive services (e.g., mission critical communication, on-line gaming) through LEO satellite backhaul and video on demand through MEO satellite backhaul.

### 5.20.4	Post-conditions

MLK improved offer of their services in a cost efficient manner. MLK is able to provide wide range of services (latency sensitive, latency tolerant, high data rate and high reliability) by simultaneously exploiting multiple satellite backhauling paths.

### 5.20.5	Existing features partly or fully covering the use case functionality

3GPP TS 22.261 [2] specifies the requirements for satellite backhauling i.e. that the 5G system shall support a mechanism to determine suitable QoS parameters for traffic over a satellite backhaul, based e.g., on the latency and bandwidth of the specific backhaul.

3GPP TS 22.261 [2] specifies the requirement that the 5G system shall be able to support mechanisms to differentiate charging information for traffic carried over satellite backhaul.

The above listed requirements are focused on usage of a single satellite orbit. The cases of possible simultaneous usage of multi-orbit satellite connectivity are not explicitly considered.

### 5.20.6	Potential New Requirements needed to support the use case

[PR 5.20.6-001] The 5G system using multi-orbit satellite network for backhauling shall be able to take into account the respective capabilities (e.g., latency, data rate) and availability of the different satellite access (e.g., over GEO, MEO, LEO) to map the backhaul traffic with a given aggregated QoS.

[PR 5.20.6-002] The 5G system shall be able to collect and distinguish charging information related to backhaul traffic over different orbits of the satellite network.