3GPP TSG-SA WG1 Meeting #108	S1-244359

Orlando, Florida, USA, 18-22 November2024								(revision of S1-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0038 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New use case: Availability status of a MC User (Presence of a FRMCS User) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS_Ph6 |  |  |  |  |  | Date: |  |  | 2024-11-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is a future proof feature for FRMCS, to allow FRMCS users be aware of the availability status of the other FRMCS users when logged in the FRMCS system |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide the status of MC Service ID and Functional Alias |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing facilities for the end user, as with this feature he/she can filter a predefined list based on the status on the HMI of his/her terminal. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.18 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 9.18	Presence

### 9.18.1	Introduction

The presence function provides the availability status of a user to other FRMCS users currently logged in the system, based on FRMCS User identity or functional identity. This function can also provide HMI facilities, allowing for example, filtering of a predefined list of FRMCS users based on their status. The following use cases are defined:

-	Presence of a FRMCS User identity

-	Presence of a FRMCS user registered to a functional identity

-	Availability of a functional identity

### 9.18.2	Use case: Presence of a FRMCS User identity

#### 9.18.2.1	Description

A user identified by the FRMCS system is automatically assigned to an availability status. The availability status of FRMCS User identity and the updates of it, are displayed on the HMI of the FRMCS User’s device.

#### 9.18.2.2	Pre-conditions

The user is successfully logged into the FRMCS system and has obtained a FRMCS User identity.

#### 9.18.2.3	Service flows

1) The FRMCS user identity of the FRMCS user is automatically set to ‘available’ by the system, once successfully logged in.

2) The FRMCS user (e.g. driver2) initiates a communication to another FRMCS user (e.g. driver3), using their FRMCS user identities, because they haven’t registered to functional aliases. When the two FRMCS users are connected in the call, the FRMCS user identities of both participants of the call are automatically set to ‘busy (in communication)’ by the FRMCS system

3) Driver 2 has terminated the communication with driver 3. Both driver2 and driver3 have no other calls to handle immediately, so their respective FRMCS user identities are automatically set by the system to ‘available’ again.

4) The FRMCS user attempts to log-in into another FRMCS device. He/she is successfully logged-in and obtained another FRMCS user identity. Since driver 2 is now connected to two FRMCS devices, he/she decides to manually set one of his/her FRMCS User identities to status ‘do not disturb’.

5) The FRMCS user has successfully log-off the FRMCS system. His/her associated FRMCS User identity is automatically set to ‘offline’.

6) A controller is just starting his/her journey tasks and is successfully logged to the FRMCS system and has activated his/her functional identity. The availability of the FRMCS users in his/her area of responsibility is displayed based on their FRMCS User identity status.

#### 9.18.2.4	Post-conditions

A controller is aware of the availability of the FRMCS users in his/her area of responsibility based on the status of their FRMCS user identity.

#### 9.18.2.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-9.18.2-001] | The FRMCS System shall automatically set the status of the FRMCS Users based on the status of their MC Service ID, when not registered to a functional identity yet. | A/T | TS 22.280 | Availability of the MC Users based on the status of MC Service ID, automatically set by the system, is not defined. CR is needed. |
| [R-9.18.2-002] | The FRMCS System shall set the status of a MC Service ID to: - Available, when successfully logged into the FRMCS system or when not involved in any communication - Busy when involved in communication - Offline, when logged-off - Do not disturb (manual option) | A/T | TS 22.280 | Status of the MC service ID, based on conditions, is not defined. CR is needed. |
| [R-9.18.2-003] | The FRMCS system shall allow an authorized user to be aware of the status of the MC Service IDs of FRMCS users in his/her area of responsibility. | A/T | TS 22.280 | Availability of the status of FRMCS users to authorised user, based on MC Service Id, is not defined. CR is needed. |

### 9.18.3	Use case: Presence of a FRMCS User registered to a functional identity

#### 9.18.3.1	Description

A FRMCS user registered to a functional identity is automatically assigned to an availability status. The availability status of a functional identity and the updates of it, are displayed on the HMI of the FRMCS User’s device.

#### 9.18.3.2	Pre-conditions

The FRMCS user is successfully logged into the FRMCS system and has also successfully registered to a functional identity.

#### 9.18.3.3	Service flows

1) The functional identity of the FRMCS user is automatically set to ‘ready for operation’ by the system, once successfully registered.

2) An FRMCS User, e.g. leading driver, involved in a multi-user communication, needs to respond to an incoming controller to driver communication using another FRMCS device. He/she decides to put on-hold temporarily the multi-user communication. In that case, his/her functional identity, used for the multi-user communication, is automatically set by the FRMCS system to ‘Offline’.

3) A controller is starting his/her journey tasks and is just successfully logged to the FRMCS system and has activated his/her functional identity. The availability of the FRMCS users in his/her area of responsibility is displayed based on their functional identity status.

#### 9.18.3.4	Post-conditions

A controller is aware of the availability of the FRMCS users in his/her area of responsibility based on the status of their functional identity.

#### 9.18.3.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-9.18.3-001] | The FRMCS System shall allow displaying on the HMI, the status of all functional alias(es) the FRMCS user has activated. | A/T | TS 22.280 | Availability of the MC Users based on the status of functional alias(es), automatically set by the system, is not defined. CR is needed. |
| [R-9.18.3-002] | The FRMCS System shall allow an authorized user to display the status of the functional alias(es) of all the FRMCS users in his/her area of responsibility. Status of activated functional alias(es) shall be: - Ready for operation, when registered in. - Offline, when temporarily inactive | A/T | TS 22.280 | Status of activated functional alias(es) and update of the status when not used during a communication is not defined. CR is needed. |
| [R-9.18.3-003] | The FRMCS system shall allow an authorized user to be aware of the status of the FRMCS users in his/her area of responsibility based on the status of their functional alias(es) | A/T | TS 22.280 | Availability of the status of FRMCS users to authorised user, based on functional alias(es), is not defined. CR is needed. |

### 9.18.4	Use case: Availability of a functional identity

#### 9.18.4.1	Description

A FRMCS user wants to register to a functional identity. Before attempting this registration, he/she wants to know the status of this functional identity. The availability status of a functional identity is displayed on the HMI of the FRMCS User’s device.

#### 9.18.4.2	Pre-conditions

The FRMCS user is successfully logged into the FRMCS system and has obtained his FRMCS User identity but is not registered yet to a functional identity.

#### 9.18.4.3	Service flows

1) The FRMCS user asks to register to a functional identity.

2) If the requested functional identity is not registered yet for another FRMCS user, the FRMCS system responds with status ‘not registered’.

3) If the requested functional identity is available to be registered by multiple FRMCS users and at least one FRMCS user has been successfully registered to it, the FRMCS system responds with status ‘ready for operation’.

4) If the requested functional identity is not available to be registered by multiple FRMCS users and at least one FRMCS user has been successfully registered to it, the FRMCS system responds with status ‘transfer pending’.

#### 9.18.4.4	Post-conditions

The FRMCS User is aware of the status of a functional identity before attempting to register to it.

#### 9.18.4.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-9.18.4-001] | The FRMCS System shall provide the status of the functional identities (not register, ready for operation, transfer pending) to a FRMCS User, before attempting to register for it. | A/T | TS 22.280 | Not covered. CR is needed. |

End of changes