3GPP TSG-SA WG1 Meeting #108	S1-244387

Orlando, Florida, USA, 18-22 November2024								(revision of S1-244343)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.989 | CR | 0036 | rev | 1 | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New use case: Train driver takes-over an Ad hoc Group Call from another than the default FRMCS device |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UIC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S1 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_FRMCS_Ph6 |  |  |  |  |  | Date: |  |  | 2024-11-13 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Allow train driver to transfer an Ad hoc Group to a back-up device (e.g. handheld) outside of the cabin where the default device is hosted, e.g., in case of an accident. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Consider a default FRMCS device for Ad hoc Group calls 2) Take-over of a functional alias by an MC User |  |  |  |  |  |  |  |  |
| Take |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This feature is already existing in GSM-R and for that reason is mandatory for migration period. Moreover, it is an operational need for resilience. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 6.22.1, 6.22.6 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Addressed area: The multi-train voice communication and REC (Railway Emergency communication) are based on the concept of predefined initiating area and predefined addressed area. Based on this initiating area, the FRMCS system will determine a corresponding predefined addressed area where the users are eligible to be addressed. The eligible users will be addressed depending on their location and functional identity. Associated to this addressed area, a list of predefined additional users (e.g. controllers) will be also addressed.

Automatic Train Operation (ATO): Automatic Train Operation applications are responsible for acceleration to the permitted speed, speed reduction where necessary due to speed restrictions and stop at designated stations in the correct location.

Automatic Train Protection (ATP): Automatic Train Protection applications are responsible for giving Limit of Movement Authority to a train based on the train’s current speed, its braking capability and the distance it can go before it must stop.

Balise: An electronic beacon or transponder placed between the rails of a railway as part of an automatic train protection or operation (ATP/ATO) system.

Business communication applications: communication applications that support the railway business operation in general, such as wireless internet, etc.

Controller (Train Controller): A Ground FRMCS User provided with special capabilities by the FRMCS System.

Default FRMCS Device: In the case that the user is logged in multiple devices, the default FRMCS device is the 3GPP UE that is actively used for communication, in the case that the communication needs to be automatically connected. The FRMCS User’s default FRMCS device is set up by either the FRMCS User or by the FRMCS System Administrator.

Driver (Train Driver): A Mobile FRMCS User provided with special capabilities by the FRMCS System.

External System(s): A general category of stationary FRMCS Users. For example, External Systems could be systems monitoring for trains passing a red light to initiate a railway emergency call.

FRMCS Application: The application on a 3GPP UE offering railway specific communication services to the FRMCS User by making use of the communication capabilities offered by the 3GPP UE and the 3GPP network.

FRMCS Equipment Identity: The identity by which a FRMCS equipment can be addressed.

FRMCS Equipment Type: Indicates the purpose the FRMCS equipment is being used for, FRMCS equipment of different equipment types do have different capabilities.

FRMCS Equipment: The FRMCS Equipment consists of a 3GPP UE and a FRMCS Application residing on it. It may be combined with legacy railway communication equipment (e.g. GSM-R or TRS)

FRMCS Functional Identity: The identity related to a user or related to the equipment, as specified in 9.3 "Role management and presence" indicating its special Role (e.g. as Driver of a specific train, usually a train number) can be addressed.

FRMCS Network: this is a sub-part of the FRMCS System.

FRMCS Roaming: The ability for a FRMCS User to make use of FRMCS Applications in a Visited (FRMCS) Network.

FRMCS System: The system providing railway specific communication constituted of the FRMCS Equipment, the 3GPP transport and the application servers in the network. Legacy networks are not included in the FRMCS System.

FRMCS User Identity: The identity by which a FRMCS User can be addressed.

FRMCS User: A human user or a machine making use of the railway specific communication. FRMCS Users can be connected via 3GPP RAT, wired connectivity or other radio technology

Ground FRMCS User: A general category of FRMCS Users that are predominantly stationary. Mostly they are connected via wired connectivity but may be using also wireless in certain conditions.

Home FRMCS Network: The Home FRMCS Network is the network in which the FRMCS User is engaged in a subscription.

Initiating area: Predefined geographical area where the initiator of a communication is located and which determines the addressing rules used for the selection of the users to be included in the communication. The initiating area is used for the establishment of a multi-train or a REC communication.

Mobile FRMCS User: A general category of FRMCS Users that are mobile. Thus, they are connected via wireless connectivity all the time.

Mobile Intelligent Assistant: 5G enabled robot with autonomous movements and artificial intelligence to support passengers in the Railway Smart Station.

Off-Network communication: direct communication between FRMCS Users in proximity.

On-Network communication: indirect communication between FRMCS Users connected to FRMCS Network(s).

Performance communication applications: applications that help to improve the performance of the railway operation, such as train departure, telemetry, etc.

Radio Block Centre (RBC): A train sends its position and speed information periodically to the RBC. The RBC uses the received information to decide movement authority of the train.

Rail Infrastructure Manager: A company that owns or manages rail infrastructure; within this document the Rail Infrastructure Manager owns, administrates and operates the FRMCS Network.

Railway Smart Station: a train station where the 5G-based services such as IoT and AI, are used for providing assisting railway services.

Railways Undertaking: A company that offers train freight or passenger transportation services, making use of FRMCS network for their operational communication needs that is operated by a Rail Infrastructure Manager.

Role (Functional Role): The function a FRMCS User or a FRMCS Equipment is currently performing. Examples of Roles are Driver, Controller or shunting staff, etc. This is indicated by the FRMCS Functional Identity.

Shunting: manoeuvring trains in order to change their location or composition.

Trackside staff: Staff working as trackside maintenance and/or shunting members

Trainborne equipment: FRMCS Equipment which is physically embedded in train

Visited (FRMCS) Network: A Visited (FRMCS) Network can be either another FRMCS Network than the Home FRMCS Network, or a Public Land Mobile Network (PLMN).

Zone: A 2-dimensional region of a pre-determined size.

Zone resolution: The pre-determined size of the given zone.

Next change

## 6.22	Voice communication transfer related use cases

### 6.22.1	Introduction

In this chapter the use cases related to the transfer of incoming or ongoing voice communication, as well as the forwarding of incoming voice communication, using Ad hoc Group Calls and also the take-over of an ongoing Ad hoc Group Call from another than the selected as default FRMCS device, are described. The following use cases are defined:

-	Transfer of an incoming voice communication

-	Transfer of an ongoing voice communication

-	Service interworking and service continuation with GSM-R

-	Call forwarding for Ad hoc Group Call

-	Take-over by the train driver of an ongoing Ad hoc Group Call from another than the selected as default FRMCS device

The use cases are applicable for user-to-user and multi-user voice communications.

NOTE 1:	The assumption for all use cases is that Ad hoc Group Calls will be used for all operational railway communications.

Next change

### 6.22.6	Use case: Train driver takes-over an ongoing Ad hoc Group Call from another than the default FRMCS device

#### 6.22.6.1	Description

The use case describes the usage from a train driver of another device (e.g., handheld), as a back-up FRMCS device in case the cabin device dedicated to voice calls and RECs, cannot be used. This can happen either because the cabin device is out of service or any external train equipment (e.g., antenna, ...) is defected or in case of accident (e.g. collisions with obstacles or animals, etc.) where the train driver is sometimes obliged to leave the train cabin. Consequently, the train driver is obliged to take-over ongoing emergency calls, multi-train calls, driver to controller calls, etc., from this alternative FRMCS equipment, which is not considered as the default one for this kind of communications.

#### 6.22.6.2	Pre-conditions

The train driver is successfully logged into the FRMCS system using this cabin device and can be addressed by the FRMCS system through his/her FRMCS User Identity_A on the cabin device.

After successful logging, the train driver also selected the cabin device, as default one for his/her communications, meaning that communications in automatic commencement mode can be automatically connected to this device.

Moreover, the train driver is also successfully logged in the FRMCS system using a handheld. He/she can also be addressed by the FRMCS system on this handheld using his/her FRMCS User Identity_B.

To summarise, the train driver has successfully registered his/her FRMCS User Identity_A to the functional identity_A and FRMCS User Identity_B to the functional identity_B and can be addressed using both functional identities, depending on the device.

The functional identity_A of the train driver cannot be shared between multiple FRMCS Users.

#### 6.22.6.3	Service flows

1) The train driver has initiated a REC, using functional identity_A. The REC is in progress. The train is stopped due to a collision with an obstacle. The cabin device is not working properly and the train driver decides to leave the train cabin after having informed the controller responsible for the area.

2) The train driver wants to use the handheld to continue the REC call outside the cabin. As he/she is already logged to the handheld, FRMCS User Identity_B request FRMCS system to take-over the functional identity_A.

3) The FRMCS System checks if there is any conflict with the other functional identities of FRMCS User Identity_B before allowing functional identity_A to be taken over.

4) The FRMCS User Identity_B takes over functional identity_A through the handheld and gets connected to the ongoing REC.

5) The FRMCS system informs FRMCS User Identity_A that functional identity_A has been deactivated for him/her.

6) The train driver performs remote log off from the cabin device.

#### 6.22.6.4	Post-conditions

1) The train driver continues the REC call from the handheld, using the functional identity_A.

2) Functional identity_A has been deregistered for FRMCS User Identity_A of the cabin device.

#### 6.22.6.5	Potential requirements and gap analysis

| Reference Number | Requirement text | Application / Transport | SA1 spec covering | Comments |
| --- | --- | --- | --- | --- |
| [R-6.22.6-001] | The FRMCS system shall allow a ‘human’ FRMCS user to login into multiple FRMCS devices, separately, by using different FRMCS User Identities (e.g., FRMCS User Identity_A in the default FRMCS device, such as cabine device and FRMCS User Identity_B in the secondary UE, such as handheld). | A | 22.280 | [R-5.10-001] |
| [R-6.22.6-002] | The FRMCS system allows the ‘human’ FRMCS user to designate an FRMCS device as the default FRMCS equipment for user-to-user and multi-user communication when logged into multiple FRMCS devices using different FRMCS User Identities. | A | 22.280 | Choose a default FRMCS device for Ad hoc Group Calls is not covered. CR is needed |
| [R-6.22.6-003] | The FRMCS system shall allow the ‘human’ FRMCS user to take-over a functional alias, when logged into multiple FRMCS devices, using separate FRMCS User Identities for each device. | A/T | 22.280 | [R-5.9a-009] Partially covered. Only authorised user is allowed to take-over a functional alias. Update is needed to include other MC users. CR is needed |

End of changes