T-doc. S1-010018

3GPP TSG-SA1 #11 Meeting

Capetown, South Africa, 6th – 9th February 2001

Title: 		Work Item Description for Release 4: ODB (Operator Determined Barring) for Packet Oriented Services

Source: 	NEC

Purpose: 	SA2 : For Approval

SA1 : For Information

This contribution proposes the Work Item Description on ODB for Packet Oriented Services.

This WID had been approved in the SA2 meeting that was held in 22nd - 26th January 2001 at Los Angeles.

This contribution is for information to SA1, since this WI has some impacts to the specifications that are owned by SA1.

Work Item Description

# ODB (Operator Determined Barring) for Packet Oriented Services (SA)

- 3GPP Work Area

|  | Radio Access |
| --- | --- |
| X | Core Network |
| X | Services |

- Linked work items

None

- Justification

As stated in the current ODB stage1 specification, the purpose of ODB feature is to be able to limit the service provider's financial exposure to new subscribers, or to those who have not promptly paid their bills.

In order to fulfil this purpose, this network feature has to be effective for both CS domain and PS domain. Currently in release 99, the ODB feature works only for CS domain.

- Objective

Packet Oriented Services, particularly data services, are different in nature to Circuit Oriented Services, and therefore have different requirements for Operator Determined Barring. Currently there is no requirements described in the ODB stage 1 specification for Packet Oriented Services.

- Service Aspects

As described in the following categories, the Service Provider may at any time activate this feature and this shall terminate any relevant services in progress, and bar future requests for service covered by the barring category:

- It shall be possible to bar subscribers completely from the Packet Oriented Services.

- It shall be possible to bar a subscriber from requesting Packet Oriented Services from access points that are within the HPLMN whilst the subscriber is roaming in a VPLMN.

- It shall be possible to bar a subscriber from requesting Packet Oriented Services from access points that are within the roamed to VPLMN.

The term 'access point' is used to indicate the GGSN or part of the GGSN that is specified by a particular APN.

- MMI-Aspects

None

- Charging Aspects

None

- Security Aspects

None

- Impacts

| Affects: | USIM | ME | AN | CN | Others |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

- Expected Output and Time scale (to be updated at each plenary)

| New specifications |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Spec No. | Title |  | Prime rsp. WG | 2ndary rsp. WG(s) | Presented for information at plenary# | Approved at plenary# |  | Comments |
| None |  |  |  |  |  |  |  |  |
| Affected existing specifications |  |  |  |  |  |  |  |  |
| Spec No. | CR | Subject |  |  |  |  | Approved at plenary# | Comments |
| 22.041 (ODB stage1) | S1-00852 S1-00853 | - Inclusion of section 5.2 to add requirement for ODB of packet services and to add the definition of the term 'Service Access Point' |  |  |  |  | TSG SA#10 | Small modification to the ODB barring categories for PS will be submitted to the next SA1 meeting from NEC. |
| 22.078 (CAMEL stage 1) |  | - Explicitly indicate that ODB for PS does not have any interaction with CAMEL. |  |  |  |  | TSG SA#11 |  |
| 23.060 (GPRS stage2) |  | - Annex A (APN and GGSN selection) in 23.060 needs to be enhanced to describe the barring mechanism. - ODB PS parameters need to be added to Information storage in HLR and SGSN. - ODB functionality is declared in the section 5.3 Network Access Control Functions. |  |  |  |  | TSG SA#11 |  |

- Work item raporteurs

NEC, Toshiyuki Tamura (tamurato@nsf.ncos.nec.co.jp)

- Work item leadership

TSG-SA2

- Supporting Companies

NTT DoCoMo, NTT Comware, NEC, Fujitsu, NTC, Vodafone and blu spa

- Classification of the WI (if known)

|  | Feature (go to 14a) |
| --- | --- |
|  | Building Block (go to 14b) |
| X | Work Task (go to 14c) |

14a		The WI is a Feature: List of building blocks under this feature

14b		The WI is a Building Block: parent Feature

14c		The WI is a Work Task: parent Building Block

ODB (Operator Determined Barring) for Packet Oriented Services (SA)