TSG-SA WG1 SWG #21	S1-030792

Sophia Antipolis, France, 7th–11th July 2003	Agenda Item:

| CR-Form-v7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.140 | CR | CRNum |  rev | - |  Current version: | 6.1.0 |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Conditional delivery behaviour |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source:  | Nokia |  |  |  |  |  |  |  |  |  |
| Work item code:  | MMS |  |  |  |  |  |  | Date:  |  | 24/06/2003 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | REL-6 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) |  |
| Reason for change:  |  | In certain situations the user may want to modify the delivery mechanism according to a set of network, MMS and terminal parameters. This change request introduces a new requirement to allow the user to specify a per message delivery mechanism. The change request also states that the network can set up a default mechanism for the users so in order to improve the MMS experience. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | A new requirement is introduced to allow the user to select the most appropriate delivery mechanism subject to a number of information on the MM. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Description of how conditional delivery of messages can be specified and used is less complete. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  | X |  | Other core specifications  |  |  | 23.140 |  |  |  |
| Affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at http://www.3gpp.org/specs/CR.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://ftp.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2001-03 contains the specifications resulting from the March 2001 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

## 5.2	Multimedia message delivery and submission

-	Submission mechanism

The MMS shall support multimedia messages or messages elements to be submitted from the sender’s terminal.

-	Push Mechanism

The MMS shall be able to support a request for multimedia messages or messages elements to be automatically delivered to the recipient's terminal.

-	Pull Mechanism

The MMS shall be able to support a request for multimedia messages or messages elements to be delivered to the recipient’s terminal on request by the recipient.

Note:	Push and pull delivery mechanisms could be identical; the criteria which decide on the type of mechanism (push / pull) are either described in the User Services Profile or out of the scope of this specification.

-	Concurrency

The MMS shall be able to support MM delivery to and from the user’s terminal not be restricted during other active services (subject to the capabilities of the terminal and the network).

-	Streaming

The MMS shall be able to support streaming for MM delivery from the MMS system to the terminal.

Support for streaming for MM upload from the terminal to the MMS system will be considered for future releases.

-	Preferred Bearer

It shall be possible to define a list of precedence for bearers in the configuration information sets for delivery and submission of MM (e.g. GPRS, CSD). By default, the terminal shall be able to support automatic bearer selection (i.e. without user intervention) based on the order of precedence defined in the configuration information sets on the USIM[7] or SIM [8]. The user shall be able to enable or disable automatic bearer selection. When disabled, manual bearer selection shall be available from the list of bearers.

- Conditional delivery mechanism

It shall be possible for the user to define in the User Profile a set of conditions that determine which delivery mechanism should be used for the submission of a MM.

Such conditions shall include the roaming status of the recipient (e.g. inside or outside the home network) and may also include:

- identity of the MM originator

- time of day (of the recipient's home network)

- The notification message shall relay the information of the user's preferred delivery mechanism to the ME and if a mismatch is identified between the delivery mechanism configured in the terminal and the suggested delivery mechanism, a warning shall be displayed to the user. It shall be possible for the user to follow the recommendation given in the notification message.

- Furthermore, the terminal may also display a warning prior the download of a message depending on some terminal parameters such as:

- available storage capacity

- remaining battery life

- available bearers

For example, the user may elect to have all MMs downloaded automatically when in the home network, be able to manually select whether to download a MM or not when roaming.

It shall be possible for the network operator to program a default set of rules for the delivery mechanism in the User profile. Such rules can be overridden by the user.

Note: the way the user profile is accessed and modified is not subject of standardisation.