3GPP TSG-SA WG1 Meeting #76bis	S1-170116

Spokane, WA, USA, 16-20 January 2017

Title:	Update of 3D video composition for V2X scenario

Agenda Item:	6.1

Source:				Intel Corporation, IAESI

Contact:			Ana Lucia Pinheiro (ana.lucia.a.pinheiro@intel.com)

Eyal Ben-Sa’adon, Esaadon@advaoptical.com

Abstract: This document removes the TBDs and brackets (“[ ]”) from the 3D video composition requirements.

## 5.25	3D video composition for V2X scenario

### 5.25.1	Description

This use case consists of multiple UEs supporting V2X application moving in an area. The UEs may belong to the same PLMN or different PLMNs. The UEs may also be camped in different cells.

Those UEs have a camera and they take a video of the environment, and send this video to a server. The server can be in the cloud or in the near the UE point of attachment (i.e., mobile edge compute (MEC)). The server/MEC will then post-process the videos received and combine the information in order to create a single 3D video of the environment. The 3D video can then be used for analysis in different scenarios, such as sharing the video with end-users in a car race, evaluation of possible accident by law enforcement, etc.

The UEs location information, allows the server to accurately represent the location, relative speed and distance of vehicles, pedestrians, and any objects in that area.

### 5.25.2		Potential requirements

[PR.5.25-001]	The 3GPP system shall provide a mechanism so that a server outside the 3GPP domain is able to time-synchronize different videos received from different UEs, each UE having a maximum absolute speed of 250 Km/h.

[PR.5.25-002]	The 3GPP system shall support a data rate of 10 Mbps in the uplink per UE (to support 4K/UHD video).

[PR.5.25-003]	The 3GPP system shall support a mechanism for UEs to be able to calculate 50 absolute location fixes per second, with 0.14  meters of accuracy for each location fix.

NOTE:	This requirement is to support 250 Km/h (69 meters per second), so 50 location fixes per second implies one location fix every 1.4 meters. The accuracy is set to be one order of magnitude less (i.e., 0.14 meters).