3GPP TSG-SA WG1 Meeting # 89e	S1-201032

# Electronic Meeting, Feb, 10 – 14, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 22.011 | CR | 0310 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the traffic handling over non-3GPP access for PS DATA Off |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA1 |  |  |  |  |  |  |  |  |  |
| Work item code: | PS_DATA_OFF |  |  |  |  |  | Date: |  |  | 20-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 3GPP system supports both 3GPP RATs and non-3GPP access. Current speficification is not clear on how to handle traffic over non-3GPP accesses in relation to PS Data Off activation/deactivation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that, - PS Data Off Exempt service is applicable to 3GPP RATs only |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is ambiguous on how traffic is treated over non-3GPP access |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 10 	3GPP PS Data Off

## 10.1	Description

3GPP PS Data Off is a feature which, when configured by the HPLMN and activated by the user, prevents transport via PDN connections in 3GPP access networks of all data packets except IP packets required by 3GPP PS Data Off Exempt Services.

## 10.2	Requirements

The 3GPP system shall provide a mechanism by which an operator can configure which operator services are defined as 3GPP PS Data Off Exempt Services for their own subscribers.

When 3GPP PS Data Off is activated in the UE, in order to preserve charging consistency:

-	the UE shall inform the network that 3GPP PS Data Off is activated,

-	the UE shall cease the sending of uplink IP Packets of all services that are not 3GPP PS Data Off Exempt Services via PDN connections in 3GPP access networks,

-	the network shall cease the sending of downlink IP Packets to the UE for all services that are not 3GPP PS Data Off Exempt Services via PDN connections in 3GPP access networks,

-	the UE shall cease the sending of uplink traffic over non-IP PDN types via PDN connections in 3GPP access networks, and

-	the network shall cease the sending of downlink traffic over non-IP PDN types via PDN connections in 3GPP access networks

NOTE 1:	Disabling of traffic on both the uplink and downlink is needed in order to provide consistency of charging between HPLMN and VPLMN, as well as consistency between what the user expects and what the user may be billed for.

When 3GPP PS Data Off is activated, if a PDN connection in non-3GPP access networks is available, the UE and the network may cease sending IP packets or traffic over non-IP PDN types also 3GPP PS Data Off Exempt Services via 3GPP access network.

When a PDN connection is associated with both 3GPP access network and non-3GPP access network, activation of 3GPP PS Data Off status (i.e., cease of sending IP Packets or traffic over non-IP PDN types for services that are not 3GPP PS Data Off Exempt Services) applies only to 3GPP access network of the PDN connection.

Each of the following operator services shall be configurable by the HPLMN operator to be part of the 3GPP PS Data Off Exempt Services:

-	MMTel Voice;

-	SMS over IMS;

-	USSD over IMS (USSI);

-	MMTel Video;

-	Particular IMS services not defined by 3GPP, where each such IMS service is identified by an IMS communication service identifier;

-	Device Management over PS

-	Management of USIM files over PS (e.g. via Bearer Independent Protocol); and

-	IMS Supplementary Service configuration via the Ut interface using XCAP.

3GPP PS Data Off may be activated based on roaming status, and the HPLMN may configure up to two sets of 3GPP PS Data Off Exempt Services for its subscribers: one is used when in HPLMN and another when roaming.

NOTE 2: The updating to the sets of configured 3GPP Data Off Exempt Services in the VPLMNs and HPLMN is not guaranteed to take effect in real time. The updating to the sets of configured 3GPP Data Off Exempt Services in the UEs is not guaranteed take effect in real time.

The user should be made aware of the operator services that are 3GPP PS Data Off Exempt Services.

NOTE 3: 	The system can support falling back to operate over the CS domain in case an operator service is not configured to be a 3GPP PS Data Off Exempt Service and an equivalent CS domain operator service exists for the operator service.