3GPP TSG-SA WG1 E-meeting #89e	S1-201086

Electronic Meeting Feb. 10 - 14, 2020

Title:	Reply LS
on Data Off applicability to non-3GPP PDU Session and MA PDU Session (S1-201010)

Release:	Rel-16

Work Item:	ATSSS, PS_Data_Off

Source:	3GPP SA1

To:	3GPP SA2

Cc:

Contact Person:

Name:			SungDuck Chun

Tel. Number:

E-mail Address:			sungduck.chun@LGE.COM

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA1 would like to thank SA2 for the LS.

Regarding the questions raised by the LS, the responses are:

Q1: Should Data Off apply to PDU Sessions transported over non-3GPP access?

A1: No. PS Data off is not applicable to PDU Sessions transported over non-3GPP access.

Q2: If the answer to Q1 is YES, is it applicable to both EPS and 5GS, or only to 5GS? and starting from which release noting that there may be service incompatibility between different UE releases/network releases which cannot be completely overcome?

A2: See Answer to Q1.

Q3: If the answer to Q1 is YES, should Data Off settings for PDU Sessions transported over non-3GPP Access be distinguished from Data Off settings for PDU Sessions transported over 3GPP Access (e.g. due to different billing schemes for non-3GPP Access and 3GPP Access)?

A3: See Answer to Q1

Q4: Multi-Access PDU Sessions are PDU Sessions that simultaneously have active User Plane over both 3GPP access and non-3GPP. If the answer to Q1 is NO, should Data Off apply also to the traffic carried over non-3GPP access for such Multi-Access PDU Sessions?

A4: Even if PS Data Off is activated, it does not affect the data transport over non-3GPP access.

2. Actions:

To SA2:

ACTION:

SA1 kindly requests SA2 to take above into account.

3. Date of Next TSG-SA WG1 Meetings:

SA1#90				2020-05-18	2020-05-22	TBD, North America

SA1#91				2020-08-24	2020-08-28	Wroclaw, Poland